The Malaysian Journal of Analytical Sciences Vol 15 No 2 (2011): 138 – 149

SLOPE FAILURE AT BUKIT ANTARABANGSA, AMPANG, SELANGOR, AND ITS RELATIONSHIP TO PHYSICAL SOIL PROPERTIES

Muhammad Barzani Gasim1*, Sahibin Abd. Rahim1, Mohd. Ekhwan Toriman2 dan Diyana Ishnin1

1Pusat Pengajian Sains Sekitaran dan Sumber Alam,

Fakulti Sains dan Teknologi,

2Pusat Pengajian Sosial, Pembangunan & Sekitaran,

Fakulti Sains Sosial dan Kemasyarakatan,

Universiti Kebangsaan Malaysia,

43600 UKM Bangi, Selangor, Malaysia

*Corresponding author: dr.zani@ukm.my

Abstract

Slope failure which occurred on 6 December 2008 at Bukit Antarabangsa, Ampang Selangor has caused mortalities and loss of properties whereas more than 20 houses were flattened. Prior to slope failure, it was heavily downpoured for a few hours that increased the soil saturation and plasticity properties. A total of 10 soil samples were randomly taken from stable and unstable slopes to determine physical soil properties, infiltration rate and their relationship to rainfall pattern. Soils were analyzed in terms of their physical properties; five years (2005-2009) of daily rainfalls were analyzed to determine their relationship to infiltration rate at each sampling station. Infiltration rate is determined by using infiltrometer double ring. Analysis of physical soils properties shows that soil texture was dominated by sandy soil with relatively high percentage of sand. Values of clay dispersion coefficient were relatively stable to very stable  from 0.013% to 11.85% and organic content from 1.38 % to 2.74 %. Range of porosity was from 50.12% to 62.31%, while the average levels of hydraulic conductivity was from level 2 to 5 or relatively slow to fast. Percentage of soil aggregate stability was from 5.12 % to 48.42 % and this value indicates that relative strength of soil mechanical pressure is inversely proportional to the percentage of water content. Soil plasticity value was high to very high but characterized by inactive colloids. Distribution of monthly rainfall was from 38 mm to 427mm. The infiltration rate during sampling time was from 3.0 cm/hr to 7.0 cm/hr; but it was expected from 10.94cm/hr to 915.05 cm/hr during slope failures. Overall, it was interpreted that physical soil properties was closely interrelated with slope stability, structure of sandy soil will enhanced soil porosity stage and enhance the infiltration process during heavy rainfall, and finally  triggering of slope failure.

Kata kunci: Bukit Antarabangsa, infiltration rate, sandy soil, slope failure

References

- Syers J.K. & Rimmer D.L. 1994. Soil science and sustainable land management in the tropics. United Kingdom: Cab International.  77-78.

- Fitzpatrick, E.A. 1974. An Introduction to Soil Science. New York: Longman Scientific and Technical. 255

- Othman Yaacob & Shamshuddin Jusop. 1982.  Sains Tanah. Kuala Lumpur: Dewan Bahasa dan Pustaka, Kementerian Pelajaran Malaysia. 402

- Young, A. 1980. Tropical soils and soil survey. Cambridge University Press. 468

- Muhammad Barzani Gasim & Brunotte, D.A. 1987. Structural Behavior of Crocker Formation and its implication to landslides. 9th Southeast Asian Geotechnical Conference, Bangkok, Thailand, 7-11 December 1987. 1-57-1-67.

- BERNAMA. 2009. Dua kementerian selaras garis panduan pembangunan kawasan cerun. http://www.bernama.com/bernama/v5/bm/newsgeneral.php?id=432697[13 Ogos].

- Marhana Abdul Latif. 2006. Penentuan kestabilan cerun disebabkan oleh penyusupan hujan. Laporan Projek Sarjana Muda. Universiti Teknologi Malaysia.

- Herbel C.H. & Gile L.H. 1972. Field moisture regimes and morphology of some Arid-land soils in New Mexico.  In. Bruce R.R., Flach K.W. & Taylor H.M. Field Soil Water Regime. Wisconson USA: Soil Science Society of America. 119–152.

- Reynolds W.D. & Elrick D.E. 2005. Measurement and Characteristic of Soil Hydraulic Properties. Canada: CRC Press.

- Miller D.E. 1973. Water retention and flow in layered soil profiles. In. Bruce R.R., Flach K.W & Taylor H.M (Edts). Field Soil Water Regime. 107-117. Wisconsin USA: Soil Science Society of America.

- Shamshuddin Jusop. 1981. Asas Sains Tanah. Kuala Lumpur: Dewan Bahasa dan Pustaka, Kementerian Pelajaran Malaysia. 273

- Ng, T.F. 1994. Microstructures of the deformed granites of the eastern Kuala Lumpur-Implication for mechanisms and temperatures of deformation. Bull. Geo. Soc. Malaysia 35: 47-59.

- Tjia, H.D. 1972. Fault movement, reoriented stress field and subsidiary structures. Pacific Geology 5: 49-70.

- Azman, A.Ghani. 2000. Chemical variation of muscovite from the Kuala Lumpur granite, Peninsular Malaysia.  Bull. Geo. Soc. Malaysia 44: 117-124.

- Tan Siang Fei & Tajul Anuar Jamaluddin. 2000. Geologi kejuruteraan kawasan Taman Bukit Utama, Ulu Kelang-Ampang.  Bull. Geo. Soc. Malaysia 44: 55-58.

- Kez’di A. 1974. Handbook of Soil Mechanic. Jil. 1: Soil Physics. New York: Elsevier Publishing Company.