The Malaysian Journal of Analytical Sciences Vol 15 No 2 (2011): 159 – 166

MEASUREMENT OF 226Ra, 228Ra AND 40K IN SOIL IN DISTRICT OF KUALA KRAI USING GAMMA SPECTROMETRY 

Zaini Hamzah1, Siti Afiqah Abdul Rahman1*, and Ahmad Saat2
1Faculty of Applied Sciences,

 2Institute of Science, 

Universiti Teknologi MARA,

 40450 Shah Alam, Selangor, Malaysia

*Corresponding author: siti_afiqah@ymail.com
Abstract

The granitic region is known to have high natural radionuclides content. The natural background of the area will be elevated and the exposure rate also will be higher as compared to other region. The present study is focusing on the presence of natural uranium isotopes using its progenies in soils belong to the river basin of the granitic region of Kuala Krai district, Malaysia. Granitic characteristics of the region were believed to produce significant concentrations of natural radionuclide such as uranium and thorium. This paper presents the results of measurement activity concentration 226Ra, 228Ra and 40K in soil using Gamma Spectrometry to estimate activity concentration of radionuclides in fourteen soil samples collected from this study area. The range of activity concentration of 226Ra, 228Ra and 40K is 40.2-264.0, 49.2-312.9 and 491.1-1184.2 Bq/kg respectively. These results were used to estimate the hazards index and annual exposure rate to the member of public.
Keywords: 238U activity concentration, granitic region, gamma spectrometer
References
1. Zaini Hamzah, Siti Afiqah Abdul Rahman, Ahmad Saat, Siti Shahrina Agos, Zaharuddin Ahmad, (2010).  Measurement of 226Ra in River Water using Liquid Scintillation Counting Technique. Journal of Nuclear and Related Technologies. 7: 12-23.
2. Miah, F.K., Roy, S., Touhiduzzaman, M. and Alam, B., (1998). Distribution of Radionuclides in Soil Sample in and around Dhaka City. Applied Radiation & Isotopes. 49: 133-137.

3. A., El-Bahi, S.M., Ahmed, F., Abdel-Haleem, A.S., (2001). Natural Radioactivity and Radon Exhalation Rate of Soil in Southern Egypt. Applied Radiation & Isotopes. 55: 873-879.

4. Chiozzi, P., Pasquale, V., Verdoya, M., (2002). Naturally Occuring Radioactivity at the Alps-Apennines Transition. Radiation Measurement. 35: 147-154.

5. Myrick, T.F., Berven, B.A. and Haywood, F.F., (1983). Determinationof Concentration of Selected Radionuclides in Suface Soil in the US. Health Physics. 58: 417-418.

6. Tenniseen,A.C., (1994). Nature of earth materials. Printice-Hall, NJ07632: 333-334.

7. Ahmad Saat, Zaini Hamzah, Hamimah Jamaluddin, Husna Mardhiah Muda, (2011). Assessment of Outdoor Radiation Hazard of Natural Radionuclides in Tourism Beach Areas. Proceeding of 3rd International Symposium & Exhibition in Sustainable Energy & Environment,1-3 June 2011, Melaka, Malaysia. 123-127.

8. Masitah Alias, Zaini Hamzah, Ahmad Saat, Mohamat Omar, Abdul Khalik Wood, (2008). An Assessment of absorbed Dose and Radiation Hazard Index from Natural Radioactivity. Malaysian Journal of Analytical Sciences. 12: 195-204.

9. Zaini Hamzah, Ahmad Saat, Noor Hayati Mashuri, Seh Datul Ridhuan, (2008). Surface Radiation Dose and Radionuclide Measurement in Ex-Tin Mining Area, K. Gajah, Perak. Malaysian Journal of Analytical Sciences. 12: 419-431.

10. Saïdou, Bochud, F., Laedermann, J.P., Njock, M.G.K. and Froidevaux, P., (2008). A Comparison of Alpha and Gamma Spectrometry for Environmental Natural Radioactivity Surveys. Applied Radiation & Isotopes. 66: 215-222.
11. Garcia-Talavera, M., (2003). Evaluation of the Suitability of Various γ Lines for the γ Spectrometric Determination of 238U in Environmental Samples. Applied Radiation & Isotopes. 59: 165-173.

12. Ahmad Saat, Zaini Hamzah, Mohammad Fariz Yusop, Muhd Amiruddin Zainal, (2010). Experimental Determination of the HPGe Spectrometer Efficiency Calibration Curves for Various Sample Gometry for Gamma Energ from 50 keV to 2000 keV. Progress of Physics Research in Malaysia. 39-42.

13. Kurnaz, A., Küçükömeroğlu, B., Keser, R., Okumusoglu, N.T., Korkmaz, F., Karahan, G., and Çevik, U., (2007). Determination of Radioactivity Levels and Hazards of Soil and Sediment Samples in Firtina Valley (Rice, Turkey). Applied Radiation & Isotopes. 65: 1281-1289.

14. UNCSEAR, United Nations Scientific Committee on the Effect of Atomic Radiation, (2000). Sources, effects and risk of Ionizing Radiation, United Nations, New York.

15. Beretka, J., Mathew, P.J., (1985). Natural Radioactivity of Australian Building Materials, Industrial Wastes and By-products. Health Physics. 48(1): 87-95.

16. Iqbal, M., Tufail, M., and Mirza, S.M., (2000). Measurement of Natural Radioactivity in Marble Found in Pakistan using a NaI(TI) Gamma-ray Spectrometer. Journal of Environmental Radioactivity. 51: 255-265.

17. Lee, S.K., Wagiran, H., Ramli, A.T., Apriantoro, N.H., Wood, A.K., (2009). Radiological monitoring: terrestrial natural radionuclides in Kinta district, Perak, Malaysia. Journal of Environmental Radioactivity. 100: 368-374.

18. El-Reefy, H.I., Sharshar, T., Zaghloul, R., and Badran, H.M., (2006). Distribution of Gamma-ray Emitting Radionuclides I the Environment of Burullus Lake: I. Soils and Vegetations. Journal of Environmental Radioactivity. 87: 148-169.

19. Sujo, L.C., Cabrera, M.E.M.,Villalba, L., Villalobos, M.R., Moye, E.T., Leόn, M.G., Tenorio, R.G., Garcia, F.M., Peraza, E.F.H., Aroche, D.S., (2004). Uranium-238 and Thorium-232 Series Concentrations in Soil, Radon-222 indoor and Drinking Water Concentrations and Dose Assessment in the City of Aldama, Chihuahua, Mexico. Journal of Environmental Radioactivity. 77: 205-219.
20. Higgy, R.H., and Pimpl, M., (1998). Natural and Man-made Radioactivity in Soils and Plants Around the Research Reactor of Inshass. Applied Radiation & Isotopes. 49(12): 1709-1712.

21. UNCSEAR, United Nations Scientific Committee on the Effect of Atomic Radiation, (2008). Sources and Effects of Ionizing Radiation. Report to General Assembly with Scientific Annexes, United Nations, New York.

22. UNCSEAR, United Nations Scientific Committee on the Effect of Atomic Radiation, (1993). Exposure from Natural Sources of Radiation, United Nations, New York.

23. Huy, N.Q. and Luyen, T.V., (2004). A Method  to Determine 238U Activity in Environmental Soil Samples by Using 63.3-keV-photopeak-gamma HPGe Spectrometer. Applied Radiation & Isotopes. 61: 1419-1424.

