The Malaysian Journal of Analytical Sciences Vol 16 No 2 (2012): 148 – 156

EFFECTS OF VARIOUS PARAMETERS ON IN-HOUSE COCKTAIL DEVELOPED FOR THE MEASUREMENT OF GROSS ALPHA AND BETA IN AQUEOUS ENVIRONMENTAL SAMPLES USING LIQUID SCINTILATION COUNTING TECHNIQUE
(Kesan Pelbagai Parameter Ke Atas Koktil yang Dibangunkan Sendiri bagi Pengukuran Gros Alfa dan Beta Di Dalam Sampel Air Persekitaran Menggunakan Teknik Pembilang Sintilasi Cecair)

Zaini Hamzah1, Masitah Alias1*, Ahmad Saat2, Abdul Kadir Ishak3 and Zaharudin Ahmad3
1Faculty of Applied Sciences,

Universiti Teknologi MARA, 40450 Shah Alam, Selangor

2International Education Centre (INTEC),

Universiti Teknologi MARA, 40200 Shah Alam, Selangor

3Radiochemistry and Environmental Laboratory,

Malaysian Nuclear Agency, 43000 Bangi, Selangor

*Corresponding author: ellyaa@yahoo.com
Abstract

Liquid scintillation counting (LSC) is a versatile technique for measuring beta emitters in organic liquid samples. When this technique is applied to the aqueous environmental samples, the limitation is to incorporate the aqueous phase which contains the radionuclides of interest into an organic phase which can readily mix with a scintillator that is responsible for the detection of photon through excitation processes. The in-house cocktail was developed by mixing the organic solvent which contains primary and secondary solutes with a specific emulsifier (Triton-N101) which is capable of linking the aqueous phase to the organic phase through its hydrophilic and hydrophobic tails. The best proportion for water: toluene: Triton-N101 is 2: 4: 4, respectively. Since the aqueous environmental samples are analyzed, there is a need to study various effects such as pH, buffer and salts on the cocktail to ensure the stability of the cocktail being used. 3H and 232U tracers were used throughout this study. pH value shows an optimal condition for both tracers, while  acid, base, buffer and salts all show decreased in counting efficiency when the their concentrations increased.

Keywords: liquid scintillation counting, cocktail, emulsifier, Triton N101, tritium

References

1. LSC Technical Tips from Packard (SC-005) (1997). Counting Solutions
2. Perkin Elmer Application Note (2007). LSC in Practice Counting 14C in High Activity Sodium Carbonate Solution.
3. Zaini Hamzah, Masitah Alias, Zaharudin Ahmad, 2011. Discriminator setting cocktail preparation for analysis of alpha and beta emitters in aqueous solution using liquid scintillation counter. The Malaysian Journal of Analytical Sciences, 15(1), 27-36  
4. A. Dyer (1980). Liquid Scintillation Counting Practice, publisher (pg 30)
5. Victor Jobbagy, Norbert Kavasi, Janos Somlai, Peter Dombavari, Csaba Gyongvosi, Tibor Kovacs. (2011). Gross alpha and beta activity concentrations in spring waters in Balaton Upland, Hungary. Radiation Measurement, 46: 159-163.

6. Gursel Karahan, Neset Ozturk and Ahmet Bayulken (2000). Natural radioactivity in various surface waters in Istanbul, Turkey. Water Resources, 34(18), 4360-4370.
7. UNSCEAR, United Nations Scientific Committee on Exposure to Atomic Radiation Report 1993.
8. H. Ozmen, F. Kulaher, A. Cukurovah, M. Dogru (2004). Concentration of heavy metal and radioactivity in surface water and sediment of Hazar Lake (Elazig, Turkey). Chemosphere, 55, 401-408.
9. Higuchi, H. (1981). The analysis of radium in the environmental samples. Radioisotopes, 30, 618.
10. United Nation, Guidance on Transformation/Dissolution of Metals and metal Compounds in Aqueous Media Report 2007.
11. Alan H. Welch, Zoltan Szabo, David L.Parkhurst, Peter C. Van Metre (1995). Gross beta activity in ground water: natural sources and artifacts of sampling and laboratory analysis. Applied Geiochemistry, 10, 491-503.

