The Malaysian Journal of Analytical Sciences Vol 16 No 2 (2012): 194 – 201

DEVELOPMENT OF A METHOD FOR THE SPECIATION OF MERCURY IN ENVIRONMENTAL SAMPLE ANALYSIS
(Pembangunan Kaedah Bagi Analisis Penspesian Merkuri Dalam Sampel Sekitaran)

M. M. Rahman1*, M. Awang1, A. M. Yusof 2, A. K. H. Wood3, S. Hamzah3 and A. Shamsiah3
1Department of Pharmaceutical Chemistry, Faculty of Pharmacy,

 International Islamic University Malaysia, Indera Mahakota, 25200 Kuatan, Pahang, Malaysia.

2Department of Chemistry, Faculty of Science, 

Universiti Teknologi Malaysia, Skudai, JB, 81310 Malaysia.

3Malaysian Institute for Nuclear Technology Research (MINT), 

Bangi, Kajang, 43000 Malaysia

*Corresponding author: mdrahman@iium.edu.my
Abstract

Trace concentrations of mercury in water samples were determined by a method involving a simple and rapid procedure connecting the flow-injection inductively coupled plasma mass spectrometry (FI-ICP-MS) and graphite furnace atomic absorption spectrometry (GFAAS) techniques. Mercury vapor, generated by sodium borohydride as the reductant, was stabilized by potassium dichromate (K2Cr2O7), then released by controlled heating and detected by FI-ICP-MS. A flow injection sample introduction system with time based injection was used and the sensitivity was found to be proportional to the mass of mercury introduced. Methyl mercury (II) was preconcentrated using the ammoniumpyrrolidindithiocarbamate (APDTC)-chloroform extraction procedure and the chloroform extract was introduced into the graphite tube. A linear calibration graph was obtained for 5 -150 ng mercury in 2.5 ml of chloroform extract. Because of the high stability of MeHg (II)-APDTC complexes, it is possible to evaporate the extract in order to obtain a crystalline powder to be dissolved with a few micro liters of chloroform enacting MeHg (II) and Hg (II) can be detected at sub-nanogram levels.

Keywords: Methyl mercury, Flow Injection-Inductively Coupled Plasma Mass Spectrometry, Graphite Furnace Atomic Spectrometry and ammoniumpyrrolidindithiocarbamate
References

1.
Ellis L. A. and Roberts D. J., (1996). Novel method to determine mercury in sediment using a gold coated dual 'bent' tube trap. J. Anal. At. Spectrom., 11, 1063-1066. 

2.
Bunce N., (1994). Environmental chemistry, Wuerz Pub. Ltd. Winnipeg, Canada, 2nd Edi., pp., 232-233.

3.
Filippelli M., (1984). Determination of trace amounts mercury in sea water by graphite furnace atomic absorption spectrometry." Analyst, 109, 515-517.

4.
Boomer, D. W. and Powell M. J., (1987). Determination of uranium in environmental samples using inductively coupled plasma mass spectrometry. Anal. Chem., 59, 2810-2821.

5.
Ebenzer D., Denoyer R. E., and Tyson F. J., (1996). Flow injection determination of mercury with preconcentration by amalgamation on a gold- platinum gauze by inductively coupled plasma mass spectrometry." J. Anal. At. Spectrom., 11, 127-132.

6.
Krull I. S., (1991). Trace metals analysis and Speciation. New York, USA; Elsevier.

7. Ebdon L., Hill S., and Ward R. W., (1987). Directly coupled liquid chromatography - atomic spectroscopy. Analyst; 112, 1-16.
8.
schintu. M., T. Kaur and Kudo A., (1989). Inorganic and methyl mercury in inland waters. Wat. Res., 6, 699-704.

9.
Wan C. C., Chen C. S. and Jiang S. J., (1997). Determination of mercury compounds in water samples by liquid chromatography- inductively coupled plasma mass spectrometry with an in situ nebulizer/ vapor generator. J. Anal. At. Spectrom, 12, 683-687.

