The Malaysian Journal of Analytical Sciences Vol 17 No 1 (2013): 24 – 29

LEACHATE CHARACTERIZATION FROM A CLOSED LANDFILL IN AIR HITAM, PUCHONG, MALAYSIA

(Pencirian Air Larut Resap dari Tapak Pelupusan Ditutup Air Hitam, Puchong, Malaysia)

Nur Fatin Dahlia Mat Salleh1*, Ku Halim Ku Hamid2
1Faculty of Applied Sciences, 

2Faculty of Chemical Engineering, 

Universiti Teknologi MARA, 40450 Shah Alam, Malaysia

*Corresponding author: fatin_dahlia@hotmail.com
Abstract

Leachate, wastewater that was collected from landfill is known to have pungent smell and may impose serious harm to human health and the environment. Air Hitam, Puchong Sanitary Landfill has stopped its land filling operation since December 2006 and is under post-closure maintenance stages. After several years of stopping its operation, a landfill will still produce leachate hence it needs constant monitoring and maintenance. The main aim of this paper was to characterize leachate produced from Air Hitam, Puchong Closed Landfill, according to several important parameters: pH, temperature, chemical oxygen demand (COD), ammoniacal nitrogen (NH4-N), total organic carbon (TOC), total solids, volatile organic acids (VOA) and heavy metals content, to determine its suitability in producing methane by identifying its phase. Leachate samples were drawn weekly for a period of 3 months from three different ponds, untreated raw leachate pond 1 and treated leachate pond 2 and 3. Results obtained showed that the average values were around 25°C, average pH 8, highest COD reading was 5,248 mg/L, TOC highest at 6,797 mg/L, VOA highest at 1,424 mg/L and ammoniacal content of 3.10 mg/L the highest.

Keywords: Leachate, characterization, closed landfill, COD, Air Hitam sanitary landfill
References
1. Renou, S., Givaudan, J., Poulain, S., Dirassouyan, F., & Moulin, P. (2008). Landfill leachate treatment: Review and opportunity. Journal of Hazardous Materials 150: 468 - 493.

2. Kjeldsen, P., Barlaz, M., Rooker, A., Baun, A., Ledin, A., & Christensen, T. (2002). Present and long-term composition of MSW landfill leachate: A review. Critical Reviews in Environmental Science and Technology 32:4: 297 - 336.

3. Abdul Aziz, H., Alias, S., Adlan, M.N., Faridah, Asaari, A.H., Zahari, M.S. (2007). Colour removal from landfill leachate by coagulation and flocculation processes. Bioresource Technology 98: 218 - 220.

4. Salem, Z., Hamouri, K., Djemaa, R., & Allia, K. (2008). Evaluation of landfill leachate pollution and treatment. Desalination 220: 108 - 114.

5. Aziz, S., Abdul Aziz, H., Yusoff, M., Bashir, M., & Umar, M. (2010). Leachate characterization in semi-aerobic and anaerobic sanitary landfills: A comparative study. Journal of Environmental Management 91: 2608-2614.

6. O’Keefe, D.M., Chynoweth, D.P. (2000). Influence of phase separation, leachate recycle and aeration on treatment of municipal solid waste in simulated landfill cells. Bioresource Technology 72: 55-66.
7. Air Hitam Sanitary landfill (2011). Retrieved July 2011, from www.whbenvironment.com/index.php
8. APHA; AWWA; WPCF. (2005). Standard Methods for the Examination of Water and Wastewater 21st Edition. Washington: American Public Health Association .

9. Al Sabahi, E., Abdul Rahim, S., Wan Zuhairi, W., Al Nozaily, F., & Alshaebi, F. (2009). Leachate composition and groundwater pollution at municipal solid waste landfill of Ibb City, Yemen. Sains Malaysiana 38 (3): 295 - 304.

10. Al-Yaqout, A., & Hamoda, M. (2003). Evaluation of landfill leachate in arid climate - a case study. Environmental International 29: 593 - 600.

11. Chen, P. H. (1996). Assessment of leachates from sanitary landfills: Impact of age, rainfall, and treatment. Environment International Vol 22: 225 - 237.

12. Simon, J. (2002). Enzymology and bioenergetics of respiratory nitrite ammonification. FEMS Microbiology Review 26: 285 - 309.

13. Wiszniowski, J., Robert, D., Surmacz-Gorska, J., Miksch, K., & Weber, J. (2006). Landfill leachate treatment methods: A review. Environ Chem Lett 4 : 51 - 61.

14. Bilgili, M.S., Demir, A., Ozkaya, B. (2007). Influence of leachate recirculation on aerobic and anaerobic decomposition of solid wastes. Hazardous Materials 143: 177 - 183.

15. Morling, S. (2010). Nitrogen removal and heavy metals in leachate treatment using SBR technology. Hazardous Materials 174: 679 - 686.

16. Nguyen, P.H.L., Kuruparan, P., Visanathan, C. (2007). Anaerobic digestion of municipal solid waste as a treatment prior to landfill. Bioresource Technology 98: 380 - 387.

17. Yuan, Q., Sparling, R., Oleszkiewicz, J.A. (2011). VFA generation from waste activation sludge: Effect of temperature and mixing. Chemosphere 82: 603 - 607.

18. Department of Meteorological, Ministry of Science, Technology and Innovation, Malaysia (2011). Retrieved November 2011, from http://www.met.gov.my/index.php?option=com_frontpage&Itemid=1

19. Bendz, D., Singh, V., & Akesson, M. (1997). Accumulation of water and generation of leachate in a young landfill. Journal of Hydrology 203:  1 - 10.

20. Rincon, B., Borja, R., Martin, M.A., Martin, A. (2009). Evaluation of the methanogenic step of a two-stage anaerobic digestion process of acidified olive mill solid residue from a previous hydrolytic-acidogenic step. Waste Management 29: 2566 - 2573.

21. Christensen, T., Kjeldsen, P., Bjerg, P., Jensen, D., Christensen, J., Baun, A. (2001). Biogeochemistry of landfill leachate plumes. Applied Geochemistry 16: 659 - 718.

