The Malaysian Journal of Analytical Sciences Vol 17 No 2 (2013): 291 – 299

OPTIMIZATION of A distillation unit in terms of POTENTIAL ENVIRONMENTAL IMPACT AND ECONOMICS
(Pengoptimuman Unit Penyulingan dari Segi Potensi Impak Ekonomi dan Alam Sekitar)

Alhassan Salami Tijani1*, Naveed Ramzan2

1Faculty of Mechanical Engineering 

Universiti Teknologi MARA, 40450 Shah Alam, Selangor, Malaysia.

2Department of Chemical Engineering, 

University of Engineering and Technology, G.T Road Lahore (54890) Lahore-Pakistan

*Corresponding author: alhassan@salam.uitm.edu.my
Abstract

Process energy integration and continuous improvement of process technology are increasing issues to ensure profitability of chemical productions. These objectives are increasingly important due to long-term environmental impact of energy degradation, such as resource depletion, emissions and the release of “waste” heat. The earlier energy conservation, process economics and environmental aspects are integrated into the process development, the easier and less expensive it is to improve the process design. In this work different distillation process design alternatives have been considered with respect to evaluations of process economics and potential environmental impacts. “Optimum design alternatives” are analyzed related to these objectives. A multi-criteria decision making technique such as (Analytic Hierarchy Process) AHP is applied for ranking the alternatives. This method reveals that the heat pump distillation unit which has the highest score of 52 % is the best alternative when compare with base case. In terms of the effluent streams the base case has a less potential environmental impact (PEI) compared with heat pump. The lower total PEI/kg (7.45E-01) of the base case illustrates that the material utilization efficiency of the base case is better than the heat pump whose PEI/kg is 8.14E-01.

Keywords: economic, thermodynamic efficiency, distillation unit, waste reduction (WAR) algorithm
References
1. Hui Chen, Ya Wen, Michelle D. Waters, and David R. Shonnard. (2002). Design Guidance for Chemical Processes    Using Environmental and Economic Assessments. Ind. Eng. Chem. Res, 41, 4503-4513.

2. Teresa M. Mata, Raymond L. Smith, Douglas M. Young, Carlos A.V. Costa. (2003). Evaluating the environmental friendliness, economics and energy efficiency of chemical processes: heat integration, Clean Techn Environ Policy 5, 302–309.

3. Dantus, M. M.; High, K. A. (1996). Economic Evaluation for the Retrofit of Chemical Processes through Waste Minimization and Process Integration. Ind. Eng. Chem. Res., 35, 4566.
4. Cano-Ruiz, J. A.; McRae, G. J. (1998). Environmental Conscious Chemical Process Design. Annu. Rev. Energy Environ. 23,499.
5. Lou, H.H., Kulkarni, M.A., Singh, A. and Hopper, J.R. (2004). Sustainability assessment for industrial systems. Ind Eng Chem Res, 43(15): 4233–4242.
6. Achour, M.H., Haroun, A.E., Schult, C.J., et al. (2005). A new method to assess the environmental risk of a chemical process Gasem. Chem Eng Process, 44(8): 901–909.
7. Hau, J.L., Yi, H.S. and Bakshi, B.R., (2007). Enhancing life cycle inventories via reconciliation with the laws of thermodynamics. J Ind Ecol, 11(4): 1–21
8. J. Rafael Alca´ntara-Avila, Manabu Kano, Shinji Hasebe. (2012). Environmental and economic optimization of distillation structures to produce anhydrous ethanol, 22nd European Symposium on Computer Aided Process Engineering, 30: 712–716. 

9. Wang, M., Shi, Y., Xia, X., Li, D., Chen, Q. (2013). Life-cycle energy efficiency and environmental impacts of bioethanol production from sweet potato. doi: http:// dx.doi.org/10.1016/j.biortech.01.067.

10. Gomez-Castro, F. I., Rico-Ramirez, V., Segovia-Hernandez, J. G., Hernandez-Castro, S., & El- Halwagi, M. M. (2013). Simulation Study on Biodiesel Production by Reactive Distillation with Methanol at High Pressure and Temperature: Impact on Costs and Pollutant Emissions, Computers and Chemical Engineering doi:10.1016/j.compchemeng.01.007.

11. Hilaly AK, Sikdar SK. (1994) Pollution balance: a new method for minimizing waste production in manufacturing processes. J Air Waste Manag. Assoc 44:1303–1308.

12. Young DM, Cabezas H. (1999). Designing sustainable processes with simulation: the waste reduction (WAR) algorithm. Comput Chem Eng 23:1477–1491.

13. Douglas Young, Richard Scharp, Heriberto Cabezas. (2000). The waste reduction (WAR) algorithm: environmental impacts, energy consumption, and engineering economics Waste Management 20 (2000) 605±615.
14. Seider, W. D., Seader, J.D., Lewin, D.R., 2003, Product and Process Design principles, Synthesis, Analysis and Evaluation, John Willey & Sons, Inc., New York. Pg. 142-145.
15. Douglas, J. M., (1988). Conceptual Design of Chemical Processes. McGraw-Hill Book Company, New York, pg.425-432.
16. Satty, T. (1980). The Analytic Hierarchy Process; McGraw-Hill: New York, pg. 560-601.
