discrimination of black ballpoint pen inks by HIGH PERFORMANCE LIQUID CHROMATOGRAPHY (HPLC)
(Diskriminasi Pen Mata Bola Berdakwat Hitam Menggunakan Kromatografi Cecair Berprestasi Tinggi)

Mohamed Izzharif Abdul Halim*, Norashikin Saim , Rozita Osman , Halila Jasmani , 
Nurul Nadhirah Zainal Abidin
Faculty of Applied Sciences 

Universiti Teknologi MARA, 40450 Shah Alam,Selangor
*Corresponding author: izzharif@salam.uitm.edu.my

Abstract
In this study, thirteen types of black ballpoint pen inks of three major brands were analyzed using high performance liquid chromatography (HPLC). Separation of the ink components was achieved using Bondapak C-18 column with gradient elution using water, ethanol and ethyl acetate. The chromatographic data obtained at wavelength 254.8nm was analyzed using agglomerative hierarchical clustering (AHC) and principle component analysis (PCA). AHC was able to group the inks into three clusters. This result was supported by PCA, whereby distinct separation of the three different brands was achieved. Therefore, HPLC in combination with chemometric methods may be a valuable tool for the analysis of black ballpoint pen inks for forensic purposes. 
Keyword: Ballpoint inks, principle component analysis, HPLC, chemometric

Abstrak

Dalam kajian ini, tiga belas jenis dakwat pen mata bulat daripada tiga jenama di analisis menggunakan kromatografi cecair prestasi tinggi (HPLC). Pemisahan komponen-komponen dakwat diperolehi menggunakan turus “Bondapak C-18” secara elusi berperingkat mengunakan kombinasi air, etanol dan etil asetat. Data kromatografi yang diperolehi pada panjang gelombang 254.8 nm dianalisis menggunakan pengelompokan hierarki agglomerative, (AHC) dan analisis komponen prinsip, (PCA). AHC membolehkan sample dakwat dikelaskan kepada tiga kelompok. Hasil daripada analisis AHC disokong oleh PCA yang juga menunjukkan pemisahan yang jelas bagi tiga kelompok mengikut jenama. Oleh itu, gabungan di antara HPLC dengan kaedah kemometrik merupakan teknik yg berguna untuk analisis dakwat pen mata bulat bagi tujuan forensik.

Kata kunci: Dakwat pen mata bulat, analisis komponen prinsip, HPLC, kimometrik

Introduction
Over recent years, technological developments have rapidly expanded the range of pen designs and ink composition used throughout the world [1].Quill and nib pens have given way to other classes of writing instruments such as the ballpoint, roller ball and gel pens which have different requirements for the properties of their writing inks [2]. These inks may contain many substances aiming to improve ink characteristic [3].  Although all black ballpoint pen inks may look the same, there can be some important differences in their chemical composition, constituent and substances. This study emphasizes on black ballpoint pen inks available in Malaysian market as these pens are often used in the occasion of signing important documents.                Black ballpoint pen inks are also used in documents with significant financial value like checks, insurance claims, wills, contract and tax return [4]. The examination of inks is often performed to evaluate the authenticity of these document. A ballpoint pen ink consists of synthetic dyes in (acidic and/or basic), pigments (organic and/or inorganic) and a range of additives [5]. Inks of similar color may consist of different dye composition and are frequently the subjects of forensic examinations. The inks can be discriminated by non-destructive or destructive methods [3, 6] depending on whether a sample needs to be taken from the document, a process that would alter it. Destructive methods include high performance liquid chromatography (HPLC), thin layer chromatography (TLC), capillary electrophoresis (CE), gas chromatography/mass spectroscopy (GC/MS) and fourier transform infrared (FTIR) Spectroscopy [2, 3, 6]. Non-destructive methods include scanning electron microscopy (SEM), video spectral comparative (VSC) and Raman Spectroscopy [3, 6, 7]. In this study, HPLC was used as this method is able to provide detailed information regarding the components of the inks. The data obtained was subjected to chemometric methods as these techniques allow the extraction of more information based on the similarities and differences among samples in a data set [8]. The application of chemometric techniques such as agglomerative hierarchical clustering (AHC) and principle component analysis (PCA) helps in the interpretation of complex chromatographic data sets to better understanding of the studied subject. The goal of AHC is to identify relatively similar, that is, homogeneous, group of objects (ink samples) in the space of measured features. Combination of PCA and AHC could be used to investigate the measurable variability between the same colors of pens of different brands by matching the ink’s components provided in the chromatograms.
Experimental
Chemicals and Reagents
Ethanol 95% was purchased from Merck (Darmstadt, Germany), Ethyl acetate and acetone were obtained from System (ChemAR). 
Samples

Thirteen black ballpoint pens of three major brands consisting of different types were purchased from local stores (Table 1).

Table 1 Types of pens from three major brands
	Sample
	Brand
	Type

	CbH1
	CARERA
	Carera B3 II

	CbH2
	CARERA
	Carera B7

	CbH3
CbH4
SbH1
SbH2
SbH3
SbH4

SbH5
PbH1
PbH2
PbH3

PbH4
	CARERA
CARERA
STABILO
STABILO
STABILO
STABILO
STABILO
PILOT
PILOT
PILOT
PILOT
	Carera B8
Carera B6
Stabilo galaxy 818 F
Stabilo liner 308 F
Stabilo liner 308 M
Stabilo galaxy 818 XF

Stabilo Point 88
Pilot BP – SF
Pilot BP – SM
Pilot supergrip M

Pilot supergrip F


Sample preparation
Ink samples were obtained by cutting 1.0 cm of each ink barrel. Each ink barrel was placed into different vial and 5 ml of ethanol was added into each vial. The sample was sonicated for about 15minutes and the solution was analyzed by HPLC.
High Performance Liquid Chromatography (HPLC)
The gradient elution employed for HPLC separation is shown in the Table 2. The column used was µ Bondapak C-18, 30 cm  x  3.9mm. Detection was done by a dual wavelength UV monitoring at 254.8 nm.

Table 2 Gradient elution for HPLC system
	Time range
	Solvent Composition
	Flow Rate
	Temperature

	0 min – 8 min
8 min – 13 min
13 min – 25 min
	water-ethanol-ethyl acetate (55:40:5)
water-ethanol-ethyl acetate (40:40:20)
water-ethanol-ethyl acetate (35:40:25)
	 1.0ml/min
1.0ml/min
1.5ml/min
	27˚C
27˚C
27˚C


Chemometric analysis of the HPLC data

The selected retention times for each sample were subjected to AHC and PCA using XLSTAT 2012 software.
Result and Discussion
Fig. 1 shows a chromatogram obtained from one of the ink samples. Major components selected (based on retention times ) in black ballpoint pen inks are shown in the Table 3.
[image: image1.png]fa Instrument 1 (offline): Data Analysis
Fie Method Sequence Graphics Integration Calbration Report Spectra Batch View Abort Hel

st [ | vetnoss (3 bedy (13 (B copremeraceTammomENLICAIM
tearentneos = L Dl

2
sa b b2 4 | @ © O | [ ResdRepiosess Data Made |

=

& e 2012082

& e izoss . DateTime Operator vial DataFile SompleName  /  Method Name Man..  Sample Info Sample Am... ISTD Amount &
5 sngluns I 1pfoun 12:0000 At o PEANATONN002.D ~ runt o o 2

0 LWATER SAMPLELICA a _2/19/2013 3:16:52 PM  SYAFINAZ o PPEANAT000000.D SMPLE 1 SYAFINAZM - RUN 1 o o

52 METSLLFLRON A onopniaamanaom  saemier o ceountmont et 1 voemin ST . o 9

£ noueo70213

3 NoLIEL40213 2 Calibration [\ Signal [ Purty 1, Spectum
2 voLtE-2u

Aot Ao D U [ | Reports  shon 2908 S 4§ oo seoss. avarooonsnn) = [ @ @ | % (A (1 70 D[] o e

- NOLI-FRACTION UGL
£ NOLL-SAE i DAD1 A, 5ig=254,8 Ref=360,100 (SHAFINAZ\PEANATOD0000.0)
Snousm
2 how TRy
Snouz
22 panaquAT
0 SAMPLE UGL
= s

§ single Runs
e
2w
0 WaNI12032012
2
0 ZUHAIRID20412
0 ZUHAIRT100112
0 2UHAIRIZ21211
=G 20111

Dats |Methods

W £ BRI R B [ (o) 7 0 ] v = ]

B
i formation i Tme  mea Homn  wiah Aest symmew
o b O e - - 3 7
i e N AT N T
J Report Layout Date [19Feb-13, 15:16:52 ] 3 2816 35103 2934 01836 4222 0,88
SR N N 1 T
St i T ¥
&) verincation 00/P¥) oo S 5a et ot oo | siw | orer
2R et P - B s T
(B pingnosis Hethod SVAFNAZT N T 7 2o i | o
e P e S EET A

Tt @resty

Instrument 1 ( oo, pef




Figure 1 Chromatogram of ink sample (PbH4)
Table 3 Major components selected for chemometric analysis
	Retention time range
	Assigned component name

	1.2 – 1.5 min
2.1 – 2.3 min
2.3 – 2.6 min
2.6 – 2.9 min
3.6 – 3.9 min
4.3 – 4.6 min
5.0 – 5.3 min
5.3 – 5.6 min
11.8– 12.0 min
12.0 – 15.0 min
	Component A
Component B
Component C
Component D
Component E
Component F
Component G
Component H
Component I
Component J


Agglomerative Hierarchichal Clustering (AHC)

AHC is a process of subdividing a set of entities into subsets in which the members are similar to each other, but different from members of other subsets. A shown in the dendogram in figure 2, three clusters representing the three brands of inks were obtained. The various types of inks from Stabilo pens were grouped in cluster 1. Cluster 2 and 3 is formed by inks from Carera and Pilot pens, respectively. Therefore, the AHC analysis on the HPLC chromatogram data indicates that this approach is suitable to classify inks of different brands. 

[image: image2.png]Dendrogram

[ ]

500

50

00

350

Lrrepumssiq

300
51

150

100

50

rHAd

THAd

€HAd

THAd

€HAD

THAD

THAD

THID

SHAS

THAS

€HAS

THAS

THAS





Figure 2 Dendrogram of inks samples based on HPLC data

Principal component analysis (PCA)
Principal component analysis provides information on the most meaningful parameters that describe the whole data sets rendering data reduction with a minimum loss of original information [9]. The PCA technique allowed the identification of an association between variables, thus reducing the dimensionality of the data sets. It is a powerful technique for pattern recognition that attempts to explain the variance of a large set of inter-correlated variables and transform them into a smaller set of independent (uncorrelated) variables (principal components). Based on the eigenvalue criterion, only the PCs with eigenvalue greater than one are considered important. This criterion is based on the fact that the average eigenvalue of the autoscaled data is just one [10].

PCA analysis, applied to HPLC inks sample datasets, resulted in three latent factors explaining 76.83% of the total variance. Figure 3 shows the PCA scores plot obtained showing a clear separation of ink samples from different brands for the first two principle components (F1 and F2). All the Stabilo inks are clustered in the left region of the plot. Most of the Carera inks are grouped in the upper-right region, white Pilot inks are clustered together in the lower-right part of the plot. The results are strongly supported the AHC analysis discussed earlier.

[image: image3.png]F2(34.53%)

-12

Observations (axes F1 and F2: 76.83 %)

-16

@ CbH2 CbH3
49 CbHL
CbH4
ShH2aShHS
W Sohi
SbH3SbH4
@ PbH4
@ Pbiil
PbH3
@ PbH2
-12 -8 -4 0 4 8

F1(42.30 %)

12





Figure 3 Score plot of F1 versus F2

Conclusion
In this study, the application of chemometric techniques (agglomerative hierarchical clustering and principle component analysis) on HPLC data can successfully evaluate the variations of thirteen types of black ballpoint pen inks of different brands. These results suggest that the HPLC profile of inks of different brands differ due to their similarities and differences in the chemical compositions. AHC grouped the various types of inks into three clusters of similar ink component’s characteristic of each brand. The results concurred with the PCA. Combination of HPLC and chemometric techniques provided a more objective interpretation of the results for the analysis of black ballpoint pen inks for forensic purposes.
Acknowledgement
The authors would like to acknowledge the financial support obtained from Ministry of Higher Education (MOHE), Malaysia for this project (Project number: 600-RMI/ST/FRGS/5.3.Fst/6/2010) and Universiti Teknologi MARA. 
References
1. Adam, C.D., Sheratt, S.L. & Zholobenko, V.L. (2008). Classification and individualization of black ballpoint pen inks using principle component analysis of UV-vis absorption spectra. Forensic Science International, 174(1): 16-25.
2. Wahab Williams, M.R., Moody, C., Arceneaux, L.A., Rinke, C., White, K. & Sigman, M.E. (2009).  Analysis of black writing ink by electrospray ionization mass spectrometry. Forensic Science International, 191(1-3):       97-103.
3. Thanasoulias, N.C., Parisis, N.A. & Evmirids, N.P. (2003). Multivariate chemometrics for the forensic discrimination of blue ball-point pen inks based on their Vis Spectra. Forensic Science International, 138(1-3):      75-84.
4. Djozan, D., Baheri, T., Karimian, G. & Shahidi, M. (2008). Application of the micro–FTIR spectroscopy, Raman spectroscopy and XRF method examination of inks.  Forensic Science International, 158(2-3): 164-172.
5. Denman, J. A., Skinner, W.M., Kirkbride, K..P. & Kempson, I.M. (2010). Organic and inorganic discrimination of ballpoint pen inks by ToF-SIMS and multivariate statistics. Applied Surface Science, 256(7): 2155–2163.
6. Dirwono, W., Park, J.S., Agustin-Camacho, M.R., Kim, J., Park, H-M., Lee, Y. & Lee, K-B. (2010). Application of micro-attenuated total reflectance FTIR spectroscopy in the forensic study of questioned documents involving red seal inks. Forensic Science International, 199(1-3): 6-8.
7. Senvaitiene, J. & Beganskiene and Kareiva, A. (2005). Spectroscopic evaluation and characterization of different historical writing inks. Vibrational Spectroscopy, 37(1): 61-67.
8. Gallidabino, M., Weyermann, C. & Marquis, R. (2011). Differentiation of blue ballpoint pen inks by positive and negative mode LDI-MS. Forensic Science International, 204(1-3): 169-178.
9. Kowalkowski, T., Zbytniewski, R., Szpejna, J. & Buszewski, B. (2006). Application of chemometrics in river water classification. Water Research, 40(4): 744-752.
Cluster  1





Cluster  2





Cluster  3





PILOT





CARERA





STABILO








