Malaysian Journal of Analytical Sciences Vol 18 No 1 (2014): 185 – 194

DETERMINATION OF SUPPORTED 222Rn ACTIVITY

CONCENTRATION IN GROUND WATER FROM

CAMERON HIGHLANDS AREA USING GAMMA SPECTROMETRY

(Penentuan Kepekatan Aktiviti 222Rn Sokongan di dalam Air Tanih dari Kawasan Cameron Highlands Menggunakan Spektrometri Gama) 
Ahmad Saat1,2, Nor Shazlina Zainal1*, Zaini Hamzah1 and Abdul Khalik Wood1
1Faculty of Applied Sciences, 

Universiti Teknologi MARA, 40450 Shah Alam, Selangor, Malaysia

2International Education College (INTEC), 

UiTM Kampus Seksyen 17, 40200 Shah Alam, Selangor, Malaysia

*Corresponding author: elinshazlinazainal89@gmail.com
Abstract

Humans may be exposed to the emission of energetic alpha particle from supported radon decaying process in ground water when it is consumed. Cameron Highland’s water supply was chosen as study area since it coming from hilly area of Gunung Pass, Gunung Brinchang and Gunung Cantik whose geological formations made from mainly igneous rocks (intrusive rocks, mainly granite with minor granodiorite) that flows into Sungai Terla, Sungai Ulu Bertam and Sungai Jasar. Determination of supported radon in this ground water was carried out using eighteen ground water samples collected from Cameron Highlands. Water samples (filtered and unfiltered) were poured into Marinelli beaker and closed tightly. These samples were kept at least for three weeks to allow the equilibrium between 226Ra and 222Rn. The measurement was accomplished by measuring water samples using low background gamma spectrometry (ORTEC) using a hyper pure coaxial germanium detector linked to a multi-channel digital analyzer system. 222Rn was measured at its daughter’s photo peaks: 214Pb (352 keV) and 214Bi (609 keV). Results of the study show the supported 222Rn activity concentration ranges from 0.19 to 0.66 Bq/L for unfiltered, filtered and untreated samples, 0.32 to 0.39 Bq/L of 222Rn for treated water at water treatment plant and 0.26 to 0.55 Bq/L of 222Rn for domestic water. These values are lower than the activity concentration of radon in drinking water as proposed by USEPA (11 Bq/L). 

Keywords: Ground water, supported radon, natural radioactivity, gamma spectrometry
References
1. Zaini Hamzah, Ahmad Saat and Mohammed Kassim, (2011). Determination of Radon Activity Concentration in water using Gamma Spectrometry and Liquid Scintillation Counter Techniques. In: 3rd International Symposium and Exhibition in Sustainable Energy and Environment - ISESEE 2011, 1st-3rd June 2011, Melaka, Malaysia. 191 - 193.

2. Badhan, K, Mehra, R. and Sonkawade, R. G., (2010). Measurement of radon concentration in ground water using RAD7 and assessment of average annual dose in the environs of NITJ, Punjab, India. Indian Journal of Pure and Applied Physics, 44: 508 - 511.

3. Guidelines for Drinking Water Quality; in corporating first addendum, (2006). World Health Organization, 1, Recommendation, 3rd ed, 20 Avenue Appia, 1211 Geneva 27, Switzerland. 595 pp.
4. Ahmad Saat, Nor Shazlina Zainal and Zaini Hamzah, (2012). Assessment of Supported Radon in Ground Water from Highland Area Using Portable Continuous Radon Monitor. Journal of Nuclear and Related Technologies, 9 (1): 22 - 33. 

5. Ahmad Saat, Zaini Hamzah, Wan Muhammad W. A. Kadir, Zaharidah Abu Bakar, Zuraidah A Munir, Siti Mariam Sumari and Misbah Hassan, (2010). Some Remarks on Radon Concentration at Various Locations in Peninsular  Malaysia. Journal of Nuclear and Related Technologies, 7 (1): 49 - 57.
6. US Environmental Protection Agency, (1999). Radon in Drinking Water Health Risk Reduction and Cost Analysis; Notice.   Federal Register, USEPA 1999, 5th January, Washington, DC, US. 64 (38): 9560 – 9599.
7. Walia, V., Bajwa, B. S. and Virk, H. S., (2003). Radon monitoring in groundwater of some areas of Himachal Pradesh and Punjab states, India. Journal of Environmental Monitoring, 5: 122 - 125. 
8. Alessandro, W. D. and Vita, F., (2003). Ground water radon measurements in the Mt. Etna area. Journal of Environmental Radioactivity, 65: 187 - 201. 

9. Geological Map of Peninsular Malaysia, (2007). Director General of Minerals and Geoscience Malaysia.

10. E-map Pahang, (2008). Director of National Mapping, Malaysia.

11. Abdallah, S. M., Habib, R. R., Nuwayhid, R. Y., Chhatila, M. and Katul, G., (2007). Radon Measurement in Well and Spring Water Lebanon. Radiation Measurement, 42: 298 - 303. 
12. Osman, A. A. A., Salih, I., Shaddad, I. A., Din, S. E., Siddeeg, M. B., Eltayeb, H., Idriss, H. Hamza, W. and Yousif, E. H., (2008). Investigation of Natural Radioactivity Levels in Water around Kadugli, Sudan. Applied Radiation and Isotope, 66: 1650 - 1653. 
13. López, R. N., Segovia, N., Cisniege, M. G., López, M. B. E., Armienta, M. A., Seidel, J. L., Peña, P., Godínez, L. and Tamez, E., (2002). Determination of radon, major and trace elements in water samples from springs and wells of northern Mexico State, Mexico. Geofisica Internacional, 41: 407- 404.
14. Zaini Hamzah, Ahmad Saat and Mohammed Kassim, (2011). Determination of radon activity concentration in hot spring and surface water using gamma spectrometry technique. The Malaysian Journal of Analytical Sciences, 15 (2): 288 - 294.
15. Mohammed Kassim, (2012). Method Development and Determination of Radon Activity Concentration in Water using Gamma Spectrometry Technique. MSc Thesis, Universiti Teknologi Mara, Malaysia. 104 pp.

16. APHA, (1998). Standard Methods for Examination of Water and Wastewater (20th). Washington, DC: American Public Health Association.  

17. Ahmad Saat,  Zaini Hamzah, Mohammad Fariz Yusop and Muhd Amiruddin Zainal, (2010). Experimental Determination of the HPGe Spectrometer Efficiency Calibration Curves for Various Sample Geometry for Gamma Energy from 50 keV to 2000 keV. Progress of Physics Research in Malaysia – PERFIK, 2009.
18. Zaini Hamzah, Seh Datul Riduan and Ahmad Saat, (2011). Assessment of Radiation Health Risk in Cameron Highlands Tea Plantations. The Malaysian Journal of Analytical Sciences, 15 (2): 130 - 137. 

19. Talha, S.A., Lindsay, R., Newman, R.T, de Meijer, R.J., Maleka, P.P., Hlatshwayo, I.N., Mlwilo, N.A. and Mohanty, A.K., (2008). γ-Ray spectrometry of radon in water and the role of radon to representatively sample aquifers. Applied Radiation and Isotopes, 66: 1623 – 1626. 
20. National Water Quality Standard for Malaysia, (2010). Malaysia Environmental Quality Report. Department of Environment Malaysia (MDOE). 6 pp.

21. Chau, K.W., and Jiang, Y.W., (2002). Three-dimensional pollutant transport model for the Pearl River Estuary. Water Research, 36 (8): 2029 – 2039.

22. Makalahmad, A., Eisakhani, M. and Isa, M.H., (2008). Developing MIKE-11 Model for Water Quality Simulation in Bertam River, Cameron Highlands. In: International Conference on Construction and Building Technology – ICCBT 2008, 16th – 20th June 2008, Kuala Lumpur, Malaysia. 8 pp.
23. Eisakhani, M. and Makalahmad, A., (2009). Water Quality Assessment of Bertam River and its Tributaries in Cameron Highlands, Malaysia. World Applied Sciences Journal, 7 (6): 769 - 776.  

24. Lydie, R. M. and Nemba, R. M, (2008). Quantitative Determination of 226Ra and 228Ra in Reservoir and Tap Water in Yaounde Area, Cameroon. Global Journal of Environmental Research, 2 (3):  110 - 113. 

25. Lee, J. M. and Kim, G., (2006). A simple and rapid method for analyzing radon in coastal and ground waters using a radon-in-air monitor. Journal of Environmental Radioactivity, 89: 219 - 228. 
26. Radium, Radon and Uranium: Health Information Summary, (2007). Department of Environmental Services, New Hampshire (NHDES). 4 pp.
27. Guidelines for Drinking-water Quality, (2011). World Health Organization, 4th ed, WHO Press, 20 Avenue Appia, 1211 Geneva 27, Switzerland. 564 pp.

