Malaysian Journal of Analytical Sciences Vol 19 No 1 (2015): 77 – 87

ENHANCEMENT OF PALM OIL EXTRACTION USING CELL WALL DEGRADING ENZYME FORMULATION

(Peningkatan Pengekstrakan Minyak Kelapa Sawit Melalui Penggunaan Enzim Degradasi Dinding Sel)

Hemavathi Silvamany and Jamaliah Md Jahim*

Department of Chemical and Process Engineering,

Faculty of Engineering and Built Environment,

Universiti Kebangsaan Malaysia, 43600 Bangi, Selangor, Malaysia

*Corresponding author: jamal@eng.ukm.my

Abstract

In this recent work, application of aqueous enzymatic process to enhance recovery of palm oil was studied. Experiments were carried out to investigate the structural carbohydrate composition of oil palm mesocarp (Elaeis guineensis) and to analyze the effect of different combination of enzymes on the palm oil recovery and degree of digestibility and the respective correlation. The optimum combination of enzymes comprising of Cellic CTec2 (X1), Cellic HTec2 (X2) and Pectinex Ultra SP-L (X3) for Aqueous Enzymatic Oil Extraction Process (AEOEP), were determined using Simplex Lattice mixture design under fixed parameters. Maximum oil recovery of 88% was achieved with ratio of enzymes at 0.46: 0.34: 0.2 (X1:X2:X3), at enzyme loading of 30 mgprotein /10 gsubstrate, substrate loading of 50 %w/v, pH 4.8, and 2 hours of incubation at 50 ºC. The conversion of reducing sugar at corresponding condition was measured to evaluate the effectiveness of enzymes in degrading fruit cell wall releasing trapped oil. Moreover, transmission electron microscopy (TEM) was utilized to indicate the increase in cell wall disintegration leading to higher release of oil with enzymatic treatment.

Keywords:  aqueous enzymatic process, Elaeis guineensis, palm oil extraction, Cellic CTec2, Cellic HTec2, Pectinex Ultra SPL

Abstrak

Kajian ini bertujuan untuk menganalisis penggunaan proses enzimatik akueus dalam meningkatkan pemulihan minyak sawit. Eksperimen telah dijalankan untuk menentukan komposisi struktur karbohidrat dalam mesokarpa kelapa sawit (Elaeis guineensis) dan menganalisis kesan gabungan enzim berbeza ke atas pemulihan minyak sawit dan tahap penghadaman serta korelasi masing-masing. Gabungan enzim yang optimum terdiri daripada Cellic Ctec2 (X1), Cellic Htec2 (X2) dan Pectinex Ultra SPL (X3) untuk AEOEP ditentukan menggunakan reka bentuk campuran Simplex Lattice pada keadaan terkawal. Pemulihan minyak maksimum sebanyak 88% telah dicapai dengan nisbah enzim pada 0.46: 0.34: 0.2 (X1:X2:X3), pada pemuatan enzim 30 mgprotein /10 gsubstrat, pemuatan substrat 50% w/v, pH 4.8, dan pengeraman selama 2 jam pada suhu 50ºC. Penukaran gula penurun dikaji untuk menilai keberkesanan enzim dalam mengdegradasi dinding sel mesokarpa kelapa sawit untuk melepaskan minyak terperangkap. Selain itu, imbasan mikroskop transmisi (TEM) digunakan untuk menunjukkan peningkatan dalam perpecahan dinding sel yang membawa kepada pengeluaran minyak yang lebih tinggi dengan rawatan enzim.

Kata kunci: 	proses enzimatik akueus, Elaeis guineensis, minyak kelapa sawit, Cellic Ctec2, Cellic Htec2, Pectinex Ultra SPL

References

- Teoh, Y. P. and Mashitah, M. D. (2010). Cellulase Production by Pycnoporus Sanguineus on Oil Palm Residues through Pretreatment and Optimization Study.  Journal of Applied Sciences, 10 (12): 1036-1043.

- Ng, F. Y., Yew, F. K., Basiron, Y. and Sundram, K. (2011). A Renewable Future Driven with Malaysian Palm Oil Based Green Technology.  Journal of Oil Palm & The Environment, 2:1-7.

- Kok, S., Ong-Abdullah, M., Ee, G.C., Namasivayam, P. (2011). Comparison of nutrient composition in kernel of tenera and clonal materials of oil palm (Elaeis guineensis Jacq.). Food Chem., 129 (4): 1343-1347.

- Sime Darby Plantation (2014), Palm Oil Facts and Figures, accessible at : http://www.simedarby.com/upload/Palm_Oil_Facts_and_Figures.pdf, accessed in October 2014

- Teixeira, C. B., Macedo, G. A., Macedo, J. A., Silva, L. H. M. and Rodrigues, A. M. C. (2013). Simultaneous extraction of oil and antioxidant compounds from oil palm fruit (Elaeis guineensis) by an aqueous enzymatic process. Bioresources Technology, 129: 575-581.

- Tan, C. H., Ghazali, H. M., Kuntom, A., Tan, C. P. and Ariffin, A. A. (2009). Extraction and physicochemical properties of low free fatty acid crude palm oil. Food Chemistry, 113: 645-650.

- Rosenthal, A., Pyle, D. L. and Niranjan, K. (1996). Aqueous and enzymatic processes for edible oil extraction. Enzyme Microb. Technol., 19 (6): 402–420.

- Deng, Y., Pyle, D. L. and Niranjan, K. (1992). Studies of aqueous enzymatic extraction of oil from rapeseed. Agri. Eng. Rural Dev. I Conf Proc. International Academic Publishers, Beijing 1: 190-195.

- Passos, C. P., Yilmaz, S., Silva, C. M. and Coimbra, M. A. (2008). Enhancement of grape seed oil extraction using a cell wall degrading enzyme cocktail. Journal of Food Chemistry, 115: 48-53.

- Rovaris, A. A., Balsamo, G. M., Costa, A. C. O., Arisi, A. C. M., Micke, G. A., Piovezan, M., Petkowicz, C. L. O. and Amante, E. R. (2013). Chemical characterization of liquid residues from aqueous enzymatic extraction of soybean oil. Journal of Food Science and Technology, 51: 51-58.

- Soto, C., Chamy, R. and Zuniga, M. E. (2007). Enzymatic hydrolysis and pressing conditions effect on borage oil extraction by cold pressing. Journal of Food Chemistry, 102: 834-840.

- Jiang, L., Hua, D., Wang, Z. and Xu, S. (2010). Aqueous enzymatic extraction of peanut oil and protein hydrolysates. Journal of Food and Bioproducts Processing, 88: 233-238.

- Aliakbarian, B., Faveri, D. D., Converti, A. and Perego, P. (2008). Optimisation of olive oil extraction by means of enzyme processing aids using response surface methodology. Biochemical Engineering Journal, 42: 34-40.

- Najafian, L., Ghodsvali, A., Khodaparast, M. H. H. and Diosady, L. L. (2009). Aqueous extraction of virgin olive oil using industrial enzymes. Journal of Food Research International, 42: 171-175.

- Barrios, V. A., Olmos, D. A., Noyola, R. A. and Lopez-Munguia, C. A. (1990). Optimization of an enzymatic process for coconut oil extraction. Oleagineux, 45 (1): 35-42.

- McGlone, O. C., Lopez-Munguia, C. A. and Carter, J. V. (2006). Coconut oil extraction by a new enzymatic process. Journal of Food Science, 51 (3): 695-697.

- Perez, E.E, Fernadez, M.B., Nolasco, S. M. and Crapiste, G. H. (2013). Effect of pectinase on the oil solvent extraction from different genotypes of sunflower (Helianthus annuus L.) Journal of Food Engineering, 117: 393-398.

- Sineiro, J., Dominguez, H., Nunez, M. J. and Lema, J. M. (1997). Optimization of the enzymatic treatment during aqueous oil extraction from sunflower seed. Journal of Food Chemistry, 61 (4): 467-474.

- Cheah, S. C., Augustin, M. A. and Ooi, L C. L. (1990). Enzymatic extraction of palm oil. Palm Oil Res. Bull. Malaysia 20: 30-36.

- Rathi, C. L., Pradhan, S., Javvadi, S. and Wani, A. (2012). An enzyme composition and process for extracting oil from palm oil fruits. WO2012011130 A3.

- Bradford, M. M. (1976). A Rapid and Sensitive Method for the Quantitation of Microgram Quantities of Protein Utilizing the Principle of Protein-Dye Binding.  Analytical Biochemistry, 72 (1–2): 248-254.

- Sluiter, A., Hames, B., Ruiz, R., Scarlata, C., Sluiter, J., Templeton, D. and Crocker, D. (2011). Determination of Structural Carbohydrates and Lignin in Biomass.  Golden, CO:U.S. Department of Energy National Renewable Energy Laboratory.

- Yoshida, M., Ichinose, H., Liu, Y., Kaneko, S., Uchida, S., and Kawarada, K. (2008). Effect of cellulose crystallinity, hemicellulose and lignin on the enzymatic hydrolysis of Miscanthus sinensis to monosaccharides. Biosci. Biotechnol. Biochem., 72 (3): 805-10.

- Glauet, A. M. (1980). Fixation, Dehydration and Embedding of Biology Specimens, in Practical methods in electron microscopy, A M Glauet, ed (North-Holland, Amsterdam).

- Lamsal, B. P., Murphy, P. A., and Johnson, L. A. (2006). Flaking and extrusion as mechanical treatments for enzyme-assisted aqueous extraction of oil from soybeans. J.Am. Oil Chem. Soc., 83 (11): 973-979.

- Ricochon, G. and Muniglia, L. 2010. Influence of enzymes on the oil extraction processes in aqueous media. Journal of Technology-Innovation, 17(6): 356-359.

- Li, J., Zu, Y. G., Luo, M., Gu, C. B., Zhao, C. J., Efferth, T. and Fu, Y. J. (2012). Aqueous enzymatic process assisted by microwave extraction of oil from yellow horn (Xanthoceras sorbifolia Bunge.) seed kernels and is quality evaluation. Journal of Food Chemistry, 138: 2152-2158.

- Ho, L.S., Nair, A.,Yusof, H.M., Kulaveerasingam, H.and Jangi, M.S. (2014). Morphometry of Lipid Bodies in Embryo, Kernel and Mesocarp of Oil Palm: Its Relationship to Yield. American Journal of Plant Sciences, 5: 1163- 1173.

- Tzen, J.T.C. (2012) Integral Proteins in Plant Oil Bodies. ISRN Botany, 2012: 16.

- Owolarafe, O. K., and Faborode, M. O. (2007). Micro-structural characterization of palm fruit at sterilization and digestion stages in relation to oil expression. Journal of Food Engineering, 85: 598-605.

- Wi, S. G., Singh, A. P., Lee, K. H., and Kim, Y. S. (2005). The pattern of distribution of pectin, peroxidase and Lignin in the Middle Lamella of Secondary xylem fibres in Alfalfa (Medicago sativa). Annals of Botany, 95: 863-868.