Malaysian Journal of Analytical Sciences Vol 19 No 1 (2015): 251 – 260

THE INFLUENCE OF WATER IN WATER-MICROEMULSIFIED-DIESEL AND ITS IMPACT ON FUEL PROPERTIES AND ENGINE PERFORMANCE

(Pengaruh Air dalam Air-Termikroemulsi-Diesel dan Kesannya ke atas Sifat-sifat Bahan Api dan Prestasi Enjin)

Hastinatun Mukayat1, Ismail Ab. Raman3, Khairiah Badri1,2, Wan Mohd Faizal Wan Mahmood4, Suria Ramli1,2*

1School of Chemical Sciences and Food Technology,

2Polymer Research Center,

Faculty of Science and Technology,

Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

3Advanced Oleochemical Technology Division (AOTD),

Malaysian Palm Oil Board (MPOB),

6, Persiaran Institusi,Bandar Baru Bangi, 43000 Kajang, Selangor, Malaysia

4Department of Mechanical and Materials Engineering, Faculty of Engineering and Built Environment,

Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

*Corresponding author: su_ramli@ukm.edu.my

Abstract

This investigation has been performed to study the effect of water on water microemulsified diesel on the fuel properties and engine performance. Selective tests of engine performance were performed in order to compare the efficiency in the engine between formulated water microemulsified diesel and conventional diesel fuel. The formulated water microemulsified diesel used were diesel/T80/1-penthanol/water 60:20:15:5 wt% (System 1), 55:20:15:10 wt% (System 2) and 50:20:15:15 wt% (System 3). These fuels withstand stability over a month in storage. The result showed that the engine brake specific fuel consumption (BSFC) increased as the water concentration in the system increased. Instead of showing higher fuel consumption than diesel fuel, System 1 and System 2 fuels show good fuel properties that meet the ASTM requirements for cloud point, calorific value and pour point for biodiesel. Further investigation in the gas emission test must be carried out in order to finding its potential as alternative fuels.

Keywords: brake specific fuel consumption, engine performance, fuel consumption, water microemulsified diesel

Abstrak

Kertas kerja ini melaporkan hasil kajian kesan kandungan air ke atas bahan api termikroemulsi air-diesel pada sifat-sifat bahan api dan prestasi enjin. Ujian prestasi enjin yang terpilih telah dilakukan bagi membandingkan kecekapan antara bahan api termikroemulsi dan diesel konvensional di dalam enjin. Formulasi bahan api termikroemulsi yang digunakan ialah diesel/T80/1-pentanol/air 60:20:15:5 wt% (Sistem 1), 55:20:15:10 wt% (Sistem 2) dan 50:20:15:15 wt% (Sistem 3). Formulasi bahan api ini adalah stabil melebihi sebulan. Keputusan menunjukkan bahawa penggunaan bahan api tentu brek (BSFC) enjin meningkat dengan peningkatan kepekatan air di dalam sistem mikroemulsi. Walaupun keputusan menunjukkan bahawa bahan api termikroemulsi air-diesel memerlukan penggunaan bahan api yang lebih tinggi berbanding dengan bahan api diesel, tetapi bahan api Sistem 1 dan Sistem 2 menunjukkan sifat-sifat bahan api yang baik dan memenuhi piawai seperti takat keruh, nilai kalori dan takat tuang. Ujian pembebasan gas mesti dilakukan pada kajian akan datang bagi mengetahui potensinya sebagai bahan api alternatif.

Kata kunci: penggunaan bahan api tentu brek, prestasi enjin, penggunaan bahan api, air-diesel termikroemulsi

References

- Wang, F., Fang, B., Zhang, Z., Zhang, S. & Chen, Y. (2008). The Effect of Alkanol Chain on the Interfacial Composition and Thermodynamic Properties of Diesel Oil Microemulsion.  Fuel  87(12): 2517-2522.

- Ding, Z., Hao, A. & Wang, Z. (2007). Water-in-Gasoline Microemulsions Stabilized by Polyglycerol Esters.  Fuel  86(4): 597-602.

- Selim, M. Y. E. & Elfeky, S. M. S. (2001). Effects Of Diesel/Water Emulsion on Heat Flow and Thermal Loading in a Precombustion Chamber Diesel Engine.  Applied Thermal Engineering  21: 1565-1582.

- Lif, A. & Holmberg, K. (2006). Water-in-Diesel Emulsions and Related Systems.  Adv Colloid Interface Sci  123-126: 231-239.

- Jeong, I., Lee, K.-H. & Kim, J. (2008). Characteristics of Auto-Ignition and Micro-Explosion Behavior of a Single Droplet of Water-in-Fuel.  Journal of Mechanical Science and Technology  22(1): 148-156.

- Armas, O., Ballesteros, R., Martos, F. & Agudelo, J. (2005). Characterization of Light Duty Diesel Engine Pollutant Emissions Using Water-Emulsified Fuel.  Fuel  84(7-8): 1011-1018.

- Yahaya Khan, M., Abdul Karim, Z. A., Hagos, F. Y., Aziz, A. R. & Tan, I. M. (2014). Current Trends in Water-in-Diesel Emulsion as a Fuel.  Scientific World Journal  Article ID 527472.

- Awang, R. & May, C. Y. (2008). Water-in-Oil Emulsion of Palm Biodiesel.  Journal of Oil Palm Research  20: 571-576.

- Balat, M. & Balat, H. (2008). A Critical Review of Bio-Diesel as a Vehicular Fuel.  Energy Conversion and Management  49(10): 2727-2741.

- Nagarajan, R. & Ruckenstein, E. (2000). Molecular Theory of Microemulsions.  Langmuir  16(16): 6400-6415.

- Neuma De Castro Dantas, T., Da Silva, A. C. & Neto, A. a. D. (2001). New Microemulsion Systems Using Diesel and Vegetable Oils.  Fuel  80(1): 75-81.

- Broze, G. (1997).  Phase Equilibria. Liquid Detergents.  Surfactant Science Series  67: 35-65.

- Alahmer, A. (2013). Influence of Using Emulsified Diesel Fuel on the Performance and Pollutants Emitted from Diesel Engine.  Energy Conversion and Management  73: 361-369.

- Djordjevic, L., Primorac, M., Stupar, M. & Krajisnik, D. (2004). Characterization of Caprylocaproyl Macrogolglycerides Based Microemulsion Drug Delivery Vehicles for an Amphiphilic Drug.  International Journal of Pharmaceutics  271(1-2): 11-19.

- Fanun, M. (2007). Conductivity, Viscosity, Nmr and Diclofenac Solubilization Capacity Studies of Mixed Nonionic Surfactants Microemulsions.  Journal of Molecular Liquids  135(1–3): 5-13.

- Dantas Neto, A. A., Fernandes, M. R., Barros Neto, E. L., Castro Dantas, T. N. & Moura, M. C. P. A. (2011). Alternative Fuels Composed by Blends of Nonionic Surfactant with Diesel and Water: Engine Performance and Emissions.  Brazillian Journal of Chemical Engineering  28(3): 521-531.

- Roila, A. & May, C. Y. (2008). Water-in-Oil Emulsion of Palm Biodiesel.  Journal of Palm Research  20: 571-576.

- Badran, O., Emeish, S., Abu-Zaid, M., Abu-Rahma, T., Al-Hasan, M. & Al-Ragheb, M. (2010). Impact of Emulsified Water/Diesel Mixture on Engine Performance and Environment.  International Journal of Thermal and Environmental Engineering  3(1): 1-7.

- Alahmer, A., Yamin, J., Sakhrieh, A. & Hamdan, M. A. (2010). Engine Performance Using Emulsified Diesel Fuel.  Energy Conversion and Management  51(8): 1708-1713.

- Buyukkaya, E., Benli, S., Karaaslan, S. & Guru, M. (2013). Effects of Trout-Oil Methyl Ester on a Diesel Engine Performance and Emission Characteristics.  Energy Conversion and Management  69: 41-48.