Malaysian Journal of  Analytical Sciences Vol 19 No 1 (2015): 261 – 267

EFFECTS OF FUNCTIONAL GROUP OF NON-IONIC SURFACTANTS ON THE STABILITY OF EMULSION

(Kesan Perbezaan Kumpulan Berfungsi Surfaktan Bukan Ionik Terhadap Kestabilan Emulsi)

Nursakinah Ibrahim1,2, Ismail Ab Raman1, Muhammad Rahimi Yusop2*

1Advanced Oleochemicals Technology Division,

Malaysian Palm Oil Board,

No 6, Persiaran Institusi, Bandar Baru Bangi, 43000 Kajang, Selangor, Malaysia

2School of Chemical Sciences and Food Technology, Faculty of Science and Technology,

Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

*Corresponding author: rahimi@ukm.edu.my

Abstract

Three mixed blend non-ionic surfactants: fatty acid polyethoxylates (POE) (20), POE (20) sorbitan monooleate (or Tween 80) and fatty alcohol POE (25) bounded by 20 -25 moles of ethylene oxides (EO) mixed with Laureth-1 were choosen to analyse their different functional group and its properties to the emulsion. The oil phase used is palm methyl ester C12-18. The emulsification process was carried out at 52 ± 2˚C by using high speed homogeniser. Emulsifying properties were investigated through a hydrophyl-lipophyl balance (HLB) value range from HLB 10, 11, 12, 13 and 14. The formation and stability of the emulsions were characterized through mean droplet size, zeta potential and surface tension. This study shows blend fatty alcohol POE (25EO) with HLB 11±1 shows optimum mean droplet size 1.970 μm to 2.099 μm, stable zeta potential at -37.91 mV to -40.8 mV and low surface tension value at 31.186 mN/m to 32.865 mN/m.

Keyword: surfactant, Hydrophyl-lipophyl balance (HLB), polyoxyethelene (POE)

Abstrak

Tiga campuran surfaktan bukan ionik: asid lemak polietoksilat (POE) (20), POE (20) sorbitan monooleat (Tween 80) dan alkohol lemak POE (25) yang mempunyai 20 - 25 molekul etilena oksida (EO), dicampur dengan Laureth-1 telah dipilih untuk dianalisis sifat perbezaan kumpulan berfungsi dan sifat terhadap emulsi. Fasa minyak yang digunakan adalah metil ester sawit C12-18. Nilai keseimbangan hidrofilik-lipofilik (KHL) oleh ketiga-tiga campuran surfaktan dihasilkan pada KHL 10, 11, 12, 13 dan 14. Pembentukan dan kestabilan emulsi dicirikan melalui saiz titisan, keupayaan zeta dan tegangan permukaan. Hasil kajian menunjukkan campuran alkohol lemak POE (25EO) dengan KHL 11±1 mempunyai optimum saiz titisan 1.970 μm hingga 2.099 μm, keupayaan zeta yang stabil pada -37.91 mV hingga -40.8 mV dan tegangan permukaan rendah pada 31.186 mN/m hingga 32.865 mN/m.

Kata kunci: surfaktan, Nilai keseimbangan hidrofilik lipofilik (KHL), polietoksilat (POE)

References

- Tadros, T. (2004). Application of Rheology for Assessment and Prediction of the Long-term Physical Stability of Emulsions. Adv Colloid Interface Sci, 109: 227–258.

- Boyd, J., Parkinson, C. & Sherman, P. (1972). Factors Affecting Emulsion Stability, and the HLB Concept. J Colloid   Interface Sci, 41(2): 359–370.

- Schmidts, T., Dobler, D., Guldan, A., Paulus, N. & Runkel, F. (2010). Multiple W/O/W Emulsions — Using the Required HLB for Emulsifier Evaluation. Colloids Surfaces A: Physicochemical Eng Aspects, 372(1-3): 48–54.

- Griffin, W. (1954). Calculation of HLB Values of Non-ionic Surfactants. J Soc Cosmetic Chemist, 5: 249–256.

- Castro, M. P., Rojas, A. M., Campos, C. A. & Gerschenson, L. N. (2009). Effect of Preservatives, Tween 20, Oil Content and Emulsion Structure on the Survival of Lactobacillus Fructivorans in Model Salad Dressings. LWT - Food Sci Tech, 42(8): 1428–1434.

- Han, J. & Washington, C. (2005). Partition of Antimicrobial Additives in an Intravenous Emulsion and their Effect on Emulsion Physical Stability. Int J Pharmaceutics, 288(2): 263–71.

- Smaoui, S., Hlima, H. Ben, Jarraya, R., Kamoun, N. G. & Ellouze, R. (2012). Cosmetic Emulsion from Virgin Olive Oil: Formulation and bio-physical evaluation. African J Bio, 11(40): 9664–9671.

- Jiang, L. C. & Omar, D. (2011). Physicochemical Characterization of Nonionic Surfactants in oil-in-water (O/W) Nano-emulsions for New Pesticide Formulations. Int J Applied Sci Tech, 1(5), 131–142.

- Hino, T., Kawashima, Y. & Shimabayashi, S. (2000). Basic Study for Stabilization of w/o/w Emulsion and its Application to Transcatheter Arterial Embolization Therapy. Adv Drug Delivery Rev, 45: 27–45.

- Esquena, J. & Forgiarini, A. (2001). Formation and Stability of Nano-Emulsions in Mixed Nonionic Surfactant Systems. Progr Colloid Polym Sci, 118: 184–189.

- Wia, A. E. & Chibowski, E. (2002). Zeta Potential and Droplet Size of n-tetradecane/ethanol (protein) Emulsions. Colloids Surfaces B: Biointerfaces, 25: 55–67.

- Jin, X., Streett, D. a., Dunlap, C. a. & Lyn, M. E. (2008). Application of Hydrophilic–Lipophilic Balance (HLB) Number to Optimize a Compatible Non-ionic Surfactant for Dried Aerial Conidia of Beauveria Bassiana. Bio Control, 46(2): 226–233.

- Jiang, L. C. & Omar, D. (2011). Physicochemical Characterization of Nonionic Surfactants in oil-in-water (O/W) Nano-emulsions for New Pesticide Formulations. Int J Applied Sci Tech, 1(5): 131–142.

- Wang, L., Dong, J., Chen, J., Eastoe, J. & Li, X. (2009). Design and Optimization of a New Self-Nano Emulsifying Drug Delivery System. J  Colloid Interface Sci, 330(2): 443–448.

- Usón, N., Garcia, M. J. & Solans, C. (2004). Formation of water-in-oil (W/O) Nano-Emulsions in a water/mixed Non-ionic Surfactant/oil Systems Prepared by a low-energy Emulsification Method. Colloids Surfaces A: Physicochemical Eng Aspects, 250: 415–421.

- Street, B. (2013). Formation and Stability of an oil in water Emulsion containing Lecithin , Xanthan gum and Sunflower oil. Bio Control, 20(5): 2173–2181.

- Peng, L.-C., Liu, C.-H., Kwan, C.-C. & Huang, K.-F. (2010). Optimization of water-in-oil Nanoemulsions by Mixed Surfactants. Colloids Surfaces A: Physicochemical Eng Aspects, 370(1-3): 136–142.

- Orafidiya, L. O & Oladimeji, F. A. (2002). Determination of Required HLB Values of Some Essential Oil. Internatioal J Pharmaceutics, 237: 241-249.

- Bahloul, B., Lassoued, M. A. & Sfar, S. (2014). A Novel Approach for the Development and Optimization of Self Emulsifying Drug Delivery. International J Pharmaceutical, 466: 341-348

- Li, C., Mei, Z., Liu, Q., Wang, J., Xu, J. & Sun, D. (2010). Formation and Properties of Paraffin Wax Submicron Emulsions Prepared by the Emulsion Inversion Point Method. Colloids Surfaces A: Physicochemical Eng Aspects, 356(1-3): 71–77.

- Kundu, P., Agrawal, A., Mateen, H. & Mishra, I. M. (2013). Stability of oil-in-water Macro-emulsion with Anionic Surfactant: Effect of Electrolytes and Temperature. Chem Eng Sci, 102: 176–185.

- Lim, C. J., Mahiran, B., Dzolkifli, O., Basyaruddin, M. A. R., Salleh A. B. & Raja, N. Z. R. A. R. (2011). Self Assembly Behaviour of Alkylpolyglucoside (APG) in Mixed Surfactant-Stabilized. J Molecular Liquid, 158: 175-181.

- Mattia, C. D. D., Saccheti G., Mastrocola D., Sarker, D. K. & Pittia. P. (2010). Surface Properties of Phenolic Compounds and Their Influence on the Dispersion Degree and Oxidative Stability of Olive O/W Emulsion. Food Hydrocolloids, 24: 652-658.