ASSESSMENT OF SURFACE WATER QUALITY USING MULTIVARIATE STATISTICAL TECHNIQUES IN THE TERENGGANU RIVER BASIN

(Penilaian Kualiti Air Permukaan Menggunakan Teknik Statistik Multivariat bagi Lembangan Sungai Terengganu)

Aminu Ibrahim1*, Hafizan Juahir1, Mohd Ekhwan Toriman1, Adamu Mustapha2, Azman Azid1,

Hamza A Isiyaka1

# 1East Coast Environmental Research Institute

# Universiti Sultan Zainal Abidin, 21300 Terengganu, Malaysia

# 2Kano State University of Science and Technology, Wudil 3244 Kano, Nigeria

*Corresponding author: aminu479@gmail.com

Received: 23 November 2014; Accepted: 11 January 2015

Abstract

Multivariate Statistical techniques including cluster analysis, discriminant analysis, and principal component analysis/factor analysis were applied to investigate the spatial variation and pollution sources in the Terengganu river basin during 5 years of monitoring 13 water quality parameters at thirteen different stations. Cluster analysis (CA) classified 13 stations into 2 clusters low polluted (LP) and moderate polluted (MP) based on similar water quality characteristics. Discriminant analysis (DA) rendered significant data reduction with 4 parameters (pH, NH3-NL, PO4 and EC) and correct assignation of 95.80%. The PCA/FA applied to the data sets, yielded in five latent factors accounting 72.42% of the total variance in the water quality data. The obtained varifactors indicate that parameters in charge for water quality variations are mainly related to domestic waste, industrial, runoff and agricultural (anthropogenic activities). Therefore, multivariate techniques are important in environmental management.

Keywords: cluster analysis, discriminant analysis, principal component analysis,water quality, Terengganu river basin

Abstrak

Teknik multivariat statistik termasuk analisis kelompok, analisis diskrminan, dan analisis komponen prinsipal/analisis faktor telah digunakan untuk mengkaji perubahan dan pencemaran sumber ruang di lembangan sungai Terengganu sepanjang 5 tahun pemantauan parameter kualiti air di 13 tiga belas stesen yang berbeza. Analisis kelompok (CA)mengkelaskan 13 stesen ke dalam 2 kelompok iaitu rendah tercemar (LP) dan sederhana tercemar (MP) berdasarkan ciri-ciri kualiti air yang sama. Analisis diskriminan (DA) menberikan pengurangan data penting kepada 4 parameter (pH, NH3-NL, PO4 dan kekonduksian) dan penandaan betul adalah 95.80%. PCA/FA diaplikasikan kepada set data, menghasilkan lima faktor terpendam menyumbang kepada 72.42% daripada jumlah varians bagi data kualiti air. Varifaktor yang diperolehi menunjukkan bahawa terdapat parameter yang bertanggungjawab terhadap perubahan kualiti air terutamanya yang berkaitan dengan sisa domestik, perindustrian, aliran dan pertanian (aktiviti antropogenik). Oleh itu, teknik multivariat adalah penting dalam pengurusan alam sekitar.

Kata kunci: analisis kelompok, analisis diskriminan, analisis komponen principal, kualiti air, lembangan sungai Terengganu

Introduction

Water quality has turned into one of the major ecological concerns overall and is affected by common and anthropogenic unsettling influence, for example, wastewater, overflow effluents, land recovery, air testimony and environmental change [1]. Surface waters are helpless against contamination as a consequence of common techniques, namely, disintegration, precipitation data, weathering of crustal materials and anthropogenic exercises such as urban, industrial, horticultural exercises [2, 3, 4]. Lately, more consideration has been paid to surface water quality as a result of its solid linkage with human prosperity [5, 6]. The nature of stream anytime reflects a few significant impacts, including the lithology of the bowl, environmental inputs, and climatic conditions [7] and represented by both characteristic procedure and anthropogenic impacts [8].

In any area not yet influenced by human activity, the variability in natural water  quality depends on upon the mix of the accompanying environmental factors [9]: the occurrence of highly dissolvable or easily weathered minerals of which the request of weathering is halite, gypsum, calcite, dolomite, pyrite, olivine; the separation to the coastline; the rainfall or river runoff; the occurrence of peat swamps, wetlands and squashes which discharges extensive amounts of disintegrated natural matter, and different elements incorporate the surrounding temperature, thickness of  weathered rocks, natural soil spread.

Thus, wastewater from agriculture, industries and urban exercises and frequently common techniques, for example, disintegration and weathering debases water quality and debilitate their utilization for drinking, mechanical, farming, entertainment or different purposes [10]. Clean stream water is a crucial product for the prosperity of human social orders, and harm of inland aquatic system was one of the most genuine natural issues of the most recent century [11]. Since, river water forms the principle inland water asset for residential, industrial and farming purposes, it is basic to deflect and control rivers contamination [12] and to have certified data on water quality for successful administration.

Characterization of the spatial variety and source allotment of water quality parameters can deliver an enhanced understanding of the ecological circumstance and aid strategy producers to plan needs for practical water administration [13]. The level of water quality is dictated by the substance of physical, concoction and natural parameters accessible in it. Relationship between two parameters may cause to builds or abatement in the amassing of others. This affiliation or relationship is normally attained using multivariate factual methods [14, 15, 16].

However, the application of various multivariate statistical methods were used such as cluster analysis (CA), discriminant analysis (DA), principal component analysis (PCA) and factor analysis (FA) to interpret and revealed useful information from huge complicated data about water quality studies [2, 23]. Many studies have been carried out related to these methods they include: Assessment of Xianjing Watershed China using multivariate Zang et al [6] also Zhoa and Chui [17] used PCA and CA to identify the latent pollution source and classify the sampling stations. Similarly, Juahir et el [18] used multivariate methods such as CA, DA, PCA and FA to assess surface water quality of Kinta River, Malaysia. Moreover, multivariate techniques including CA, DA, PCA and FA were used by Shrestha and Kazama [7] to assess the water quality data set having 12 parameters of 13 stations of Fuji river basin from 1992-2002 to get temporal and spatial variations and identify latent pollution sources.

In the present study multivariate statistical techniques CA, DA, PCA/FA were applied to assess the spatial variation in the river water quality data sets of Terengganu River Basin. The objectives of the study are to fish out similarities of the sampling sites as well as water quality parameters using cluster and discriminant analyses. However, the study also identifies the possible pollution sources relating to spatial variation of the water quality data for Terengganu River Basin. The results of this study are required to be useful to streamline stream observing arrange and give a significant instrument in creating evaluation techniques for compelling water quality administration.

Materials and Methods

# Study Area

Terengganu river basin is located (40 41̍ – 50 20’N, 102031̍ – 1030 9 ̍ E), East Coast Peninsular Malaysia. It has a length of 100 km and a total catchment area of approximately 500 km2 [19]. Terengganu River basin where included Nerus River, Telemong River, Bereng River, and Pueh River. It originates from Lake Kenyir flows through Kuala Terengganu and flows into South China Sea (Figure 1).

The climate is tropical rainforest climate (Koppen Geiger Classification: Af), with no dry or cold season as it is constantly moist (throughout the year). The annual average temperature is 26.70 C (800F) and average monthly varies by 30C (5.40F). Total annual precipitation averages 2911mm (114.6 inches). More so, there are two types of monsoons the southwest monsoon season is usually occur in the latter half of May or early June and ends in September. The northwest monsoon season usually starts in early November and ends in March.

The Terengganu is having a population of over 1,125,000 in 2013 [20]. The study area is pristine environment in the upstream catchment region turning urbanized and industrialized downstream with the significant settlement of Kuala Terengganu city at the mouth. Making and paramount area uses incorporate woodland, business ranch (e.g., oil palm, coconut, and rubber, cocoa), agriculture, rural/urban settlements, past mining activities and industry [21]. The development of a hydroelectric power dam upstream has changed the hydro geochemical compartments comprising of the Kenyir Lake and the principle tributary of Terengganu River [19].

Therefore, the river is polluted by domestic and municipal waste, agricultural activities, run-off and industrial activities. In general, it is contaminated by point source pollution and non-point source pollution. Besides, is compulsory to threaten these problems through determining the variations in water quality [22, 23].

# Figure 1.  Showing study area and monitoring stations

# Water Quality Data Sets

Data sets of 13 water quality parameters and 271 observations (13 × 271), for a period of five years (2003 – 2007) were monitored at 13 stations of Terengganu river basin by Department of Environment, Malaysia (DOE). Preliminary work was conducted on the data sets following sorting station by station, assembling and transformation. Data transformation additionally aids to standardize the entire information set so as to satisfy the supposition of  bunch and element examination [24]. Non numerical variables were subjected to transformation.

The parameters monitored are dissolved oxygen (DO), biological oxygen demand (BOD), Chemical oxygen demand (COD), Suspended solid (SS), pH, ammoniacal nitrogen (NH3-NL), temperature (T), conductivity (EC), turbidity (Tur), nitrate (NO3), phosphate (PO4), E coli, and coliform. The data sets were administered to multivariate statistical techniques viz: cluster analysis (CA), discriminant analysis (DA) and principal component analysis PCA/FA [23, 25,26]. All statistical calculation was done through Microsoft office EXCEL 2007 and XLSTAT 2014.

# Cluster Analysis

CA is a system connected to gathering the information into groups or classes. The point is to create a set of clusters where by the item in the same cluster are like one another however unique in relation to those in different groups. Hierarchical agglomerate cluster is the most widely recognized methodology, which gives instinctive closeness relationship between any one example and the whole information set, and is regularly outlined by a dendogram (tree diagram) [27].

In this study hierarchical agglomerative CA was performed on the normalized data set by ward method, using Squared Euclidean distance as a measure of similarity. More so, the result of cluster (dendogram) gives a visual synopsis of grouping procedures, demonstrating a picture of the clusters and their vicinity with a lessening in dimensionality of the starting information [7].

# Discriminant Analysis

Discriminant analysis (DA) is utilized to characterize cases into downright needy qualities. It is utilized to focus variables that segregate between regularly happening gatherings [18]. Also it develops a discriminant function (DF) for each group using the raw data [28]. This is calculated as equation 1 below:

F (Gi) = Ki + ∑jn =1 WijPij	     					       		        (1)

where i is the number of groups, (G) Ki, the constant inherent to each group, n the number of parameters used to classify a set of data into a given group and wj is the weight coefficient assigned by DF analysis (DFA) to a given parameter (Pj) [ 23, 28, 29, 30].

DA depict the connections between two or more pre specified groups of examining substances focused around a set of two or additionally segregating variables. In any case, DA was performed on the first information utilizing the standard, forward stepwise and back ward stepwise modes. These were utilized to create DFs to survey spatial varieties in the river water quality. The stations (spatial) are the grouping (dependent) variables, while all the 11 measured water quality parameters are independents variables. DA develops a discriminant capacity (DF) for each one group given a few quantitative (independent) variables and straight out (dependent) variables [18, 23].

# Principal Component Analysis

The PCA is intended to change the original variables into new, uncorrelated variables (axis), known as essential components, which are direct consolidation of the original variables. It gives a goal method for discovering lists of this sort so that the variety in the information set can be represented as succinctly as could reasonably be expected [31]. PCA gives data on the significant parameter that portray most of the information set, managing information diminishment with least loss of unique data. The principal component (PC) can be expressed as equation 2 below:

Zij = ai1x1j + ai2x2j + ai3x3j + --------- + aimxmj						                                    (2)

where z is the component score, a is the component loading, x the measured value of variable, i is the component number, j the sample number of variables.

FA follows after vital part examination. The fundamental motivation behind FA is to abatement the commitment of less noteworthy variables to illuminate significantly a greater amount of the information structure originating from the PCA. This can be accomplished through rotating the pivot characterized by PCA as indicated by settled standards to build new variables, otherwise called varifactors (VF).

Principal component analysis utilized standardized variables to evacuate noteworthy PCs to further diminishing the commitment of variables with minor noteworthiness; these PCs were subjected to varimax rotation (raw) producing VFs [6, 25, 23, 26, 31, 33, 34]. The FA can be expressed as equation 3 below:

zji = af1f1i + af2f2i + af3f3i + ------------- + afmfmi + efi						         (3)

where z is the measured variable, a is the factor loading, f is the factor score, e the residual term accounting for errors or source of variation, i is the sample number and m the total number of factors.

Results and Discussion

The descriptive statistics of the river water quality data of Terengganu River Basin shows that coliform, e-coli, conductivity and turbidity possess the highest concentration of mean. Table 1 illustrates the concentration of each parameter.

# Table 1.  Descriptive statistic of the river water quality data

| Variables | Minimum | Maximum | Mean | Standard Deviation |
| --- | --- | --- | --- | --- |
| DO (mg/l) | 1.8 | 8.33 | 6.374 | 1.230 |
| BOD (mg/l) | 1.0 | 142 | 2.768 | 9.849 |
| COD (mg/l) | 15.0 | 329 | 24.339 | 22.937 |
| SS (mg/l) | 0.5 | 104 | 44.057 | 84.754 |
| pH (mg/l) | 3.2 | 8.8 | 6.683 | 0.787 |
| NH3-NL (mg/l) | 0.01 | 10 | 0.306 | 0.900 |
| TEMP (Deg) | 24 | 82 | 27.560 | 3.753 |
| COND ((mg/l) | 0 | 979 | 67.100 | 112.155 |
| TUR (NUT) | 0 | 626 | 53.502 | 82.405 |
| NO3 (mg/l) | 0.005 | 1.3 | 0.183 | 0.145 |
| PO4 (mg/l) | 0.005 | 1.3 | 0.037 | 0.100 |
| E-Coli (cfu/100ml) | 0 | 4100 | 3454.356 | 6282.769 |
| Coliform (cfu/100ml) | 0 | 9700 | 40802.659 | 100941.724 |

# Spatial similarities and Monitoring Stations Grouping

CA classify 13 monitoring stations in to two statistically significant clusters at (Dlink/Dmax) × 100 in a convincing way as the stations in these groups have similar and natural back ground of the water quality characteristics.The output of the cluster analysis is dispensed in dendogram Figure 2. Dendogram gives the picture of the clusters describing the spatial variation in the water. Grouped stations of each clusters is shown in Figure 2.

# Figure 2. Dendogram showing the clusters of the monitoring stations

Cluster 1 (stations 4TE01, 4TE02, 4TE03, 4TE04, 4TE05, 4TE06, 4TE07, 4TE08, 4TE09, 4TE10, 4TE12 and 4TE13) correspond to low polluted sites (LP). These stations are located in the upstream only station 4TE01 at downstream site but they have similar characteristics and natural background. Moderate development in that area resulted in land use activities application not completely been engaged. Hence the impact of human beings activities on the reverie environment is relatively low at this cluster, which makes the river water quality under preserves.

Cluster 2 (station 4TE11) correspond to moderate polluted sites (MP) located at the downstream. In this site land use activities were fully practiced. Hence the station has dense population coupled with settlements, commercial activities, industrial and horticulture which attributed to the contamination of the sites. More so, the station receives pollution from horticulture (rubber plantation, palm oil plantation and Pedi plantation), domestic wastewater, and industries.

The result of cluster analysis method shows it’s useful in presenting valid classification of surface water in the entire area and also will make it necessary to construct spatial sampling strategy for future in a good form, which can lessen the monitoring stations and corresponding coasts [7].

# Discriminant Analysis

DA was used in the study to identify the variables which discriminate between two clusters in the river water quality. Indeed, DA via standard, forward stepwise mode and backward stepwise mode, the validity of spatial classification using standard mode where 13 parameters are incorporated and give 97.42%.

The Wilks Lambda value test for the standard mode gives a 0.473 and P < 0.0001. The null hypothesis stated that the means of vectors of the 2 clusters (LPS and MPS) are equal. The alternative hypothesis, alongside, states that at least one of means of vector is different from another. Since the computed P – value is lower than the significance level of alpha = 0.05, one should reject a null hypothesis and accept the alternative hypothesis. The risk of rejecting the null hypothesis while it is true is lower than 0.01%. Thus, the 2 clusters are indeed different from one another.

Stepwise discriminant analysis was performed as an explanatory analysis to determine the most significant variables among the parameters. In forward stepwise mode variables are included one by one beginning with the more significant until no significant changes are obtained, whereas in backward stepwise mode, variables are removed one by one beginning with less significant changes until no significant changes are obtained [23].

However, forward stepwise mode gives 95.80% with 4 discriminant variables (pH, NH3-NL, PO4 and EC) whereas backward mode rendered 5 five discriminant variables (BOD, pH, NH3-NL, PO4 and EC) with correct assignation of 96.55%. Therefore, stepwise forward mode presents the most significant parameters with correct assignation that contributed variation in the river water quality. Table 2 shows the confusion matrix of the DA.

# Table 2.  Classification matrix by DA for spatial variation in Terengganu River Basin

| Sampling Regions | % Correct | Regions assigned by DA |  |  |
| --- | --- | --- | --- | --- |
|  |  | LPS | MPS | Total |
| Standard DA Mode |  |  |  |  |
| LPS | 98.60 | 254 | 2 | 256 |
| MPS | 60.00 | 6 | 9 | 15 |
| Total | 97.42 | 260 | 11 | 271 |
| Forward Stepwise Mode |  |  |  |  |
| LPS | 99.61 | 255 | 1 | 256 |
| MPS | 50.00 | 7 | 8 | 15 |
| Total | 95.80 | 262 | 9 | 271 |
| Backward Stepwise Mode |  |  |  |  |
| LPS | 99.60 | 255 | 1 | 256 |
| MPS | 55.50 | 6 | 9 | 15 |
| Total | 96.55 | 261 | 10 | 271 |

LPS – Low Polluted Site, MPS – Moderate Polluted Site

# Principal component analysis/factor (Source Identification)

In case of spatial variation, PCA/FA was applied on the 13 variables of the river water quality, in order to identify the latent pollution sources. An eigenvalue 1 or greater are considered significant [35]. Five varifactors with eigenvalues >1 were obtained (Figure 3) which explain 72.21% of the variance in data sets (Table 3), where a correlation greater than 0.75 is considered “strong”; 0.75-0.50, “moderate”; and 0.50-0.30, as “weak” significant factor loading [36].

The first varifactor explained 21.85% of the total variance with strong positive loading on NH3-NL and PO4 (Table 3). These factors represent the contribution of anthropogenic activities in the environment. The presence of PO4 originate from fertilizer application in the farms, agricultural land use strongly influences stream phosphorus. PO4 come from both point source and non-point source. NH3-NL indicates the contribution of organic pollution from domestic waste and agricultural areas.

The VF2 explained 19.85% of the total variance have strong positive loading factor on suspended solid, turbidity and moderate loading on NO3. This element clarifies the anthropogenic activities on the encompassing ranges by the physiochemical wellspring of variability [7]. This is evident as farmers practicing rubber plantation and palm oil plantation around the area. In these areas farmers utilizes nitrogenous compost, which experience nitrification forms, and the waterways get nitrate nitrogen by means of ground water draining. However, strong positive loading of turbidity with moderate loading of NO3 shows association of river runoff from agricultural field alongside waste transfer movement [37].

# Figure 3. Scree plot showing the eigenvalues

# Table 3.  Factor loadings after Varimax rotation

| Variables | VF1 | VF2 | VF3 | VF4 | VF5 |
| --- | --- | --- | --- | --- | --- |
| DO | -0.407 | 0.283 | -0.145 | 0.011 | 0.600 |
| BOD | 0.070 | 0.037 | 0.975 | 0.056 | -0.018 |
| COD | 0.119 | 0.101 | 0.966 | 0.084 | -0.015 |
| SS | -0.064 | 0.784 | 0.146 | 0.087 | -0.058 |
| pH | 0.074 | -0.073 | 0.035 | 0.029 | 0.883 |
| NH3-NL | 0.837 | -0.076 | 0.219 | 0.040 | -0.224 |
| Temperature | -0.120 | -0.453 | 0.010 | 0.018 | -0.334 |
| Conductivity | 0.488 | -0.011 | 0.063 | -0.022 | -0.671 |
| Turbidity | -0.057 | 0.808 | 0.171 | 0.107 | 0.017 |
| NO3 | -0.046 | 0.626 | -0.126 | 0.169 | 0.044 |
| PO4 | 0.887 | -0.007 | 0.041 | -0.053 | 0.075 |
| E-coli | -0.017 | -0.018 | 0.289 | 0.847 | 0.029 |
| Coliform | -0.007 | 0.200 | -0.061 | 0.883 | 0.013 |
| Eigenvalue | 2.840 | 2.581 | 1.493 | 1.292 | 1.182 |
| Variability % | 21.847 | 19.852 | 11.484 | 9.940 | 9.090 |
| Cumulative% | 21.847 | 41.699 | 53.183 | 63.123 | 72.213 |

Bold indicate strong and moderate factor loading

VF3 described 11.84% of the total variance had a strong positive loading on BOD and COD. These factors explained the effects of organic pollution and reflect strong influence of anthropogenic activities in the area, probably from domestic waste and industrial waste. VF4 account for 9.94% of the total variance and has a strong loading on E-coli and coliform. This factor is strongly related to waste from domestic and municipal in the area. The loading for factor 5 was 9.090% of the total variance had strong loading on pH and moderate loading on DO. This factor resulted due to the anaerobic conditions in the river from the strong loading of dissolved organic matter which leads in the formation of organic acids.

Conclusion

In this study multivariate statistical analysis were successfully applied to explore and identify the spatial variation and potential pollution sources in the river basin. CA grouped 13 sampling stations into 2 clusters LP and MP based on similar water quality characteristics. Following the result authorities and decision makers can develop optimal strategy in which sampling stations can be reduced. DA revealed significant data reduction as it gives four parameters (pH, NH3-NL, PO4 and EC) with 95.80% correct assignation. The PCA resulted in five varifactors with total variance of 72.21% in which the major sources of pollution are related to anthropogenic activities. Therefore, multivariate techniques are important in environmental management.

Acknowledgement

The author will like to thanks the department of Environment Malaysia (DOE) and the entire staff of East Coast Environmental Research Institute University Sultan Zainal Abidin for their support.

References

- Reza, R. & Singh, G. (2010). Heavy Metal Contamination and its Indexing Approach for River Water. International Journal of Environmental Science and Technology, 7(4): 785-792.

- Sundaray, S.K., Panda, U.C., Nayak B. B. & Bhatta D. (2006). Multivariate statistical techniques for the evaluation of spatial and temporal variations in water quality of the Mahanadi river-estuarine system (India)--a case study.Environmental Geochemistry and Health, 28(4): 317-330.

- Carpenter, S.R., Caraco, N.F., Correll, D.L., Howarth, R.W., Sharpley, A.N. & Smith, V. H.  (1998). Nonpoint pollution of surface waters with phosphorus and nitrogen.Ecology Application, 8: 559–568.

- Jarvie, H. P., Whitton, B. A., & Neal, C. (1998). Nitrogen and phosphorus in east coast British rivers: speciation, sources and biological significance. Science of the Total Environment, 211: 79–109

- Filik I. C., Emiroglu, O., Ilhan, S., Arslan, N., Yilmaz, V. & Ahiska, S. (2008). Application of multivariate statistical techniques in the assessment of surface water quality in Ulubat Lake. Turkey. Environmental Monitoring and Assessment, 144: 269-276.

- Zhang, Y., Guo, F., Meng, W. & Wang, X. (2009). Water quality assessment and source identification of Daliao river basin using multivariate statistical methods. Environmental Monitoring and Assessment, 152: 105-121.

- Shrestha, S. & Kazama, F. (2007). Assessment of surface water quality using multivariate statistical techniqueas: Fuji river basin Japan. Environnmental Modeling and Software, 22(4): 464-475.

- Pejman, A. H., Bidhendi, G. R., Karbassi, A. R., Mehrdadi, N. & Bidendi, M.E. (2009). Evaluation of spatial and seasonal variations in surface water quality using multivariate statistical techniques.International Journal of Environmental Science and Technology, 6(3): 467-476.

- Mayback & Helmer, (1998). The quality of rivers from pristine statge to global pollution.Palaeogr, palaeclamatol, palaeoecol. (Global planet sect) 75, 283-309.

- Kuppusamy M. R. & Giridhar V. V. (2006). Factor analysis of water quality characteristics including      trace metal speciation in the coastal environmental system of Chennai Ennore. Environment International, 32 (2): 174-179.

- Tanriverdi, C., Alp, A., Demirkiran, A. R. & Ucjarde, F. (2010). Assessment of surface water quality of the Ceyhan River basin, Turkey.Environmental Monitoring & Assessment,167, 175-184.

- Simeonova, P., Simeonova, V. & Andreev, G. (2003). Environmetric analysis of the Struma River water quality. Central European journal of Chemistry, 2: 121-126.

- Huang, J., Ho, M. & Du, P. (2011). Assessment of temporal and spatial variation of coastal water quality and source identification along Macau Peninsular.Stochastic Environmental Research and Risk Assessment, 25: 353-361.

- Ifabiyi, I.P. (1997). Variation in Water Gravity with Rainfall Incidences: A Case Study of Ogbe Stream Ile-Ife, Ife. Res. Publications Geogr, 6(1&2): 139-144.

- Jaji M.O., Bamgbose, O., Odukoya O.O. & Arowolo T.A. (2007). Water Quality Assessment of Ogun River, South West Nigeria.  Environment Monitoring Assessment, 133: 473-482.

- Mazlum, N., Ozer, A. & Mazlum, S. (1999). Interpretation of water quality data by principal components analysis.Turkish Journal of Environmental Science, 23: 19-26.

- Zhao, Z. & Cui, F. (2009). Multivariate statistical analysis for the surface water quality of the Luan River, China.Journal of Zhejiang University Science A, 10(1): 142-148.

- Juahir, H., Zain, M.S., Yosoff, M.K., Ismail, T.T.H., Samah, A.M.A., Toriman, M.E. & Moktar, M. (2010b). Spatial assessment of Langtan River Basin (Malaysia) using environmentric techniques. Environment Monitoring Assessment, 173: 625-641.

- Noor, A.S. & Khawar, S., (2010). Geochemical baselines of major, minor and trace elements in the tropical sediments of the Terengganu River Basin Malaysia. International Journal of Sediment Research, 25: 340-354.

- Malaysia population census Department of Statistic population of Terengganu (2014). Online access on www.statitics.gov.my/portal/index.php?option=com

- Kuala Terengganu city Council (MBKT), (2014). Online access on http://mbkt.terengganu.gov.my/jadual-pbt

- Tobiszewski, M., Tsakovski, S., Simeonov, V. & Namiesnik, J. (2010). Surface water quality assessment by the use of combination of multivariate statistical classification and expert information.Chemosphere,80: 740-746.

- Singh, K.P., Malik, A., Mohan, D. & Sinha, S. (2004). Multivariate statistical techniques   for the      evaluation of spatial and temporal variations in water quality of Gomti River (India): a case study. Water Research, 38: 3980-3992.

- Voudouris, K., Panagopoulos, A. & Koumantakis, J. (2000). Multivariate statistical analysis in the assessment of hydrochemistry of the Northern Korinthia Prefecture Alluvial Aquifer System (Paloponnese, Greece). Natural Resources Research, 9: 135-146.

- Simeonova, P., Simeonova, V. & Andreev, G. (2003). Environmetric analysis of the Struma River water quality central.European journal of Chemistry, 2: 121-126.

- Singh, K.P., Malik, A. & Sinha, S. (2005). Water quality assessment and apportionment of pollution sources of Gomti river (India) using multivariate statistical techniques: a case study. Analytical Chemical Acta 538, 355-374.

- McKenna Jr., J.E. (2003). An enhanced cluster analysis program with bootstrap significance testing for ecological community analysis. Environmental Modeling & Software, 18 (3): 205-220.

- Jornson, R.A. & Wichern, D.W. (1992).  Applied Multivariate Statistical Analysis, 3rdedn, Prentice – Hall lut., New Jersey.

- Lattin, J., Carroll, D. & Green, P. (2003). Analyzing Multivariate Data. New York: Duxbury.

- Wunderlin, D.A., Diaz, M.P., Ame, M.V., Pesce, S.F., Hued, A.C. & Bistoni, M.A. (2001). Pattern recognition techniques for the evaluation of spatial and temporal variations in water quality. A case study: Suquia river basin (Cordoba, Argentina). Water Research, 35(12): 2881-2894.

- Brumelis, G., Lapina, L., Nikodemus, O. & Tabors, G. (2000). Use of an artificial model of monitoring data  to aid interpretation of principal component analysis. Environmental modelling and software, 15(8): 755-763.

- Helena, B., Pardo, R., Vega, M., Barrado, E., Fernandex, J.M. & Fernendex, L. (2000). Temporal evolution of ground water composition in an alluvial aquifer composition in an alluvial aquifer (Pisuegariver, Spain) by principal component analysis.Water Research, 34: 807-816.

- Abdul-Wahab, S.A., Bakheit, C.S. & Al-Alawi, S.M. (2005). Principal component and multiple regression analysis in modeling of ground-level ozone and factors affecting its concentrations. Environmental Modelin & Software, 20(10): 1263-1271.

- Love, D., Hallbauer, D., Amos, A. & Hranova, R. (2004). Factor analysis as a tool in groundwater quality management: two southern African case studies. Physics & Chemistry of the Earth, 29: 1135-1143.

- Kim, J.O. & Mueller, C.W. (1987). Introduction to factor analysis: what it is and how to do it. Quantitative  Applications in the Social Sciences Series. Sage University Press, Newbury Park.

- Liu, C.W., Lin, K.H. & Kuo, Y.M. (2003). Application of factor analysis in the assessment of groundwater in a Blackfoot disease area in Taiwan.Science of the Total Environment, 313(1-13): 77-89.

- Yang, Y.H., Zhou, F., Guo, H.C., Sheng, H., Liu, H., Dao, X. & He, C.J. (2010). Analysis of spatial and temporal water pollution patterns in Lake Dianchi using multivariate statistical methods. Environmental Monitoring and Assessment, 170: 407-416.