Malaysian Journal of Analytical Sciences Vol 19 No 2 (2015): 309 – 317

DIFFERENTIAL PULSE ANODIC STRIPPING VOLTAMMETRY DETERMINATION OF COPPER (II) AT GLASSY CARBON ELECTRODE IN THE PRESENCE OF BIS(BENZYLIDENE)ETHYLENEDIAMINE AS A NOVEL COMPLEXING AGENT

# (Penentuan Kuprum(II) Menggunakan Voltammetri Perlucutan Anodik Denyut Pembezaan Dengan Elektrod Karbon Berkaca Serta Kehadiran Bis(Benzilidina)Etilinadiamina Sebagai Agen Pengkompleks)

Syaza Azhari, Rahmalan Ahamad*, Farediah Ahmad

Department of Chemistry, Faculty of Science,

Universiti Teknologi Malaysia, 81310 UTM Skudai, Johor, Malaysia

*Corresponding author: rahmalan@utm.my

Received: 6 November 2014; Accepted: 5 January 2015

Abstract

# This paper reports on the application of synthesized Schiff base bis(benzylidene)ethylenediamine (BBE) to increase the detection sensitivity of copper (II) at 0.01 V vs Ag/AgCl using glassy carbon electrode. Aliquot of 2.0×10-3 M of BBE in DMF solution containing BR buffer pH 6 was introduced into the voltammetric cell followed by the addition of Cu(II) ions and then further proceeded to differential pulse anodic-stripping scan from -1.4 V to +0.3 V vs Ag/AgCl. The optimum parameters obtained were; scan rate; 25 mV/s, accumulation time; 200 sec and accumulation potential; -0.6 V. Under the optimal conditions, the detection limit of 0.4 µgL−1 and a linear response in the range of 1-10 µgL−1 were obtained. The effects of interfering ions such as Cd(II), Cr(III), Fe(II), Pb(II) and Zn(II) were also investigated. The practical applicability of this technique was illustrated by the determination of Cu(II) ions at 3µgL-1 concentration level in tap water with 94.43% of recovery.

# Keywords: bis(benzylidene)ethylenediamine, copper ion, differential pulse anodic stripping voltammetry

Abstrak

# Kertas ini melaporkan aplikasi sintesis Schiff base bis(benzilidena)ethylenadiamina (BBE) untuk meningkatkan kepekaan pengesanan kuprum(II) pada 0.01 V vs Ag/AgCl dengan menggunakan electrod karbon berkaca.  Alikuot BBE (2.0 x10-3 M) dalam DMF dimasukkan ke dalam sel voltammetri yang mengandungi larutan buffer BR pada pH 6. Larutan Cu(II) kemudian ditambah dan di imbas menggunakan teknik voltammetri pelucutan anodik denyut pembezaan dari -1.4 V kepada +0.3 V vs Ag/AgCl. Parameter optimum ialah, kadar imabasan 25 mV/s, masa pengumpulan 200 saat dan keupayaan pengumpulan -0.6 V. Dibawah kondisi optimum ini, had pengesanan ialah 0.4 ugL-1 dan julat linear yang diperolehi ialah 1-10 ugL-1. Kesan ion–ion lain seperti Cd(II), Cr(III), Fe(II), Pb(II) dan Zn(II) juga dikaji. Kaedah ini telah diaplikasikan bagi menentukan Cu(II) pada kepekatan 3ugL-1 dalam air paip dengan perolehan semula sebanyak 94.43%.

# Kata kunci: Bis(benzylidene)ethylenediamine, ion kuprum, voltammetri pelucutan anodik denyut pembezaan

References

- Zhou, L., Gao, C. & Xu, W. (2010). Magnetic dendritic materials for highly efficient adsorption of dyes and drugs. ACS Applied Materials and Interfaces,  2 (5): 1483-1491.

- Bilici, E., Yazicigil, Z., Tok, M. & Oztekin, Y. (2012). Electrochemical determination of copper(II) using modified glassy carbon electrodes. Desalination and Water Treatment,  50 (1-3): 198-205.

- Gouda, A. A. & Amin, A. S. (2014). Cloud-point extraction, preconcentration and spectrophotometric determination of trace quantities of copper in food, water and biological samples. Spectrochimica Acta - Part A: Molecular and Biomolecular Spectroscopy,  120: 88-96.

- Beck, N. G., Franks, R. P. & Bruland, K. W. (2002). Analysis for Cd, Cu, Ni, Zn, and Mn in estuarine water by inductively coupled plasma mass spectrometry coupled with an automated flow injection system. Analytica Chimica Acta,  455 (1): 11-22.

- Mesquita Da Silva, M. A., Azzolin Frescura, V. L. & Curtius, A. J. (2000). Determination of trace elements in water samples by ultrasonic nebulization inductively coupled plasma mass spectrometry after cloud point extraction. Spectrochimica acta, Part B: Atomic spectroscopy,  55 (7): 803-813.

- Alemdaroǧlu, T., Onur, E. & Akgün, H. (2000). Determination of major and trace elements in sediments of Lake Egridir, Turkey. International Journal of Environmental Studies,  57 (2): 157-166.

- Sheibani, M., Marahel, F., Ghaedi, M., Montazerozohori, M. & Soylak, M. (2011). Preconcentration of some trace metal ions on coated alumina modified by 1-((6-(-(2-hydroxynaphthalen-1-yl)methyleneamino) hexylimino) methyl) naphthalen-2-ol. Toxicological and Environmental Chemistry,  93 (5): 860-872.

- Rajabi, M., Asghari, A. & Mousavi, H. Z. (2010). Trace amounts determination of lead, zinc and copper by adsorptive stripping voltammetry in the presence of dopamine. Journal of Analytical Chemistry,  65 (5): 511-517.

- Chaiyo, S., Chailapakul, O., Sakai, T., Teshima, N. & Siangproh, W. (2013). Highly sensitive determination of trace copper in food by adsorptive stripping voltammetry in the presence of 1,10-phenanthroline. Talanta,  108: 1-6.

- Trindade, J. M., Martiniano, L. C., Gonalves, V. R. A., Souza, A. G., Marques, A. L. B., Baugis, G. L., Fonseca, T. C. O., Song, C., Zhang, J. & Marques, E. P. (2012). Anodic stripping voltammetry coupled with design of experiments for simultaneous determination of Zn+2, Cu+2, Pb+2, and Cd+2 in gasoline. Fuel,  91 (1): 26-32.

- Zhu, W. W., Li, N. B. & Luo, H. Q. (2007). Simultaneous determination of chromium(III) and cadmium(II) by differential pulse anodic stripping voltammetry on a stannum film electrode. Talanta,  72 (5): 1733-1737.

- Gholivand, M. B., Pourhossein, A. & Shahlaei, M. (2011). Simultaneous determination of copper and cadmium in environmental water and tea samples by adsorptive stripping voltammetry. Turkish Journal of Chemistry,  35 (6): 839-846.

- Oztekin, Y. & Yazicigil, Z. (2009). Preparation and characterization of a 1,10-phenanthroline-modified glassy carbon electrode. Electrochimica Acta,  54 (28): 7294-7298.

- Oztekin, Y., Yazicigil, Z., Ramanaviciene, A. & Ramanavicius, A. (2011). Polyphenol-modified glassy carbon electrodes for copper detection. Sensors and Actuators, B: Chemical,  152 (1): 37-48.

- Oztekin, Y., Yazicigil, Z., Ramanaviciene, A. & Ramanavicius, A. (2011). Square wave voltammetry based on determination of copper (II) ions by polyluteolin- and polykaempferol-modified electrodes. Talanta,  85 (2): 1020-1027.

- Oztekin, Y., Tok, M., Bilici, E., Mikoliunaite, L., Yazicigil, Z., Ramanaviciene, A. & Ramanavicius, A. (2012). Copper nanoparticle modified carbon electrode for determination of dopamine. Electrochimica Acta,  76: 201-207.

- Bastos, M. B. R., Moreira, J. C. & Farias, P. A. M. (2000). Adsorptive stripping voltammetric behaviour of UO2(II) complexed with the Schiff base N,N'- ethylenebis(salicylidenimine) in aqueous 4-(2-hydroxyethyl)-1-piperazine ethanesulfonic acid medium. Analytica Chimica Acta,  408 (1-2): 83-88.

- Ghiaci, M., Rezaei, B. & Arshadi, M. (2009). Characterization of modified carbon paste electrode by using Salen Schiff base ligand immobilized on SiO2-Al2O3 as a highly sensitive sensor for anodic stripping voltammetric determination of copper(II). Sensors and Actuators, B: Chemical,  139 (2): 494-500.

- Losada, J., Del Peso, I. & Beyer, L. (2001). Electrochemical and spectroelectrochemical properties of copper(II) Schiff-base complexes. Inorganica Chimica Acta,  321 (1-2): 107-115.

- Oztekin, Y., Ramanaviciene, A., Ryskevic, N., Yazicigil, Z., Üstünda, Z., Solak, A. O. & Ramanavicius, A. (2011). 1,10-Phenanthroline modified glassy carbon electrode for voltammetric determination of cadmium(II) ions. Sensors and Actuators, B: Chemical,  157 (1): 146-153.

- Zolezzi, S., Spodine, E. & Decinti, A. (2002). Electrochemical studies of copper(II) complexes with Schiff-base ligands. Polyhedron,  21 (1): 55-59.

- Freire, R. S. & Kubota, L. T. (2004). Application of self-assembled monolayer-based electrode for voltammetric determination of copper. Electrochimica Acta,  49 (22-23 SPEC. ISS.): 3795-3800.