Malaysian Journal of Analytical Sciences Vol 19 No 2 (2015): 318 – 324

SURFACE SEDIMENT ANALYSIS ON PETROLEUM HYDROCARBON AND TOTAL ORGANIC CARBON FROM COASTAL AREA OF PAPAR TO TUARAN, SABAH

(Analisis Sedimen Permukaan Terhadap Petroleum Hidrokarbon dan Jumlah Karbon Organik daripada Pesisiran Pantai Papar ke Tuaran, Sabah)

Siti Aishah Mohd Ali1*, Carolyn Payus1, Masni Mohd Ali2

1Water Research Unit, Faculty of Science and Natural Resources,

Universiti Malaysia Sabah, Jalan UMS, 88400 Kota Kinabalu, Sabah, Malaysia

2School of Environmental and Natural Resource Sciences, Faculty of Science and Technology,

Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

*Corresponding author: ctaishah@ums.edu.my

Received: 12 September 2014; Accepted: 5 January 2015

Abstract

Total petroleum hydrocarbons (TPH) and total organic carbon (TOC) were investigated in surface sediments along coastal area of Papar to Tuaran, Sabah. Surface sediment samples were collected in 24 different stations in each area by using Ponar grab sampler. Samples were extracted for TPH using standard method sediment/sludge APHA 5520E and analyzed using UV/VIS spectrophotometer while for TOC method analysis using United Nations Environment Programme (UNEP)/MAP Athens (2006). The range of TPH concentrations in surface sediments were recorded at 0.24 – 20.65 mg/kg dw Miri crude oil equivalents, meanwhile the TOC percentage ranged from 0.03 – 4.02%. In the mean time, the statistical analysis by Pearson correlation showed a positive correlation with coefficient, r = 0.790 which showing the TPH concentrations significantly have influence on the TOC accumulations in the surface sediment.

Keywords: total petroleum hydrocarbon, surface sediment, total organic carbon

Abstrak

Jumlah petroleum hidrokarbon (TPH) dan jumlah karbon organik (TOC) telah dikaji dalam sedimen permukaan di sepanjang kawasan persisiran pantai Papar ke Tuaran, Sabah. Sampel sedimen permukaan dikumpul di 24 stesen yang berbeza di setiap kawasan dengan menggunakan Ponar grab sampler. Sampel telah diekstrak bagi TPH menggunakan kaedah piawai sedimen /enapcemar APHA 5520E dan dianalisis menggunakan UV/VIS spektrofotometer manakala bagi kaedah analisis TOC pula menggunakan United Nations Environment Programme (UNEP)/MAP Athens (2006). Julat kepekatan TPH dalam sedimen permukaan direkodkan pada 0.24 – 20.65 mg / kg bk setara dengan minyak mentah Miri, sementara peratusan TOC adalah di antara 0.03 – 4.02%. Sementara itu, analisis statistik dengan korelasi Pearson menunjukkan korelasi positif dengan pekali, r = 0.790 yang menunjukkan kepekatan TPH ketara mempunyai pengaruh ke atas pengumpulan TOC dalam sedimen permukaan.

Kata kunci: jumlah petroleum hidrokarbon, sedimen permukaan, jumlah karbon organik

References

- Diya’uddeen, B.H., Ashri Wan Daud, W.M. & Abdul Aziz, A.R. (2011). Treatment technologies for petroleum refinery effluents: A review. Process Safety and Environmental Protection. 89: 95-105.

- Al Darwish, H.A., Abd El-Gawad, E.A., Mohammed, F.H. & Lotfy, M.M. (2005). Assessment of organic pollutants in the offshore sediments of Dubai, United Arab Emirates. Environmental Geology. 48: 531–542.

- Venkatachalapathy, R., Veerasingam, S., Basavaiah, N. & Ramkumar, T. (2010). Comparison between petroleum hydrocarbon concentrations and magnetic properties in Chennai coastal sediments, Bay of Bengal, India. Marine and Petroleum Geology, 27: 1927-1935.

- Zanardi, E., Bicego, M.C., Miranda, L.B. & Weber, R.R. (1999). Distribution and origin of hydrocarbon in water and sediment in Sao Sebastiao, Brazil. Marine Pollution Bulletin, 38: 261 – 267.

- GESAMP. (1993). Impact of oil and related chemicals and wastes on the marine environment. In: Report and Studies, GESAMP. 50: 180.

- Laws, E.A. (2000). Aquatic pollution: An introductory text. John Wiley and Sons, Inc., New York, pp. 432-479.

- Wang, Z. & Fingas, M.F. (2003). Development of oil hydrocarbon fingerprinting and identification techniques. Marine Pollution Bulletin, 47: 423-452.

- Choi, H.G., Moon, H.B., Choi, M. & Yu, J. (2011). Monitoring of organic contaminants in sediments from the Korean Coast: Spatial distribution and temporal trends (2001-2007). Marine Pollution Bulletin, 62(6): 1352-1361.

- Millie, G., Asia, L., Guiliano, M., Malleret, L. & Dumenq, P. (2007). Hydrocarbon in coastal sediments from the Mediterranean Sea (Gulf of Fos area, France). Marine Pollution Bulletin, 54: 566-575.

- Maioli, O.L.G., Rodrigues, K.C., Knoppers, B.A. & Azevedo, D.A. (2010). Pollution source evaluation using petroleum and aliphatic hydrocarbons in surface sediments from two Brazilian estuarine systems. Organic Geochemistry, 41: 966-970.

- Baudouin, C., Charveron, M., Tarroux, R. & Gall, Y. (2007). Environmental pollutants and skin cancer. Cell Biological Toxicology, 18: 341-348.

- Kucuksezgin, F., Kontas, A., Altay, O., Uluturhan, E. & Darilmaz, E. (2006). Assessment of marine pollution in Izmir Bay: Nutrient, heavy metal and total hydrocarbon concentrations. Environment International, 32: 41-51.

- Medeiros, P. M., Bcego, M. C., Castelao, R. M., Rosso, C. D., Fillmann, G. & Zamboni, A. J. (2005). Natural and anthropogenic hydrocarbon inputs to sediments of Patos Lagoon Estuary, Brazil. Environment International, 31: 77–87.

- Lick, W. (2009). Sediment and contaminant transport in surface water. CRC Press. America.

- Hostettler, F. D., Pereira, W. E., Kvenvolden, K. A., Green, V. A., Luoma, S. N., Fuller, C. C. & Anima, R. (1999). A record of hydrocarbon input to San Francisco Bay as traced by biomarker profiles in surface sediment and sediment core. Marine Chemistry, 64: 115–127.

- Lyla, S, Manokaran, S. & Khan, A. (2012). Petroleum hydrocarbon distribution in continental shelf region of southeast coast of India. International Journal of Sediment Research, 27: 73-83.

- Niemirycz, E., Gozdek J. & Koszka-Maroń, D. (2006). Variability of organic carbon in water and sediments of the Odra River and its tributaries. Polish Journal of Environmental Studies, 15. (4): 557-563.

- Zerbe, J. (1993). Hydrocarbons and its derivatives in water. Environmental and Natural Resources Protection.

- Al-Abdali, F., Massoud, M.S. & A1-Ghadban, A.N. (1996). Bottom sediments of the Arabian Gulf; IIL Trace metal contents as indicators of pollution and implications for the effect and fate of Kuwait oil slick. Environmental Pollution, 93(3): 285-301.

- Massoud, M.S., AI-Abdali, F. & AI-Ghadban, A.N. (1996). Bottom sediments of the Arabian Gulf; II. TPH and TOC contents as indicators of oil pollution and implications for the effect and fate of Kuwait oil slick. Environmental Pollution, 93(3): 271-284.

- American Public Health Association (APHA). (2005). Standard Methods for the Examination of Water and Wastewater. 21st Edition. American Public Health Association.

- United Nations Environment Programme (UNEP). (2006). Methods for sediment sampling and analysis. United Nations Environment Program.

- Marchand, M., Bodennec, G., Caprais,  J.C. & Pignet, P. 1982. The AMOCO CADIZ oil spill, distribution and evolution of oil pollution in marine sediments. In: Ecological Study of the AMOCO CADIZ Oil Spill. Report of the NOAA-CNEXO Joint Scientific Commission. pp. 143-157.

- Suratman, S., Mohd Tahir. N. & Latif, M.T. (2012). A preliminary study of Total Petrogenic Hydrocarbon distribution in Setiu Wetland, Southern South China Sea (Malaysia). Bulletin of Environmental Contamination Toxicolology, 88:755–758.

- Sutarji Kasmin. (2010). Enforcing ship-based marine pollution for cleaner sea in the Strait of Malacca. Environment Asia, 3: 61-65.

- Hafidz B. Yusoff, Zaini B. Assim & Samsur B. Mohamad. (2012). Aliphatic hydrocarbons in surface sediments from South China Sea off Kuching Division, Sarawak. The Malaysian Journal of Analytical Sciences, 16(1): 1 - 11.

- Law, A.T., Ravinthar, V. & Yeong, C.H. (1990). Oil pollution in the coastal waters off Port Dickson, Straits of Malacca. Pertanika, 13: 381-387.

- Abdullah A.R. & Samah, A.A. (1996). Oil and grease in coastal waters. In: Oil and grease pollution in the Malaysian marine environment. A.R. Abdullah and C.W. Wang (ed.), Department of Environment, Malaysia.

- Tahir, N.M., Abdullah, A.R. & Shanmugam, S. (1997). Determination of total hydrocarbon concentration in coastal waters and sediments off the east coast of Peninsular Malaysia. Environmental Geochemistry and Health, 19: 67-71.

- Abdullah, M. P. (1997). Hydrocarbon pollution in the sediment of some Malaysian coastal areas. Environmental Monitoring and Assessment, 44: 443-454.

- Siti Aishah Mohd Ali, Rohana Tair, Soon Zhi Yang & Masni Mohd Ali. (2013). Petroleum hydrocarbon in surface sediment from coastal area of Putatan and Papar, Sabah. The Malaysian Journal of Analytical Sciences, 17(2): 286-290.

- Rudolf, J., Ahmed, I.R., Medeiros, P.M. & Bernd, R.T.S. (2006). Natural product biomarkers as indicators of sources and transport of sedimentary organic matter in a subtropical river. Chemosphere, 64: 1870–1884.