# Malaysian Journal of Analytical Sciences Vol 19 No 4 (2015): 766 - 774

# NATURAL RADIONUCLIDES AND HEAVY METALS POLLUTION IN SEAWATER AT KUALA LANGAT COASTAL AREA

# (Pencemaran Radionuklid Semulajadi dan Logam Berat di dalam Air Laut di Kawasan Persisiran Pantai Kuala Langat)

# Sabarina Md Yunus* Zaini Hamzah, Ab Khalik Hj Wood, Ahmad Saat

# Faculty of Applied Sciences,

# Universiti Teknologi MARA, 40450 Shah Alam, Selangor, Malaysia

# *Corresponding author: sabarina2020@salam.uitm.edu.my

Received: 23 November 2014; Accepted: 27 June 2015

Abstract

# Rapid industrial developments along the Langat river basin play an important role in contributing to the increases of pollution loading at Kuala Langat coastal area. The major pollutant sources in this area may be originating from human activities such as industrial discharge, domestic sewage, construction, agriculture and pig farming near the tributaries that affects the water quality. In addition, Langat and Semenyih rivers flow through the mining and ex-mining area, which is related to the source of natural radionuclides contamination.  Heavy metals in the aquatic environment and more likely to enter the food chain. This study is focusing to the levels of radionuclides and heavy metals in seawater. The samples were collected using appropriate water sampler, which is then acidified until pH 2 and filtered using cellulose acetate 0.45 μm.  The concentration of these radionuclides and heavy metals were determined using Inductive Coupled Plasma Mass Spectrometer (ICPMS) after dilution. In general, the radionuclides and heavy metals concentrations are lower than Malaysia Marine Water Quality Standard (MMWQS) except for few locations where the concentration levels above the standards. The higher concentration of pollutant in the seawater may have a toxic effect on sensitive plants and living organisms. The results of pollution levels of these radionuclides and heavy metals were also compared with other studies.

# Keywords: heavy metals, pollution, radionuclides, uranium, thorium

Abstrak

# Perkembangan perindustrian yang pesat di sepanjang lembangan Sungai Langat memainkan peranan yang penting dalam menyumbang kepada peningkatan beban pencemaran di kawasan pantai Kuala Langat. Sumber pencemar utama di kawasan ini terdiri daripada aktiviti manusia seperti pelepasan industri, kumbahan domestik, pembinaan, pertanian dan penternakan babi berhampiran anak sungai yang menjejaskan kualiti air. Di samping itu, Sungai Langat dan Semenyih mengalir melalui kawasan perlombongan dan bekas lombong yang berkaitan dengan punca pencemaran radionuklid semula jadi. Logam berat dalam persekitaran akuatik lebih cenderung untuk memasuki rantai makanan. Kajian ini memberi tumpuan kepada tahap radionuklid dan logam berat dalam air laut. Sampel telah dikumpulkan menggunakan sampler air yang sesuai, yang kemudiannya diasidkan sehingga pH 2 dan ditapis menggunakan selulosa asetat 0.45 μm. Kepekatan radionuklid dan logam berat telah ditentukan menggunakan plasma induktif spektrometer jisim (ICPMS) selepas pencairan. Secara umum, radionuklid dan kepekatan logam berat adalah lebih rendah daripada Malaysia Standard Kualiti Air Marin (MMWQS) kecuali di beberapa lokasi di mana tahap kepekatan di atas piawaian. Kepekatan yang tinggi bahan pencemar dalam air laut yang mungkin mempunyai kesan toksik pada tanaman yang sensitif dan organisma hidup. Keputusan tahap pencemaran ini radionuklid dan logam berat juga dibandingkan dengan kajian-kajian lain.

# Kata kunci: logam berat, pencemaran, radionuklid, uranium, thorium

References

- Adnan, N. H., Zakaria, M. P., Juahir, H., and Ali, M. M. (2012). Faecal sterols as sewage markers in the Langat River, Malaysia: Integration of biomarker and multivariate statistical approaches. Journal of Environmental Sciences 24(9), 1600–1608.

- Sany, S. B. T., Salleh, A., Sulaiman, A. H., Sasekumar, A., Rezayi, M., and Tehrani, G. M. (2012). Heavy metal contamination in water and sediment of the Port Klang coastal area, Selangor, Malaysia. Environmental Earth Sciences 69(6), 2013–2025.

- Othman, M. S., and Gasim, M. B. (2005). Kepekatan Logam Berat dalam Air di Lembangan Sungai Semenyih , Selangor, Sains Malaysiana 34(2), 49–54.

- Mohamed, C.A.R, Theng, T. L. and Ahmad, Z. (2006). Po and 210 Pb in Cockle Tissues in West Coast of Peninsula Malaysia. Journal of Nuclear and Related Technologies 3(1), 69–75

- Hamzah, Z., Rosli, T. N. T. M., Saat, A., and Wood, A. K. (2014). An assessment of natural radionuclides in water of Langat River estuary, Selangor. AIP Conference Proceedings 228, 228–234.

- Lim, W. Y., Aris, A. Z., and Zakaria, M. P. (2012). Spatial variability of metals in surface water and sediment in the langat river and geochemical factors that influence their water-sediment interactions. The Scientific World Journal 2012: 1-14.

- APHA. (2005). Standard Methods for the Examination of Water and Wastewater (21st ed.). American Water Works Association, Water Environment Federation, Washington, DC, USA.

- Shrivastava, A. and Gupta, V. (2011). Methods for the determination of limit of detection and limit of quantitation of the analytical methods. Chronicles of Young Scientists, 2(1), 21.

- Looi, L. J., Aris, A. Z., Wan Johari, W. L., Md Yusoff, F., and Hashim, Z. (2013). Baseline metals pollution profile of tropical estuaries and coastal waters of the Straits of Malacca. Marine Pollution Bulletin, 74(1), 471–6.

- Idriss, A. (2012). Concentration of selected heavy metals in water of the Juru River, Penang, Malaysia. African Journal of Biotechnology 11(33), 8234–8240.

- Wang, J., Liu, R. H., Yu, P., Tang, a. K., Xu, L. Q., & Wang, J. Y. (2012a). Study on the Pollution Characteristics of Heavy Metals in Seawater of Jinzhou Bay. Procedia Environmental Sciences 13, 1507–1516.

- Yılmaz, S. and Sadikoglu, M. (2011). Study of heavy metal pollution in seawater of Kepez harbor of Canakkale (Turkey). Environmental Monitoring and Assessment 173(1-4), 899–904.

- Wang, R. and You, C. (2013). Uranium and strontium isotopic evidence for strong submarine groundwater discharge in an estuary of a mountainous island : A case study in the Gaoping River Estuary, Southwestern Taiwan. Marine Chemistry 157, 106–116.

- Swarzenski, P. W. and Baskaran, M. (2007). Uranium distribution in the coastal waters and pore waters of Tampa Bay, Florida, 104, 43–57.

- Cukrov, N., Barisˇ, D. and Mlakar, M. (2006). Uranium in sediments, mussels (Mytilus sp) and seawater of the Krka river estuary. Journal of Environmental Radioactivity 85, 59–70.