Malaysian Journal of Analytical Sciences Vol 19 No 4 (2015): 707 - 714

EVALUATION OF HEAVY METAL CONTAMINATION LEVELS OF BALOK RIVER SEDIMENTS IN PAHANG, MALAYSIA BASED

ON GEOACCUMULATION INDEX AND SUPPORTED

WITH ENRICHMENT FACTOR

(Penilaian Paras Pencemaran Logam Berat Terhadap Sedimen Sungai Balok di Pahang, Malaysia Berdasarkan Kepada Indeks Geo-akumulasi dan Di Sokong oleh Faktor Pengkayaan)

Mohd Zahari Abdullah1*, Nur Rashidah Abd Manap1, Ahmad Saat2, Zaini Hamzah2, Mohd Tahir Abas1

1Faculty of Applied Sciences,

Universiti Teknologi MARA Pahang,26400 Bandar Tun Razak Jengka, Pahang, Malaysia

2Faculty of Applied Sciences,

Universiti Teknologi MARA, 40450 Shah Alam, Selangor, Malaysia

*Corresponding author: umizah@pahang.uitm.edu.my

Received: 23 November 2014; Accepted: 27 June 2015

Abstract

This study was conducted in order to assess the level of metal pollution in river sediment samples, which indirectly representing the general quality of the Balok River. The samples of river sediment have been collected at nine sampling stations from the particular river. The total content of Cd, Co, Cr, Cu, Fe, Mn, Ni, Pb, V and Zn were measured using inductively coupled plasma optical emission spectrometer (ICP-OES) after an acid extraction. The mean concentrations (mg/kg) were found at 0.49, 16.16, 42.93, 24.31, 2734.69, 155.61, 16.04, 29.31, 50.71 and 159.40 for the respective metals. The assessment of the river quality was done based on the information showed by the enrichment factor (EF) and geoaccumulation index (Igeo). Analysis of the Igeo indexes clearly indicate that most of the sediment samples analyzed in this study contained the selected heavy metals in the levels of unpolluted to moderately pollute. Based on the low EF values recorded in this study, it can be concluded that the Balok River sediments is not significantly contaminated with the studied heavy metals.

Keywords: sediment, acid extraction, enrichment factor (EF), Geo-Accumulation Index (Igeo), heavy metals

Abstrak

Kajian ini dijalankan untuk menilai tahap pencemaran logam di dalam sampel sedimen sungai, yang secara tak langsung secara umum akan menggambarkan kualiti Sungai Balok. Sampel-sampel sedimen sungai telah diambil di sembilan tapak persampelan daripada sungai tersebut.   Jumlah kandungan Cd, Co, Cr, Cu, Fe, Mn, Ni, Pb, V dan Zn telah diukur menggunakan plasma gandingan teraruh – spektrometer emisi optik (ICP-OES). Nilai kepekatan purata (mg/kg) bagi setiap logam yang dikaji adalah 0.49, 16.16, 42.93, 24.31, 2734.69, 155.61, 16.04, 29.31, 50.71 dan 159.40 bagi masing-masing logam. Penilaian kualiti sungai telah dilakukan berdasarkan maklumat yang diberikan oleh faktor pengkayaan (EF) dan indeks geo-pengumpulan (Igeo).  Analisis terhadap indeks Igeo jelas menunjukkan bahawa kebanyakan sampel sedimen yang dianalisis mengandungi kandungan logam berat pada paras tidak tercemar sehingga sederhana tercemar. Berdasarkan kepada nilai-nilai EF yang rendah direkodkan dalam kajian ini, maka boleh disimpulkan bahawa sedimen Sungai Balok tidak dicemari secara ketara dengan logam dikaji.

Kata kunci : sedimen, pencernaan berasid, faktor pengkayaan (EF), Indeks Geo-akumulasi (Igeo), logam berat

References

- Abbas, F., Norli, I.A., Aness, A. and Azharmat, E. (2009). Analysis of heavy metal concentrations in sediments of selected Estuaries of Malaysia – a statistical assessment. Environmental Monitoring and Assessment 153 (1-4): 179-185.

- Raju, K. V., Somashekar, R., and Prakash, K. (2012). Heavy metal status of sediment in River Cauvery, Karnataka. Environmental Monitoring and Assessment 184(1): 361-373.

- Chakravarty, M. and Patgiri, A. (2009). Metal pollution assessment in sediments of the Dikrong River, N. E. India. Journal of Human Ecology 27 (1): 63-67.

- Brunner, I., Luster, J., Gunthardt-Goerg, M. and Frey, B. (2008). Heavy metal accumulation and phytostabilisation potential of tree fine roots in a contamination soil. Environmental Pollution 152 (3): 559-568.

- Abdullah, M. Z., Saat, A. and Hamzah, Z. (2012). Assessment of the impact of petroleum and petrochemical industries to the surrounding areas in Malaysia using mosses as bioindicator supported by multivariate analysis. Environmental Monitoring and Assessment184: 3959-3969.

- Chen, C.Y., Stemberger, R.S. and Klaue, B. (2000). Accumulation of heavy metals in food web components across a gradient of lakes. Limnology and Oceanography 45(7): 1525-1536.

- Lim, W. Y., Zaharin, A. and Zakaria, M. P. (2012). Saptial variability of metals in surface water and sediment in the Langat River and geochemical factors that influence their water sediment interactions. The Scientific World Journal 12: 412-426.

- Zhong, A. P., Guo, S.H., Li, F.M. and Jiang, K.M. (2006). Impact of anions on the heavy metals release from marine sediments. Journal of Environmental Sciences 18 (6):1216-1220.

- Barakat, A., El Baghdadi, M., Rais, J. and Nadem, S. (2012). Assessment of Heavy Metal in Surface Sediments of Day River at Beni-Mellal Region, Morocco. Research Journal of Environmental and Earth Sciences 4(8): 797-806.

- Zhou, J., Ma, D., Pan, J., Nie, W., and Wu, K. (2008). Application of multivariate statistical approach to identify heavy metal sources in sediment and waters: a case study in Yang zhong, China. Environment Geology 54:373-380.

- Baldantoni D, Maisto, G., Bartoli, G. and Alfani, A. (2005). Analyses of three native aquatic plant species to assess spatial gradients of lake trace element contamination. Aquatic Botany. 83(1): 48-60.

- Lu, X.W., Wang, L.J., Li, L.Y., Lei, K., Huang, L. and Kang, D. (2010). Multivariate statistical analysis of heavy metals in street dust of Baoji, NW China. Journal Hazardous Materials 173:744-749

- Plumlee, K. H. R. (2002). Toxicosis from agricultural chemicals. Clinical Techniques in Equine Practice 1: 94-97.

- Shomar, B. H (2006). Trace elements in major solid-pesticides used in the Gaza strip. Chemosphere, 65: 215-221.

- Gu, Y.G., Wang, Z.H., Lu, S.H., Jiang, S.J., Mu, D.H. and Shu, Y.H. (2012). Multivariate statistical and GIS-based approach to identify source of anthropogenic impacts on metallic elements in sediments from the mid Guangdong coasts, China. Environmental Pollution 163, 248-255

- El Nemr, A., Khaled, A. and El Sikaily, A. (2006) Distribution and statistical analysis of leachable and total heavy metals in the sediments of the Suez Gulf. Environmental Monitoring and Assessment 118: 89-112.

- Abrahim, G. M. S. and Parker, R. J. (2008). Assessment of heavy metal enrichment factors and the degree of contamination in marine sediments from Tamaki Estuary, Aucland, New Zealand. Environemtal Monitoring and Assessment 136:227-238.

- Özkan, E. Y. (2012). A New Assessment of Heavy Metal Contaminations in an Eutrophicated Bay (Inner Izmir Bay, Turkey). Turkish Journal of Fisheries and Aquatic Sciences 12: 135-147.

- Loska, K., Cebula, J., Pelczar, J., Wiechula, D. and Kwapulinski, J. (1995). Use of Enrichment and Contamination Factors together with Geoaccumulation Indexes to evaluate the content of Cd, Cu and Ni in the Rybnik Water Reservoir in Poland. Water, Air and Soil Pollution 93: 347-365.