Malaysian Journal of Analytical Sciences Vol 19 no 5 (2015): 951 - 959

KAJIAN PENEROKAAN TANAH DAN PERUBAHAN KUALITI AIR DI TANAH TINGGI LOJING, KELANTAN, MALAYSIA

# (Land Exploration Study and Water Quality Changes in Tanah Tinggi Lojing,

# Kelantan, Malaysia)

# Wan Adi Yusoff1*, Mokhtar Jaafar1, Mohd Khairul Amri Kamarudin2,3, Mohd Ekhwan Toriman1,2

# 1School of Social, Development and Environmental Studies, Faculty of Social Sciences and Humanities,

# Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

2East Coast Environmental Research Institute (ESERI)

# 3Faculty of Design Arts and Engineering Technology

# Universiti Sultan Zainal Abidin, Gong Badak Campus, 21300 Kuala Terengganu, Terengganu, Malaysia

*Corresponding author: wanadi.ukm@gmail.com

Received: 14 April 2015; Accepted: 19 July 2015

Abstrak

# Aktiviti penerokaan tanah di kawasan Tanah Tinggi tanpa mitigasi dapat memberi impak besar terhadap kesihatan persekitaran di dalam lembangan tersebut. Tanah Tinggi Lojing Gua Musang merupakan antara kawasan yang dibangunkan bagi aktiviti pertanian komersil. Sungai Belatop merupakan antara sungai yang terjejas dan memberi kesan terhadap bekalan sumber air penduduk setempat. Kajian ini dijalankan bagi mengetahui perkaitan antara pembangunan tanah dengan perubahan kualiti air sungai di Lembangan Sungai Belatop. Kajian dijalankan berdasarkan analisis data daripada Jabatan Pengairan dan Saliran bermula dari Februari hingga Oktober 2012 secara berkala. Dapatan menunjukkan, pembangunan tanah di kawasan Tanah Tinggi Lojing telah memberi kesan nagetif kepada kualiti air di kawasan kajian. Dimana, nilai sedimen terampai (SS) adalah tinggi dengan nilai 1161 mg/L, nilai Turbidity mencapai 991(NTU) dan terus meningkat pada musim hujan kesan aliran permukaan yang menghakis permukaan tanah tidak berkenopi. Kandungan Magnisium juga telah meningkat iaitu 0.66 mg/L kepada 1.38 mg/L. Kandungan magnisium yang tinggi terhasil daripada penggunaan baja dalam aktiviti pertanian di kawasan kajian. Kandungan baja kimia yang berlebihan menyebabkan kandungan nilai kalsium (Ca) di lapangan bernilai 3.18 mg/L. Kandungan ferum dalam air juga telah melepasi 0.3 mg/L iaitu Kelas IIA/B dan Kelas III berdasarkan IKA. Justeru itu, beberapa cadangan dikemukan bagi mengurangkan masalah.

# Kata kunci: perubahan gunatanah, muatan sedimen, pengurusan sumber air, Tanah Tinggi Lojing

Abstract

Land exploration activities in the highlands areas without mitigation can give more impact on the environment health in the river basin. Tanah Tinggi Lojing Gua Musang is one of the commercial areas developed for agricultural activities. Sungai Belatop is one of the affected river at Tanah Tinggi Lojing. This article was conducted to investigate the relationship of land development and water quality changes in the Belatop river basin.The study was conducted by analyzing data from the Department of Irrigation and Drainage starts from February to October 2012. The results indicated the development of land exploration activities in the Tanah Tinggi Lojing area has affected on water quality parameters in the area. Where, the suspended sediment (SS) is high with 1161 mg/L, turbidity value is 991 (NTU) and this parameters is increasing on the rainy season impacted from surface erosion and surface runoff at land is not covered. Magnisium content also has increased from 0.66 mg/L to 1.38 mg/L. The high content magnisium result is from fertilizers used in agricultural activities in the study area. Chemical fertilizers content excessive causes the contents of calcium (Ca) high (3.18 mg/L). Ferum content in the water has also exceeded (0.3 mg/L) ware on Class IIA/B and Class III by WQI. Therefore, some recommendations is proposed to reduce and solving this problem.

# Keywords: land use changes, sediment load, water resource management, Tanah Tinggi Lojing

References

- Ghani, A.A.B.,  Chang, C.K.,  Leow, C.S.  and  Zakaria N.A. (2012).  Sungai  Pahang  digital  flood  mapping: (2007) flood. International Journal River Basin Managemnent 10 (2): 139-148.

- Abdullah, N. M., Toriman, M. E., Md Din, H., Abd Aziz, N. A., Kamarudin, M. K. A., Abdul Rani, N. S., Ata, F. M., Saad, M. H., Abdullah, N. W., Idris, M., Jamil, N. R. (2013).  Influence of Spatial and Temporal Factors in Determining Rainfall Interception at Dipterocarp Forest Canopy, Lake Chini, Pahang. Malaysian Journal of Analytical Sciences 17 (1): 11–23.

- Armas, I., Gogoaşe Nistoran D. E., Osaci-Costache, G. and Braşoveanu, L. (2013). Morpho-dynamic evolution patterns of Subcarpathian Prahova River (Romania). CATENA 100: 83-99.

- Ashmore, P. E. and Rennie, C. D. (2013). Gravel-bed rivers: from particles to patterns. Earth Surface Processes and Landforms 38(2): 217-220.

- Kamarudin, M. K. A.,  Toriman, M. E.,  Lun, P. I.,  Abdul Aziz, N. A. and  Gasim, M. B.  (2013).   The  hydro-dynamic  of  Dong River, Hutan Lipur Lata Jarum,  Pahang,  Malaysia.  Prudence  Journal  of  Environmental Science Research 1(2): 5-11. 148.

- Toriman, M.E,  Gasim, M. B.,  Yusop, Z.,  Shahid, I., Mastura, S. A. S., Abdullah, P., Jaafar, M., Andul Aziz, N.A., Kamarudin, M. K. A., Jaafar, O., Karim, O., Juahir, H. and Jamil, N. R. (2012). Use of  137Cs activity to investigate sediment  movement  and  transport  modeling in river coastal  environment.  American  Journal of Environment Sciences 8: 417-423.

- Syed Omar, S. A., Eh Rak, A., Ahmad Sanusi, A. F., and  Md Yusoff, A. (2014).  Benthic Macroinvertebrates Composition  and  Distribution  at  Sungai  Dawai  and  Sungai  Dekong   in  Lojing  Highland,  Gua Musang, Kelantan. Jurnal Teknologi 68 (3): 125-13.

- Kamarudin, M.K.A., Idris, M. and Toriman, M.E. (2013). Analysis of Leptobarbus hoevenii in control environ-ment at natural lakes. American Journal of Agricultural and Biological Sciences 8: 142-148.

- Md Din, H., Toriman, M. E., Mokhtar, M., Elfithri, R., Abd Aziz, N. A., Abdullah, N. M., Kamarudin, M. K. A., (2012). Loading Concentrations of Pollutant in Alur Ilmu at UKM Bangi Campus: Event Mean Concentration (EMC) Approach. Malaysian Journal of Analytical Sciences, 16 (3): 353–365.

- Kamarudin, M. K. A., Toriman, M. E., Rosli, M. H.,  Juahir, H., Azid, A.,  Mohamed Zainuddin, S. F.,  Abdul Aziz, N. A. and Sulaiman, W. N. A. (2014).  Analysis  of Meander Evolution Studies on Effect from Land Use and Climate Change at Upstream Reach of Pahang River, Malaysia.  Mitigation and Adaptation Strategies for Global Change: 1-16.

- Lun, P. I., Gasim, M. B., Toriman, M. E., Rahim S. A. and Kamarudin, M. K. A. (2011). Hydrological Pattern of Pahang River Basin and Their Relation to Flood Historical Event. Jurnal e-Bangi 6(1): 29-37.

- Azid, A., Che Hasnam, C. N., Juahir, H., Amran, M. A., Toriman, M. E., Kamarudin, M. K. A., Mohd Saudia, M. S.,  Gasim, M. B. and  Mustafa, A. D. (2015).  Coastal  Erosion  Measurement  along  Tanjung  Lumpur to Cherok Paloh, Pahang during the Northeast Monsoon Season. Jurnal Teknologi 74 (1): 27–34.

- Kamarudin, M. K. A.,  Toriman, M. E.,  Sarifah A., S. M,  Idris, M.,  Jamil, N. R.  and  Gasim,  M. B.  (2009). Temporal Variability on  Lowland  River  Sediment Properties and Yield. American Journal of Environmental Sciences 5(5): 657-663.

- Heng, G. S. and Hing, T. T. (2000). Classifying Water Quality Along Sungai Pahang and its Tributaries. Sains Malaysiana 29: 257-272 1.

- Chan, N.W. (2012).  Managing  urban  rivers and  water  quality in  Malaysia for  sustainable  water resources. International Journal of Water Resources Development 28(2): 343-354.

- Jabatan Perancangan Bandar dan Desa, JPBD. (2012).  Laporan  Penemuan-Kajian  Pelan  Pengurusan Khusus (Pemuliharaan) Tanah Tinggi Kinta-Cameron Highland- Lojing. Jabatan Perancangan Bandar dan Desa Negeri Kelantan.

- DID. (2000).  Urban  Stormwater Management Manual for Malaysia. Kuala Lumpur: Department of Irrigation and Drainage. Malaysia.

- Mohd Saudia, M. S., Azid, A., Juahir, H., Ezani, E., Toriman, M.E., Amran, M. A., Mustafaa, A. D., Azamana,  F.,  Kamarudin, M.K.A.  and  Mohd  Saudib, M.  (2015).  Flood  Risk  Pattern  Recognition  Using  Integrated Chemometric Method and Artificial Neural Network: A Case Study in the Johor River Basin. Jurnal Teknologi 74 (1): 159–164.

- Sarbu C.,  Pop H.F.,  (2005).   Principal  component analysis versus fuzzy principal component analysis A case study: the quality of Danube water (1985-1996).  Talanta 65(5): 1215-1220.

- Chabukdhara M. and Nema A. (2012). Assessment of heavy metal contamination in Hindon River sediments: a chemometric and geochemical approach. Chemosphere 87:945–953.

- Mohd Saudia, M. S., Juahir, H., Azid, A., Kamarudin, M.K.A., Kasim, M. F.,Toriman, M.E., Abdul Aziz, N. A. Che Hasnam, C. N. and  Samsudin, M. S. (2015).  Flood  Risk  Pattern  Recognition  Using Chemometric Tech-nique: A Case Study In Kuantan River Basin, Jurnal Teknologi 72 (1): 137-141.

- Zhou F., Liu Y. and  Guo H., (2007). Application of Multivariate Statistical Methods to Water Quality Assess-ment of  the  Watercourses  in  Northwestern  New Territories,  Hong  Kong.   Environmental Monitoring and Assessment 132: 1-13.