
THE EFFECT OF SOLAR RADIATION EFFECT ON RADIO SIGNAL FOR RADIO ASTRONOMY PURPOSES
(Kesan Radiasi Solar Terhadap Isyarat Radio untuk Kajian Astronomi Radio) 

Nor Hazmin Sabri1*, Atiq Wahidah Azlan2, Roslan Umar3,4, Shahirah Syafa Sulan2, Zainol Abidin Ibrahim5, Wan Zul  Adli Wan Mokhtar6 

1School of Fundamental Science, Universiti Malaysia Terengganu, 
21030 Kuala Terengganu, Terengganu, Malaysia 
2Physic, Electronic and Instrumentation Programme, School of Ocean Engineering, 
Universiti Malaysia Terengganu, 21030 Kuala Terengganu, Terengganu, Malaysia
 3East Coast Environmental Research Institute (ESERI), Universiti Sultan Zainal Abidin, 
Gong Badak Campus, 21300 Gong Badak, Kuala Terengganu, Terengganu, Malaysia

4Faculty of Contemporary Islamic Studies, Universiti Sultan Zainal Abidin, Gong Badak Campus, 
21300 Gong Badak, Kuala Terengganu, Terengganu, Malaysia
5Physics Department, Faculty of Science, Universiti of Malaya, 
50603 Kuala Lumpur, Malaysia
6Department of Physics, Faculty of Science and Mathematics, Universiti Pendidikan Sultan Idris, 
35900 Tanjung Malim, Perak, Malaysia

*Corresponding author: norhazmin@umt.edu.my
Abstract

Radio astronomy is a subfield of astronomy which is discovers the celestial objects at radio frequencies. Observation in radio astronomy is conducted using single antenna or array of antennas, known as radio telescope. Other than that, radio astronomy also holds an advantage over other alternatives to optical astronomy due to its capability of observing from the ground level. In this study, the effect of solar radiation that contributes the Radio Frequency Interferences (RFI) is reviewed. The low RFI level is required to set up the radio telescope for radio astronomy observation. The effect of solar radiation on radio signal was investigated by determining the RFI pattern using spectrum analyzer. The solar radiation data was obtained from weather station located at KUSZA Observatory, East Coast Environmental Research Institute (ESERI), UniSZA. We can conclude that the solar radiation factor give the minimum significant effect to radio signal

Keywords: Solar radiation, radio signal, radio astronomy, radio frequency interference
Abstrak
Astronomi radio adalah salah satu cabang bidang astronomi yang mengkaji objek samawi dalam julat frekuensi radio. Cerapan astronomi radio dilakukan dengan menggunakan antenna tunggal atau jujukan antena yang dipanggil sebagai teleskop radio. Selain itu, astronomi radio mempunyai kelebihan berbanding alternatif lain bagi astronomi optic disebabkan kebolehannya mencerap dari paras tanah. Dalam kajian ini, kesan radiasi solar yang menyumbang kepada interferen frekuensi radio (RFI) dibincangkan.  Aras RFI yang rendah diperlukan untuk menempatkan teleskop radio untuk cerapan astronomi radio. Kesan radiasi solar keatas isyarat radio dikaji dengan menentukan pola RFI dengan menggunakan penganalisa spectrum. Data radiasi solar diperolehi dari stesen kajicuaca yang terletak di Balai Cerap KUSZA, Institut Penyelidikan Alam Sekitar Pantai Timur (ESERI), UniSZA. Konklusifnya, radiasi matahari memberi kesan minimum kepada isyarat radio. 

Kata kunci: radiasi solar, isyarat radio, astronomi radio, interferen frekuensi radio
Introduction

Radio wave propagation is influenced by the properties of the earth and the atmosphere. The curvature of the earth and the condition of the atmosphere can refract electromagnetic waves either up, away from, or down toward the earth’s surface [1]. In Malaysia, the sources of man-made RFI, such as telecommunication and navigation signals, are primarily listed by Malaysian Communications and Multimedia Commission (MCMC) [2]. The ionosphere reflects transmitted VLF radio waves back to Earth. The strength of these radio waves depends on how much or how little the ionosphere is ionized. Solar radiation contributes to ionization of the atmosphere, which can alter the strength of the transmitted VLF radio waves [3]. Radio frequency bands defined by International Telecommunications Union (ITU) together with their mode of propagation and primary uses are shown in Table 1. Solar radiations and other solar activities affect the total electron content of the ionosphere and also affect various radio frequencies used in telecommunications [4].

Table 1. Radio Frequencies Together with their Primary Propagation Characteristics and their Uses.

	S/N

	Name
	Frequency Range
	Primary Mode of Propagation
	Primary Uses

	1.
	Extremely Low Frequency (ELF)

	3 KHz
	Earth and the ionosphere. Wave guide penetrates sea water.
	Land to sub-marine communications.

	2.
	Very Low Frequency (VLF)

	3 – 30 KHz
	Between ground and lower ionosphere. It is a ground wave and uses wave guide also.
	Navigation, communication, standard frequency, and time

	3.
	Low Frequency (LF)

	30 – 300 KHz W
	Wave guide, ground wave.
	Maritime and broadcasting.

	4.
	Medium Frequency
	300 – 3000KHz
	E region of the ionosphere reflection (might). Ground wave.
	Maritime, aeronautical, international distress, and maritime/land mobile.


	5.
	High Frequency (HF)

	3 – 30 MHz
	Reflection from E and F region of the ionosphere.
	Maritime and aeronautical fixed services and broadcasting.

	6.
	Very High Frequency (VHF)

	30 – 300 MHz
	Line of sight, scatter from ionosphere. Reflection by active satellites
	Television, frequency modulated broadcasting , public safety, and aeronautical

	7.
	Ultra High Frequency (UHF)

	300 – 3000 MHz
	Line of sight (affected by ionosphere irregularities). Active satellites.
	Space communications, TV, radar, broadcasting, and navigation (fixed mobile)

	8.
	Super High Frequency (SHF)
	3000 – 30000 MHz
	Line of sight (troposphere affected by ionosphere irregularities)
	Space communication, TV, radar, broadcasting, and navigation (fixed mobile)


Source: MCMC manual of ‘Spectrum Plan [5]
Abidin et al. in 2012 have conducted an RFI investigation for setting up a VLBI station below 2.8 GHz in Malaysia [2]. The frequency window below 2.8 GHz which has been chosen are 322-328MHz, 608-614MHz, 1660-1660.5MHz and 1660.5-1668.4MHz. They found that two band below 2.8 GHz are permitted for the purpose of radio astronomy in Malaysia. They are 608-614MHz and 1660-1660.5MHz. The RFI level in these permissible bands at the best site which is Langkawi were also measured and concluded to be relatively low. They also reviewed several current VLBI observations in these two bands [6]. RFI factors are include location of radio and TV transmitters and electrics power generators. A study of the spectrum management of the country has also been made [7]. It is crucial to select areas with the least expected RFI to carry out radio astronomy observation.

Materials and Method
The main equipment involves in the data collection process (observation) is hand-held spectrum analyzer (SPECTRAN HF-6080 V4) to obtain the level of RFI. In this study, the frequencies and RFI trends of below 8GHz are investigated. Rest frequency of spectral lines below 8 GHz are listed in Table 2. This range covers at least 5 radio astronomical frequency window including the Hydrogen Line (1420MHz), Deuterium Line (328MHz), and Hydroxyl line (1660MHz). The observation was done for at least 1 hour in a row. The solar radiation data was obtained using weather station (Davis Vantage Pro2). The instrument setup for RFI observation is illustrated in Figure 1. The spectrum analyser was attached to the antenna and the computer would record and save the data collected. Concurrently, the weather station was started recording the solar radiation data.

Table 2. Rest frequency of spectral lines [8].

	Atom/ Molecule


	Line name
	Rest Frequency (GHz)

	DI
	Deuterium
	0.327384

	HI
	Neutral hydrogen
	1.420405752

	OH
	Hydroxyl radical
	1.6122310

	OH
	Hydroxyl radical
	1.6654018

	OH
	Hydroxyl radical
	1.6673590

	OH
	Hydroxyl radical
	1.7205300


Analysis will include producing RFI pattern. We also made use of the spectrogram technique in order to find consistent and non-consistent RFI. Then the relationship between RFI pattern and solar radiation pattern was determined.

[image: image1.png]Y

Antenna

!

Spectrum Analyzer

PZEESN

Computer




Figure 1. Instrument Setup for RFI observation
Result and Analysis

The results from the spectrum analyzer for frequencies up to 8GHz can be seen in Figure 2. The one-hour observation has been done 5 times at different date and time. From the figure below the radio signals detected at KUSZA Observatory were 100MHz, 200MHz, 300MHz, 500MHz, 1225MHz and 5825MHz.

[image: image2.png]—10Dec14

°898838388

(ugponorsomog




[image: image3.png]—17Dec14

°898888288

(ugponorsomog




[image: image4.png]5 Mar 2015

°8988883%8

(Waphiman sng




[image: image5.png]12 Mar 2015

SEIEIITEE

(wp)oaon somog




[image: image6.png]——o02-pr-15

“78885888

(wgphiaaer somod




          [image: image7.png]ZHIA 000°0008
2HA 000'008¢
2HA 000009
2HA 000'00p
2HA 000002
2HA 000000
2HA 0000089
2HA 0000099
2HA 000'00v9
2HA 0000023
2HA 0000003
2HA 0000085
2HA 0000035
2HA 000'00v5
2HA 0000025
2HA 0000005
2HA 000008
2HA 00000
2HA 000005
2HA 000002
2HA 0000001
2HA 000'008E.
2HA 000'009E.
2HA 000'00vE.
2HA 000'002E.
2HA 000'000€.
2HA 0000087
2HA 0000092
2HA 0000067
2HA 0000022
2HA 0000002
2HA 000008 T
2HA 0000091
2HA 00000V T
2HA 000002 T
2HA 000000 T
2H 000008
£HI 000009
ZHA 000007
ZH 000002
HIN000'0

Frequency (MHz)




Figure 2. Power Level against Frequency

The RFI pattern change compared to solar radiation for selected frequencies are shown in Figure 3 until Figure 8. The RFI level change over the variation of solar radiation was observed against time.

[image: image8.png]Solar, Power level vs Time

Solar
=—100MHz

1.5
13
11
0.9
0.7
0.5

299
S ® 3
~ © ©

(zw/m) uoneipey tejos

Q
B
n

900
850

Q9
S B
&~

0.3

=)
3
n

woiTet
Y0:ST:TT
T0:8T:TT
YOITT:ZT
TOH0:ZT
€0:L9°TT
TO:0S:TT
TOEVTT
70:9¢:TT
Y0:6T:TT
T0:TCTT
€0:STTT
€0:80:TT
00:TO:TT

| TO'PS:0T

Time (hhmm)





Figure 3. Solar and Power Level against Time at 100 MHz

[image: image9.png]Solar, Power level vs Time

Solar
=——200MHz

95

900

=)
3
®

@

W/,

700
600

cggggo
[=R=R=R=R]
nI O
uopeipey Jejos

woiTet
YOve:TT
T0:9T:TT
€0:80:CT
0:00:2T
€078 TT
€OVTT
70:9¢:TT
€0:8T:TT
00:0¢:TT
00:ZT:TT
TOHOTT
€0:95:0T
00:87:0T
0:01:0T

Time (hhmm)





Figure 4. Solar and Power Level against Time at 200MHz

[image: image10.png]Solar, Power level vs Time

Solar
———300MHz

3
2.5
1.5
1
0.5

2
0

oooogogago
S88838383
HROHLIFHN A

(zWw/M) uoneipey Jejos

900

woiTet
Y0:ST:TT
T0:8T:TT
YOITT:ZT
TOH0:ZT
€0:L9°TT
TO:0S:TT
TOEVTT
70:9¢:TT
Y0:6T:TT
T0:TCTT
€0:STTT
€0:80:TT
00:TO:TT
T0:15:0T

Time (hhmm)





Figure 5. Solar and Power Level against Time at 300MHz

[image: image11.png]Solar, Power level vs Time

Solar
=——500MHz

109
107
105
103
101
99
97
95

woiTet
YOve:TT
T0:9T:TT
€0:80:CT
0:00:2T
€078 TT
€OVTT
70:9¢:TT
€0:8T:TT
00:0¢:TT
00:ZT:TT
TOHOTT
€0:95:0T
00:87:0T
0:01:0T

cggggo
[=R=R=R=R]
nI O
uopeipey Jejos

900

=)
3
®

(cw/m]

700
600

Time (hhmm)





Figure 6. Solar and Power Level against Time at 500MHz

[image: image12.png]Solar, Power level vs Time

Solar
=—1225MHz

110
109
108
107
106
105
104
103
102
101
100

o999999
SS5383833838
KROBI MmN

(zw/m) uoneipey tejos

Qo

=)
3
&

woiTet
YOve:TT
T0:9T:TT
€0:80:CT
0:00:2T
€078 TT
€OVTT
70:9¢:TT
€0:8T:TT
00:0¢:TT
00:ZT:TT
TOHOTT
€0:95:0T
00:87:0T
0:01:0T

Time (hhmm)





Figure 7. Solar and Power Level against Time at 1225MHz

[image: image13.png]Solar, Power level vs Time

103

=)
3
&

102.5

=)
3

102

=)
3
HRO NI MmN

101.5

=)
3

Solar

101

=)
3

=)
3

=——5825MHz

100.5

=)
3
=

100

o

(zWw/M) uoneipey Jejos

woiTet
YOve:TT
T0:9T:TT
€0:80:CT
0:00:2T
€078 TT
€OVTT
70:9¢:TT
€0:8T:TT
00:0¢:TT
00:ZT:TT
TOHOTT
€0:95:0T
00:87:0T
0:01:0T

Time (hhmm)





Figure 8. Solar and Power Level against Time at 5825MHz

From Fig. 3 until 8, the effect of solar radiation has been determined. The RFI level seems not affected by the solar radiation. There are no obvious connection between solar radiation amount of energy received and the RFI pattern. This may be contributed by several factors; the observation time duration and the absence of Low Noise Amplifier (LNA). For better observation, the time duration for data collection should be conducted for 24hours to make sure the radiation from the sun along the day are taken into account. The LNA should be connected to the antenna to remove all of possible background noise for better reading. 

In research done by Ezekoye and Obodo (2007) about the effects of solar radiation on telecommunications, they investigate the effects of electron densities of the ionosphere and how they affect telecommunications [4]. They refer to the case study in year 2000, which was have maximum solar and was used to show the variations of electron density of ionosphere. In this research, they state that solar can make the telecommunication suffer of disturbances. For the conclusion, the telecommunication is adversely affected by solar activities. Electromagnetic storms from the sun may wipe out the telephone line, television signal and others. 

For future works, the threshold level for solar radiation effect on radio signal should be determined. This will contribute to the production of RFI map of Malaysia for solar radiation factor. Hence, the most suitable area with low interference could be determined for radio observation site. 

Conclusion

In conclusion, we can say that the solar radiation factor give the minimum significant effect to radio signal. This may be due the limited observation period of 1 hour and the absence of LNA. We suggest that the observation should run in 24 hours and the antenna should connect to the LNA for future observation.

Acknowledgements

This study is made possible by the usage of the grant RACE/F1/ST1/UNISZA/15. The authors gratefully acknowledge University Malaysia Terengganu, Universiti Sultan Zainal Abidin and Universiti Pendidikan Sultan Idris for the financial and experimental support of this work. Special thanks are also dedicated to other researchers Electromagnetic Research Group (EMRG) for their assistance in this work.

References

1. Adediji, A. T., Mandeep, J. S. and Ismail, M. (2014). Meteorological Characterization of Effective Earth Radius Factor (k-Factor) Wireless Radio Link Over Akure, Nigeria. Metrology Society of India 29 (2): 131-141
2. Abidin, Z. Z., Adnan, S. B. R. S. and Ibrahim, Z.A. (2009). RFI profiles of prime candidate sites for the first radio astronomical telescope in Malaysia. New Astronomy 15(3): 307–312.
3. Merola, L. (2006). A Study of the Effects of Sunrise and Sunset on the Ionosphere as Observed by VLF Wave Behavior (online). http://solar-center.stanford.edu/SID/StudentWork/LeandraMerola.pdf (21 April 2015)

4. Ezekoye, B. A. and Obodo, R. M. (2007). The Effects of Solar Radiations on Telecommunications. The Pacific Journal of Science and Technology 8 (1): 109-117

5. MCMC manual of ‘Spectrum Plan (2006). MCMC manual of ‘Spectrum Plan, Malaysian Communication and Multimedia Commission’ (2006). Resources Assignment Management Department Available: http://www.mcmc.gov.my/what_we_do/spectrum/plan.asp.

6. Abidin, Z. Z., Ibrahim, Z. A., Rosli, Z., Malim, S. F. F., Anim, N. M. and Noorazlan, N. (2012). An RFI Investigation for Setting up a VLBI Station below 2.8 GHz in Malaysia. New Astronomy 17 : 61-64

7. Abidin, Z.Z., Umar, R., Ibrahim, Z. A., Rosli, Z., Asanok, K. and Gasiprong, N.  (2013). Investigation on the Frequency Allocation for Radio Astronomy at the L Band. Publications of the Astronomical Society of Australia (PASA) 30: 47-57.
8. Umar, R., Abidin, Z. Z., Ibrahim, Z. A., Rosli, Z. and Noorazlan, N. (2014). Selection of Radio Astronomical Observation Sites and Its Dependence on Human Generated RFI. Research in Astronomy and Astrophysics 14 (2) : 241-248
