Malaysian Journal of Analytical Sciences Vol 19 No 5 (2015): 1075 - 1082

TREND ANALYSIS OF PAHANG RIVER USING NON-PARAMETRIC ANALYSIS: MANN KENDALL’S TREND TEST

(Analisis Corak Sungai Pahang Menggunakan Kaedah Bukan Parametrik: Ujian Corak

Mann Kendall)

Nur Hishaam Sulaiman1, Mohd Khairul Amri Kamarudin1,2*, Ahmad Dasuki Mustafa1,

Muhammad Azizi Amran1,  Fazureen Azaman1, Ismail Zainal Abidin1, Norsyuhada Hairoma1

# 1East Coast Environmental Research Institute (ESERI)

# 2Faculty of Design Arts and Engineering Technology

# Universiti Sultan Zainal Abidin, 21300 Kuala Terengganu, Terengganu, Malaysia

*Corresponding author: mkhairulamri@unisza.edu.my

Received: 14 April 2015; Accepted: 9 July 2015

Abstract

Flood is common in Pahang especially during northeast monsoon season from November to February. Three river cross station: Lubuk Paku, Sg. Yap and Temerloh were selected as area of this study. The stream flow and water level data were gathered from DID record. Data set for this study were analysed by using non-parametric analysis, Mann-Kendall Trend Test. The results that obtained from stream flow and water level analysis indicate that there are positively significant trend for Lubuk Paku (0.001) and Sg. Yap (<0.0001) from 1972-2011 with the p-value < 0.05. Temerloh (0.178) data from 1963-2011 recorded no trend for stream flow parameter but negative trend for water level parameter. Hydrological pattern and trend are extremely affected by outside factors such as north east monsoon season that occurred in South China Sea and affected Pahang during November to March. There are other factors such as development and management of the areas which can be considered as factors affected the data and results. Hydrological Pattern is important to indicate the river trend such as stream flow and water level. It can be used as flood mitigation by local authorities.

Keywords:  Mann Kendall, flood, trend analysis, hydrology, Pahang River

Abstrak

Banjir di Pahang pada musin  monsun timur laut yang membadai pantai timur semenanjung Malaysia pada  November hingga Febuari. Terdapat tiga buah stesen hidrologi yang ditempatkan di sepanjang Sungai Pahang telah dipilih untuk mewakili keseluruhan Sungai Pahang iaitu Lubuk Paku, Sg.Yap dan Temerloh. Dalam kajian ini juga, parameter yang digunakan ialah bersifat fizikal iaitu kelajuan arus dan paras air Sungai Pahang yang diperoleh daripada JPS. Kesemua data yang diperoleh dianalisa dengan menggunakan analisa bukan parametrik iaitu Ujian Corak Mann-Kendall. Keputusan yang diperoleh daripada hasil analisis data menunjukkan bahawa terdapat corak peningkatan positif pada data yang mewakili Lubuk Paku dan Sungai Yap dari tahun 1972-2011. Kesemua p-value yang dikira menunjukkan jelas terdapat corak dalam kedua-dua data pertama untuk Lubuk Paku (0.001) dan Sungai Yap (kurang daripada 0.0001) di mana jumlah p-value kurang daripada 0.05. Temerloh (0.178) mencatatkan tiada corak untuk kelajuan arus manakala untuk paras air ia mencatatkan corak negatif. Terdapat pelbagai faktor yang mempengaruhi corak hidrologi Sungai Pahang iaitu dari segi tiupan angin monsun dan pembangunan yang dijalankan di kawasan kajian yang memberi impak yang besar terhadap ketepatan data dan keputusan analisa. Pemantauan corak hidrologi amat penting untuk mengetahui corak sungai dan boleh digunakan sebagai maklumat persediaan banjir di Pahang.

Kata kunci:  Mann Kendall, banjir, analisis corak, hidrologi, Sungai Pahang

References

- Sulaiman W. N. A, Heshmatpoor A. and Rosli M. H. (2010). Identification of flood source areas in Pahang River Basin Peninsular Malaysia, Environment Asia (special issue) 3: 73-78.

- Saher F. N, Ali N. M, Abdul Kadir T. A, Teruggi G. and Hossain M. A. (2012). Environmental degradation inMalaysia’s Pahang river basin and its relation with river pollution: strategic plan from assesement to mitigation using geo-informatics. International conference on energy, environment and sustainable development, EEM-25.

- Jusoh, J. (2005). Hydrological Forecasting of Pahang River basin using the Rainfall-Runoff Model HEC-HMS, Universiti Teknologi Mara.

- Jabatan Meteorologi Malaysia, JMM. ( 2010).

- Gasim M. B, Toriman M. E, Idris M, Lun P. I, Kamaruddin M. K. A, Nor Azlina A. A, Mokhtar M, and Sharifah Mastura, M. A. (2013). River flow condition and dynamic state analysis of Pahang river. American Journal of Applied Sciences 10 (1): 45-47.

- Adnan N. A. and Atkinson P.M. (2008). Hydrometeorological trend analysis in a monsoon catchment. 8th annual meeting of the EMS volume 5, EMS2008-A-00368.

- Jayawardene H. K. W. I, Sonnadara D. U. J. and Jayewardene D.R (2005). Trends of Rainfall in Sri Lanka over the Last Century. Sri Lankan Journal Physic 6: 7-17.

- Simeonov, V., Stefanov, S. and Tsakovski, S. (2000). Environmetrical Treatment of Water Quality Survey Data from Yantra River, Bulgaria. Microchimica Acta 134: 15–21.

- Hamilton J. P, Whitelaw G. S. and Fenech A. (2001). Mean annual temperature and annual precipitation trends at Canadian biosphere reserves. Environmental Monitoring and Assessment 67: 239–275.

- Yue S. and Wang C. (2004). The Mann-Kendall Test Modified by Effective Sample Size to Detect Trend in Serially Correlated Hydrological Series. Water Resources Management 18: 201-18.

- Jamaludin S. and Sayang M. D. (2010). Trend in Peninsular Malaysia Rainfall Data During The Southwest Monsoon and Northeast Monsoon Season: 1975-2004. Sains Malaysiana 39(4): 533-542.

- Flood Disaster Archive (2009), National Security Council of Malaysia.

- Department of Drainage and Irrigation (DID, JPS) Flood Archive (2010).

- Mann, H. B. (1945). Nonparametric tests against trend. Econometrica 13: 245-259.

- Kendall, M. G. (1975). Rank Correlation Methods. 4th edition. London: Charles Griffin.