# Malaysian Journal of Analytical Sciences Vol 19 No 5 (2015): 1155 - 1167

# ASSESSMENT OF LAND USE CHANGE AND SEDIMENTATION MODELLING ON ENVIRONMENTAL HEALTH

IN TROPICAL RIVER

# (Penilaian Perubahan Guna Tanah dan Permodelan Sedimentasi ke atas

# Kesihatan Persekitaran di Sungai Bertropika)

# Mohd Ekhwan Toriman1,2, Mohd Khairul Amri Kamarudin1,3*, Thudchai Sansena4, Kampand Bhaktikuld5,

# Roslan Umar1, Asyaari Muhamad1, Nor Azlina Abd Aziz1, Nur Hishaam Sulaiman1

# 1East Coast Environmental Research Institute (ESERI),

# Universiti Sultan Zainal Abidin, 21300 Kuala Terengganu, Terengganu. Malaysia

# 2School of Social, Development and Environmental Studies, Faculty of Social Sciences and Humanities,

# Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

# 3Faculty of Design Arts and Engineering Technology,

# Universiti Sultan Zainal Abidin, Gong Badak Campus, 21300 Kuala Terengganu Terengganu. Malaysia

# 4Informatics and Space Technology Department Agency, 10150 Bangkok, Thailand.

# 5Faculty of Environment and Resource Studies,

# Mahidol University, Phutthamonthon 73170, Nakhon Pathom, Thailand

*Corresponding author: mkhairulamri@unisza.edu.my

Received: 14 April 2015; Accepted: 9 July 2015

Abstract

Sediments are defined as the organic and inorganic materials or solid fragments derived from the weathering processes of sand, pebbles, silt, mud and loess. The objective of this research is to forecast sediment volume in the Lam Phra Phloeng reservoir by using the Neuro-genetic Optimizer model to calculate the sediment volume from runoff, rainfall, and sediment volume data. The results from satellite imagery interpretation elucidated that from 2002 to 2005, forest area decreased approximately 50,220 km2 or 36 %, and was converted to agricultural land. By applying the USLE equation, the soil erosion area was found to increase approximately 185,341 tons/year between 2002 and 2005. This result illustrated that the impact of land use change greatly increased sedimentation volume. In applying the Neuro-genetic Optimizer model, the learning rate and momentum of this model was 0.9 and 0.1, respectively, and the initial weight value was +/-3. The model forecasted the annual sediment volume in the Lam Phra Phloeng reservoir in 2005 to be 49,855 tons with R2 equals to 0.9994. The regression model, on the other hand, forecasted the sediment volume using the equation Y=198. 48x 1.1783 with R2 equals to 0.9974, and the annual sediment volume was estimated to be 45,346 tons. The actual sediment volume in the reservoir in 2005 was obtained from The Royal Irrigation Department, which was found to be 48,697 tons.

# Keywords: sedimentation, land use change, Tropical River; USLE; neuro-genetic optimizer

Abstrak

# Sedimen boleh ditakrifkan sebagai bahan organik dan bukan organik atau serpihan pepejal yang diperolehi daripada proses luluhawa pasir, batu kecil, kelodak, lumpur dan loess. Objektif kajian ini adalah untuk meramal jumlah sedimen dalam takungan Sungai Lam Phra Phloeng dengan menggunakan model Neuro-genetik Optimizer untuk mengira jumlah sedimen daripada data larian air, hujan, dan jumlah sedimen. Hasil daripada tafsiran imej satelit pada tahun 2002-2005, kawasan hutan merosot kira-kira 50.220 km2 atau 36%, dan telah ditukar kepada tanah pertanian. Dengan menggunakan persamaan USLE, kawasan hakisan tanah didapati telah meningkatkan kira-kira 185.341 tan / tahun antara tahun 2002 dan 2005. Hasil kajian menunjukkan kesan perubahan guna tanah dan sedimentasi adalah meningkat. Berdasarkan model Neuro-genetik Optimizer, kadar pembelajaran dan momentum model ini adalah 0.9 dan 0.1, dan nilai berat badan awal adalah +/- 3. Model ini meramalkan jumlah sedimen tahunan dalam takungan Lam Phra Phloeng pada 2005 meningkat kepada 49.855 ton yang bersamaan dengan R2 0.9994. Model regresi, di bahagian lain pula, diramalkan dengan menggunakan persamaan Y = 198. 48 x 1.1783 dengan bersamaan dengan R2 0.9974. Jumlah sedimen tahunan pula dianggarkan sebanyak 45346 tan. Jumlah sedimen sebenar dalam takungan pada tahun 2005 telah diperolehi daripada Jabatan Pengairan Diraja, iaitu sebanyak 48.697 tan.

# Kata kunci: sedimentasi, perubahan guna tanah; Sungai Tropika; USLE; neuro-genetik optimizer

References

- Biswas, A. K., (1999). Management of international water: Opportunities and constraints. Journal of Water Resource Development. 15(4): 429-441.

- Kamarudin, M. K. A., Toriman, M. E.,  Rosli, M. H., Juahir, H., Azid, A., Mohamed Zainuddin, S. F., Abdul Aziz, N. A. and Sulaiman, W. N. A., (2014). Analysis of meander evolution studies on effect from land use and climate change at upstream reach of Pahang River, Malaysia. Mitigation and Adaptation Strategies for Global Change : 1- 10

- Toriman, M. E., Gasim, M.B., Yusop, Z., Shahid, I., Mastura, S.A.S., Abdullah, P., Jaafar, M., Aziz, N.A.A., Kamarudin, M. K. A., Jaafar, O., Karim, O., Juahir, H. and Jami, N.R., (2012). Use of 137Cs activity to investigate sediment movement and transport modeling in river coastal environment. American Journal of Environmental Science 8: 417-423.

- Jaafar. O., Toriman, M.E., Idris, M., Sharifah Mastura, S.A., Juahir, H., Abdul Aziz, N.A., Kamarudin, M.K.A. and Jamil, N.R., (2010). Study of water level-discharge relationship using artificial neural network (ANN) in Sungai Gumum, Tasik Chini, Pahang, Malaysia. Research Journal of Applied Sciences. 5(1): 20-26.

- Jaafar, O., Toriman, M. E., Sharifah Mastura, S. A., Gazim, M. B., Lan, P. I., Abdullah, P., Kamarudin, M. K. A. and Abdul Aziz, N.A., (2010). Modeling the impacts of ringlet reservoir on downstream hydraulic capacity of Bertam River using XPSWMM in Cameron Highlands, Malaysia. Research Journal of Applied Sciences 5(2): 47-53.

- Wischmeier, and Smith, D.D., (1978). Predicting Rainfall Erosion Losses; A Guide to conservation planning. Agriculture. Handbook No.537, USDA, Washington, DC, pp. 57.

- Hickin, E.J., (1983). River Channel Changes: Retrospect and Prospect. In:  River Channel Changes. Modern and Ancient. Fluvial System, Collinson, J.D. and J. Lewin (Eds.), Blackwell Scientific. London. pp: 61-83.

- Parveen, R. and Kumar, U., (2012). Integrated approach of Universal Soil Loss Equation (USLE) and Geographical Information System (GIS) for soil loss risk assessment in Upper South Koel Basin, Jharkhand. Journal of Geographic Information System. 4: 588-596.

- Ghosh, S. and Guchhait, S.K. (2012). Soil loss estimation through USLE and MMF methods in the lateritic ttracts of Eastern Plateau Fringe of Rajmahal Traps, India. Ethiopian Journal of Environmental Studies and Management. 5(4): 529 – 541.

- Li H., Chen X., Kyoung J. L., Cai X. and Myung S. (2010). Assessment of soil erosion and sediment yield in Liao Watershed, Jiangxi Province, China, using USLE, GIS, and RS. Journal of Earth Science. 21(6): 941–953.

- Zhang Q., Yuan L. and Andreas M. (2008). GIS-based approach for change and prediction of soil erosion. The International Archives of the Photogrammetry, Remote Sensing and Spatial Information Science 36: 859 – 864.

- Kim K.T., Kim, J.H., Park, J.S., Choi, Y.S., (2005). Analysis of soil erosion hazard zone in Farmland. proceedings.esri.com,http://proceedings.esri.com/library/userconf/proc05/papers/pap1834.pdf

- Mutalib, S. N. Rahim, S. A., Juahir, H., Azid, A., Sharif, S.M., Latif, M.T., Aris, A.Z., Zain, S.M.,   and Dominick, D. (2013). Spatial and temporal air quality pattern recognition using environmetric techniques: A case study in Malaysia.  Environmental Science: Processes Impacts.  15: 1717-1728.

- Azid, A., Juahir, H., Latif, M.T., Zain, S. M. and Osman, M. R. (2013).  Feed-forward artificial neural network model for air pollutant index prediction in the Southern Region of Peninsular Malaysia.  Journal of Environmental Protection 4(12A): 1-10.

- Shukla, K. K., (2000). Neuro-genetic prediction of software development effort. Information and Software Technology 42 (10): 701–713.

- Toriman, M.E., Ata, F. M., Kamarudin M. K. A. and Idris, M. (2013). Bed-load sediment profile and effect of river bank erosion on river cross-section. American Journal Environmental Sciences 6: 292-300.

- Lun P. I., Gasim, M. B., Toriman, M.E., Rahim, S. A., Kamarudin, M. K. A., (2011). Hydrological pattern of Pahang river basin and their relation to flood historical event. e-BANGI. 4(1): 29-37.

- Toriman, M. E., Lee Q. Y., Kamarudin, M. K. A.,  Aziz, N. A. A., Mokhtar, M., Elfithri, R. and Bhaktiku, K. (2014). Applying Seasonal Climate Trends To Agricultural Production In Tanjung Karang, Malaysia. American Journal Agriculture Biological Sciences 9 (1): 119-126.

- Kamarudin, M. K. A., Idris, M. and Toriman, M. E. (2013). Analysis of Leptobarbus hoevenii in control environment at natural lakes.  American Journal Agriculture Biological Sciences 8: 142-148.

- Gasim, M.B., Ismail, B.S., Sajaul Islam-Mir, Rahim, S.A. and Toriman, M.E., (2011). The physico-physical properties of four soil series in Tasik Chini, Pahang Malaysia. Asean Journal of Earth Sciences. 1-10.

- Kamarudin, M. K. A., Toriman, M. E., Sharifah Mastura, S. A., Idris, M., Jamil, N. R. and Gasim, M.B., (2009). Temporal variability on lowland river sediment properties and yield. American Journal of Environmental Sciences 5(5): 657-663.

- Juahir, H., Sharifuddin, M. Z., Mazlin, M., Toriman, M. E., Jalaludin, Z. and Jan, I.K.M., (2008). The use of chemometrics analysis as a cost-effective tool in sustainable utilisation of water resources in the Langat River catchment, Malaysia. American-Eurasian Journal of Agriculture and Environmental Science 4(2): 258-265.

- Gasim, M. B., Toriman, M. E., Rahim, S. A., Mir Sujaul Islam, Chek, T. C. and Juahir, H., (2006). Hydrology and water quality assessment of Tasik Chini’s feeder rivers, Pahang Malaysia. Geografia 3 (3): 1-16.