Malaysian Journal of Analytical Sciences Vol 19 No 6 (2015): 1309 - 1317

DETERMINATION OF CROSSLINK CONCENTRATION BY MOONEY-RIVLIN EQUATION FOR VULCANIZED NR/SBR BLEND AND ITS INFLUENCE ON MECHANICAL PROPERTIES

(Menentukan Kepekatan Hubung Silang Menggunakan Persamaan Mooney-Rivlin untuk Campuran Getah Tervulkan NR/SBR dan Pengaruhnya Terhadap Sifat Mekanikal)

Azreen Izzati Dzulkifli*, Che Mohd Som Said, Chan Chin Han

Faculty of Applied Sciences,

Universiti Teknologi MARA, 40450 Shah Alam, Selangor, Malaysia

*Corresponding author: izzati_dzulana@yahoo.com

Received: 23 November 2014; Accepted: 3 September 2015

Abstract

Crosslink concentration is an important property affecting the major characteristic of cured rubber. The crosslink concentration was determined using Mooney-Rivlin equation due to its simple and reliable method. Cured natural rubber and styrene butadiene rubber blend (NR/SBR) with different crosslink concentrations were obtained with different blend ratios of 100/0, 80/20, 70/30, 60/40, 50/50, 40/60 and 0/100. The crosslink concentrations were determined using Mooney-Rivlin Equation and its influence on International Rubber Hardness Tester (IRHD), tensile strength and rebound resilience of NR/SBR blend vulcanizates was investigated. The results showed different blend ratios had an influence on the crosslink concentration of the NR/SBR blend vulcanizates. Obtained data showed that high NR content in NR/SBR blend increased the crosslink concentration. The highest crosslink concentration recorded was for 100/0 blend ratio which was 0.0498 mol kg-1 RH while the lowest was 0.0295 mol kg-1 RH for 0/100 blend ratio. The study on the influence of crosslink concentration on IRHD, tensile strength and rebound resilience of NR/SBR blend vulcanizates showed that the mechanical properties increased linearly with the crosslink concentration. High NR content in NR/SBR blends resulted in higher crosslink concentration which improved the performance of mechanical properties for NR/SBR blend.

# Keywords: crosslink concentration, Mooney-Rivlin equation, natural rubber, styrene butadiene rubber

Abstrak

Kepekatan hubung silang adalah perkara penting yang mempengaruhi ciri-ciri utama getah tervulkan. Kepekatan hubung silang telah ditentukan dengan menggunakan persamaan Mooney-Rivlin kerana ia merupakan kaedah yang mudah dan telah diperakui. Campuran getah asli dan stirena butadiena (NR/SBR) dengan kepekatan hubung silang yang berbeza telah diperolehi oleh penggunaan nisbah campuran yang berbeza iaitu 100/0, 80/20, 70/30, 60/40, 50/50, 40/60 dan 0 / 100. Kepekatan hubung silang telah ditentukan menggunakan persamaan Mooney-Rivlin dan pengaruhnya ke atas Ujian Kekerasan Getah Antarabangsa (IRHD), kekuatan tegangan dan pemulihan daya tahan campuran getah NR/SBR telah dikaji. Hasil kajian menunjukkan nisbah campuran yang berbeza mempengaruhi kepekatan hubung silang getah tervulkan NR/SBR. Data yang diperolehi menunjukkan bahawa kandungan getah asli (NR) yang tinggi dalam campuran NR / SBR memberikan peningkatan terhadap kepekatan hubung silang. Kepekatan hubung silang tertinggi dicatatkan adalah untuk nisbah campuran 100/0 iaitu 0.0498 mol kg-1 RH, manakala yang terendah adalah 0.0295 mol kg-1 RH untuk nisbah campuran  0/100. Kajian ke atas pengaruh kepekatan hubung silang terhadap ujian IRHD, kekuatan tegangan dan pemulihan daya tahan  campuran getah masak NR/SBR menunjukkan bahawa sifat-sifat mekanikal meningkat secara selari dengan kepekatan hubung silang. Kandungan getah asli (NR) yang tinggi dalam campuran getah NR/SBR menyebabkan kepekatan hubung silang yang lebih tinggi dan sekaligus meningkatkan prestasi sifat mekanik campuran getah NR/SBR.

Kata kunci: kepekatan hubung silang, persamaan Mooney Rivlin, getah asli, getah stirena butadiena

References

      - El-Sabbagh, S. H. and Yehia, A. A. (2007). Detection of Crosslink Density by Different Methods for Natural Rubber Blended with SBR and NBR. Polymers and Pigmens Department, National Research Centre, Dokki, Cairo, Egypt. Egyptian Journal of Solids 30 (2): 157 – 173.

      - Findik, F., Yilmaz, R. and Koksal, T. (2004). Investigation of Mechanical and Physical Properties of Several Industrial Rubbers. Materials and Design 25 (4): 269 – 276.

      - Ismail, M. N., El-Sabbagh, S. H., and Yehia, A. A. (1999). Fatigue and Mechanical Properties of NR/SBR and NR/NBR Blend Vulcanizate. Journal of Elastomers and Plastics, 31 (3): 255 – 270.

      - Azemi Samsuri. (2009). An Introduction to Polymer Science and Rubber Technology. University Publication Centre (UPENA) ISBN: 978-967-305-288-2.

      - Bristow, G. M. and Porter, M. (1967). Determination of Degree of Chemical Crosslinking of Natural Rubber Gum Vulcanizate Networks. Journal of Applied Polymer Science 11 (11): 2215 – 2226.

      - Yahya, N. Z. N., Zulkepli, N. N., Ismail, H., Ting, S. S. and Santiago, R. (2013). NR/SBR/rNBRg Ternary Blend: Tensile and Morphology. Special issue for ICAMET, 2013, Bandung.

      - Joseph R., George, K. E and Francis, D. J. (1987). Studies on 50/50 NR/SBR Copolymer Blend. Die Angewandte Makromolekulare Chemie 152 (1): 107 – 119.

      - Goyanes, S., Lopez, C. C., Rubiolo, G. H., Quasso, F., and Marzocca, A. J. (2008). Thermal Properties in Cured Natural Rubber/ Styrene Butadiene Rubber Blends. European Polymer Journal 44 (5): 1525-1534.