# Malaysian Journal of Analytical Sciences Vol 20 No 1 (2016): 179 - 186

# Residue Determination and Monitoring OF the Levels of Metsulfuron Methyl in Selected Rivers at Tasik Chini, Pahang, Malaysia

## (Penentuan Residu dan Pemantauan Metsulfuron Metil di Sungai Terpilih di Tasik Chini, Pahang, Malaysia)

Khairiatul Mardiana Jansar* and Ismail Bin Sahid

School of Environment and Natural Resource Sciences,

Faculty of Science and Technology,

Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

*Corresponding author: mardiana@ukm.edu.my

Received: 9 December 2014; Accepted: 16 October 2015

Abstract

The levels of metsulfuron methyl (methyl 2-(4-methoxy-6-methyl-1,3,5-trazine-2-ylcarbamoylsulfamoyl) benzoate (IUPAC) (MSM) and its metabolite IN-A4098 (4-methoxy-6-methyl-1,3,5-triazin-2-amine (AMMT) were determined from surface water samples of six rivers located at Tasik Chini, Pahang, Malaysia. The optimization analytical method was developed for the determination of the MSM herbicide and its metabolite, AMMT in surface water up to 0.05 ng/ml with good linearity in the calibration range 0.1 – 10 ng/ml. The procedure involves a clean-up process using the IST ISOLUTE ENV+ solid phase extraction (SPE) method and the pesticide level are determined using the HPLC with variable wavelength detector (VWD) and 81 – 105 % recovery. The MSM compound represents one of main herbicides used in the Tasik Chini oil palm plantation. The method enables study to be made of the herbicide and its transformation products through simultaneous detection and quantification. In the field study levels of MSM in the surface water ranged from undetectable to 0.1378 ng/ml from the first season to forth season, while levels of the metabolite, AMMT ranged from undetectable to 1.473 ng/ml. The variation of the MSM and AMMT levels depended directly on the time of the pesticide application and the season.

Keywords: metsulfuron methyl, metabolite, solid phase extraction, Tasik Chini

Abstrak

Tahap pencemaran metsulfuron metil (metil 2-(4-metoksi-6-metil-1,3,5-triazin-2-ilkarbamoilsulfamoil) benzoat (IUPAC) (MSM) dan metabolit IN-A4098 (4-metoksi-6-metil-1,3,5-triazin-2-amina (AMMT) telah ditentukan daripada sampel air permukaan daripada enam batang sungai yang terletak di Tasik Chini, Pahang, Malaysia. Pengoptimuman kaedah analisis telah dibuat untuk menentukan kandungan herbisid MSM dan metabolitnya, AMMT dalam air permukaan dengan kepekatan minimum adalah 0.05 ng/ml dan julat kalibrasi adalah 0.1 – 10 ng/ml. Prosedur ini melibatkan satu proses pengekstrakkan menggunakan IST ISOLUTE ENV+ kaedah pengekstrakan fasa pepejal (SPE) dan tahap herbisid ditentukan menggunakan HPLC  dengan pengesan gelombang pelbagai (VWD) pada tahap perolehan semula adalah 81 – 105 %. Sebatian MSM merupakan salah satu daripada herbisid utama yang digunakan dalam ladang kelapa sawit Tasik Chini. Kaedah ini membolehkan herbisid dan metabolitnya dikesan dan dianalisis serentak. Di peringkat kajian lapangan, julat bacaan MSM dalam air permukaan adalah dari tidak dapat dikesan sehingga 0.1378 ng/ml dari musim pertama hingga musim keempat, manakala tahap pencemaran AMMT, yakni metabolitnya adalah dalam julat yang tidak dapat dikesan hingga 1.473 ng/ml. Tahap pencemaran MSM dan AMMT bergantung secara langsung kepada masa penyemburan herbisid dan musim kawasan tersebut.

Kata kunci: metsulfuron metil, metabolit, pengekstrakkan fasa pepejal, Tasik Chini

References

- Beyer, E .M., Duffy, M. J., Hay, J. V. and Schlueter, D. D. (1988). Sulfonylureas. In: Kearney, P.C., and Kaufman, D.D. (Eds.), Herbicides. Chemistry, Degradation and Mode of Action, Dekker, New York.

- Ray, T. B. (1984). Site of action of chlorsulfuron: inhibition of valine and isoleucine biosynthesis in plants. Plant Physiology, 75, 827 – 831.

- Health Canada Pest Management Regulatory Agency (2008). Re-evaluation Decision. Metsulfuron Methyl. Ontario: Minister of Public Works and Government Services Canada.

- Abdullah, A. R, Saraswathy, S. and Norhayati, M. T. (2001). Adsorption-desorption and mobility of metsulfuron methyl in selected Malaysian agricultural soils. Bulletin of Environmental Contamination and Toxicology 66: 762 – 769.

- Brown, H. M., Gaddamidi, V. and Lee, P. W. (1998). Sulfonylureas. In: Hutsons, D. H., and Roberts, T. (Eds.), Metabolic Pathways of Agrochemicals: Herbicides and plant growth regulators. The Royal Science of Chemistry, Cambridge.

- Kjaer, C. M. Strandberg, and Erlandsen, M. (2006). Effects on hawthorn the year after simulated spray drift. Chemosphere. 63(5): 853 – 859.

- Li, Z. J., Xu, J. M., Muhammad, A. and Ma, G. R. (2005). Effect of bound residues of Metsulfuron-methyl in soil on rice growth. Chemosphere. 58: 1177 – 1183.

- Berger, B. M. and Wolfe, N. L. (1996). Hydrolysis and biodegradation of sulfonylurea herbicides in aqueous buffers and anaerobic water-sediment systems: assessing fate pathways using molecular descriptors. Environmental Toxicology Chemistry, 15:1500 – 1507.

- Wang, H., Yang, F., Liu, G., Lei, Y. and Ye, Q. (2011). Monitoring of metsulfuron-methyl and its residues in an artificial pond. Environmental Monitoring Assessment, 174(1-4):597 – 604.

- Singh S. B. and Paul, R. (2011). Gas chromatographic method for residue analysis of metsulfuron methyl from soil. Bulletin of Environmental Contamination and Toxicology, 86(2):149 – 151.

- Bossi, R., Seiden, P., Andersen, S. M., Jacobsen, C. S. and Streibig, J. C. (1999). Analysis of metsulfuron-methyl in soil by liquid chromatography/Tandem Mass Spectrometry. Application to a field dissipation study. Agricultural Food Chemistry, 47(10):4462 – 4468.

- National Council for Air and Stream Improvement, Inc. (NCASI) (2007). Hexazinone, Imazapyr, Metsulfuron Methyl, and Sulfometuron Methyl in Water by Solid Phase Extraction and HPLC/UV. NCASI Methods Manual. Research Triangle Park, NC: National Council for Air and Stream Improvement, Inc.

- Shobha S. (2008). Persistence of metsulfuron-methyl in wheat crop and soil. Environmental Monitoring and Assessment, 147:463 – 469.

- Halimah, M. Tan, Y. A. and Ismail, B. S. (2007). New and rapid method of analysis of metsufuron methyl in water sample. Proceedings of Product Development and Nutrition Conference: PIPOC 2007 International Palm Oil Congress: Palm Oil: Empowering Change. Kuala Lumpur (Malaysia), 26 – 30 Aug, 2007: pp 176.

- Juhler, R., Sørensen, S. and Larsen, L. (2001). Analysing transformation products of herbicide residues in environmental samples. Water Research, 35(6), 1371–1378.