Malaysian Journal of Analytical Sciences Vol 20 No 1 (2016): 21 - 30

ENZYMATIC SACCHARIFICATION ON AMMONIA PRE-TREATED OIL PALM TRUNK BIOMASS FOR GLUCOSE PRODUCTION:

AN OPTIMIZATION USING RESPONSE SURFACE METHODOLOGY

(Sakarifikasi Enzim pada Prarawatan Ammonia Biomas Batang Kelapa Sawit bagi Penghasilan Glukosa: Pengoptimuman dengan Kaedah Gerak Balas Permukaan)

Long Wee Lai*, Siti Sarenah Mohd Yahya, Norakma Mohd Nor, Moohamad Ropaning Sulong

Faculty of Science and Biotechnology,

Universiti Selangor,

Jalan Timur Tambahan, 45600 Bestari Jaya, Selangor, Malaysia

*Corresponding author: zki@unisel.edu.my

Received: 14 August 2015; Accepted: 31 December 2015

Abstract

This paper reported the optimization of glucose production from ammonia pre-treated oil palm trunk biomass via enzymatic saccharification process using response surface methodology (RSM). A set of experiment was computed by face centered central composite design (FCCCD) using Design Expert software to statistically evaluate the findings. Three independent variables namely: temperature (35 – 55 °C), pH (4.5 – 6.5) and enzyme ratio (cellullase: β-glucosidase; 3:1 – 7:1) were investigated under the given conditions designed by RSM. The experimental result (4.964 ± 0.006 g.L-1) was in good agreement with RSM model prediction (4.958 g.L-1) with an error less than 0.2 %. The RSM design has been proven to successfully predict the glucose response in this study.

Keywords: 	oil palm trunk biomass, enzymatic saccharification, lignocellulosic substrate, ammonia pre-treament, response  surface methodology

Abstrak

Kertas kerja ini melaporkan pengoptimuman hasilan glukosa daripada prarawatan ammonia biomas batang kelapa sawit melalui proses sakarifikasi enzim dengan menggunakan kaedah gerak balas permukaan (RSM). Satu set eksperimen telah dibentuk  iaitu kaedah muka berpusat reka bentuk komposit dengan menggunakan perisian Design Expert bagi menilai penemuan kajian secara statistik. Tiga pembolehubah bebas iaitu: suhu (35 – 55 ° C), pH (4.5 – 6.5) dan nisbah enzim (selulase: β-glukosidase; 3:1 – 7:1) telah disiasat di bawah situasi yang direka oleh RSM. Hasil eksperimen (4.964 ± 0.006 g.L-1) yang diperoleh berada dalam persetujuan yang baik dengan ramalan model RSM (4.958 g.L-1) iaitu pada ralat kurang daripada 0.2 %. Reka bentuk RSM telah terbukti berjaya meramalkan respon glukosa dalam kajian ini.

Kata kunci: 	biomas batang kelapa sawit, sakarifikasi enzim, substrat lignoselulosa, pra-rawatan ammonia, kaedah gerak balas permukaan

References

- Chooklin, S, Kaewsichan, L. and Kaewsrichan, J. (2013). Potential Use of Oil Palm Sap on Lactic Acid Production and Product Adsorption on Dowex™ 66 Resin as Adsorbent. Asia Pacific Journal Chemical Engineering, 8(1): 23 – 31.

- Amouzgar, P, Khalil, H. P. S. A, Salamatinia, B, Abdullah, A. Z. and Issam, A. M. (2010). Optimization of Bioresource Materials from Oil Palm Trunk Core Drying Using Microwave Radiation: A Response Surface Methodology Application. Bioresource Technology, 101: 8396 – 8401.

- Yuanisa, A, Kafidul Ulum, K. and Wardani, A. K. (2015). Pretreatment of Oil Palm Trunk Lignocellulose as First Step to Produce Second Generation of Bioethanol: A Review. Jurnal Pangan Agroindustri, 3(4): 1620 – 1626.

- Lai, L. W. and Idris, A. (2013). Disruption of Oil Palm Trunks and Fronds by Microwave-Alkali Pretreatment. Bioresources, 8(2): 2792 – 2804.

- Sukri, S. S. M., Rahman, R. A., Illias, R. M. D. and Yaakob, H. (2014). Optimization of Alkaline Pretreatment Conditions of Oil Palm Fronds in Improving the Lignocelluloses Contents for Reducing Sugars Production. Romanian Biotechnology Letter, 19(1): 9006 – 9018.

- Binod, P, Satyanagalakshmi, K, Sindhu , R, Janu, K. U, Sukumaran, R. K. and Pandey, A. (2012). Short Duration Microwave Assisted Pretreatment Enhances the Enzymatic Saccharification and Fermentable Sugar Yield from Sugarcane Bagasse. Renewable Energy, 37: 109 –116.

- Jung, Y. H, Lee, H. J, Seo, J. H, Kim, S, Seung, D, Choi, I. G, Yang, T. H, Park, Y. C. and Kim, K. H. (2012). Aqueous Ammonia Pretreatment, Saccharification, and Fermentation Evaluation of Oil Palm Fronds for Ethanol Production, Bioprocess and Biosystems Engineering, 35:1497 – 1503.

- Wong, Y. C, Tan, Y. P, Taufiq, Y. Y. H. and Ramli, I. (2015). An Optimization Study for Transesterification of Palm Oil Using Response Surface Methodology. Sains Malaysiana, 44(2): 281 – 290.

- Zhang, P. Y. H, Hong, J. and Ye, X. (2009). Cellulase Assay, In: Jonathan, R.M. (ed) Biofuels: Methods and Protocols, Methods in Molecular Biology. New York: Humana Press.

- Bommarius, A. S, Katona, A, Cheben, S. E, Patel, A. S, Ragauskas, A. J,  Knudson, K. and Pu, Y. (2008). Cellulase Kinetics as A Function of Cellulose Pretreatment. Metabolic Engineering, 10: 370 – 381.

- Dawson, R. M. C, Elliot, D. C, Elliot, W. H. & Jones, K. M. (1986). Data for Biochemical Research (3rd ed.). U. K.: Oxford Science Publication.

- Ghose, T. K. (1987). Measurement of Cellulase Activities. Pure Applied Chemistry, 59: 257 – 268.

- Lai, L.W, Idris, A. and Yusof, N. M. (2014). Lignin extraction from oil palm trunk by Microwave-Alkali technique. Malaysian Journal Fundamental Applied Science, 10: 59 – 63.

- Wee, L. L, Annuar, M. S. M, Ibrahim, S. and Chisti, Y. (2011). Enzyme-Mediated Production of Sugars from Sago Starch: Statistical Process Optimization. Chemical Engineering Communications, 198(11): 1339 – 1353.

- Alshaibani, M,  Yaakob, Alsobaai, Z. A. M. and Sahri, M. (2014). Optimization of Pd-B/ɣ-Al2O3 Catalyst Preparation for Palm Oil Hydrogenation by Response Surface Methodology (RSM).  Brazilian Journal of Chemical Engineering, 31(1): 69 – 78.

- Segel, I. H. (1976). Biochemical Calculations: How to Solve Mathematic Problem in General Biochemistry. USA: John Wiley and Sons.

- Shuler, M. L. and Kargi, F. (1992). Bioprocess Engineering: Basic Concepts. New Jersey: Prentice Hall.