Malaysian Journal of Analytical Sciences Vol 20 No 3 (2016): 626 - 632

QUALITY PARAMETERS OF Curcuma longa L. EXTRACTS BY SUPERCRITICAL FLUID EXTRACTION (SFE) AND ULTRASONIC ASSISTED EXTRACTION (UAE)

(Parameter Kualiti Ekstrak Curcuma longa L. daripada Pengekstrakan Cecair Supergenting (SFE) dan Pengekstrakan Ultrasonik Berbantu (UAE))

Zaibunnisa Abdul Haiyee1*, Siti Hafsah Mohd Shah1, Khudzir Ismail1, Nooraain Hashim1,

Wan Iryani Wan Ismail2

1Faculty of Applied Sciences,

Universiti Teknologi MARA, 40450 Shah Alam, Selangor, Malaysia

2Faculty of Pharmacy,

Universiti Teknologi MARA, 42300 Puncak Alam, Selangor, Malaysia

*Corresponding author: nisha@salam.uitm.edu.my

Received: 24 February 2015; Accepted: 27 October 2015

Abstract

Turmeric is one of the prominently use herbal plants due to its diverse beneficial effects especially in Indian medicine. The rhizome part of the turmeric contains valuable compounds which have been said to owe its antimicrobial effects, anti-cancer, anti-inflammatory and enhance wound healing. Due to its short-life span and perishable properties, the conversion of the rhizome into turmeric extract is desirable. Several methods have been used for extraction such as Soxhlet extraction and pressurized liquid extraction (PLE). However, these techniques are tedious, laborious, time consuming and involves the usage of toxic organic solvent, of which safeness of the end product is doubtful. In this study, a rapid, reliable and green extraction method of supercritical fluid extraction (SFE) and ultrasonic assisted extraction (UAE) were used. SFE without modifier has resulted in 0.0006 mg/100 g of curcuminoids concentration and 5.62 % of yield (dry weight basis). UAE using ethanol was able to produce significantly the highest yield (6.40 %, dry weight basis) and the highest curcuminoids concentration (0.1020 mg/100 g). However, SFE was able to produce extract that contain significantly higher major volatile compounds; tumerone, ar-turmerone and curlone. Therefore, this study proves that both extraction methods were able to produce high quality turmeric extract.

Keywords:  	Supercritical fluid extraction, Ultrasonic assisted extraction, percentage yield, curcuminoids, volatile compounds

Abstrak

Kunyit merupakan tumbuhan herba yang terkenal disebabkan kebaikannya yang pelbagai terutamanya dalam perubatan masyarakat India. Bahagian rizom kunyit mengandungi sebatian berkhasiat yang dipercayai mempunyai kesan antimikrobial, anti-kanser, anti-keradangan dan juga mempercepatkan kadar penyembuhan luka. Disebabkan kitar hidup yang singkat dan ciri mudah rosak yang terdapat pada kunyit, maka penukaran rizom kunyit kepada ekstrak kunyit adalah amat wajar. Beberapa kaedah pengekstrakan telah digunakan seperti pengekstrakan Soxhlet dan pengekstrakan cecair bertekanan (PLE). Bagaimanapun, kaedah-kaedah tersebut amat rumit, sukar, mengambil masa yang lama serta melibatkan penggunaan pelarut organik bertoksik yang menyebabkan keraguan terhadap tahap keselamatan produk iaitu ekstrak kunyit. Di dalam kajian ini, kaedah pengekstrakan yang pantas, dipercayai serta mesra alam iaitu pengekstrakan cecair supergenting (SFE) dan pengekstrakan ultrasonik berbantu (UAE) telah digunakan. SFE tanpa bahan pengubahsuai telah menghasilkan 0.0006 mg/100 g kepekatan kurkuminoids dan 5.62 % hasil ekstrak kunyit (berdasarkan berat kering). UAE menggunakan etanol telah menghasilkan hasil ekstrak yang ketara tinggi (6.40 %, berdasarkan berat kering) dan nilai kepekatan kurkuminoids tertinggi iaitu 0.1020 mg/100 g. Walaubagaimanapun, SFE telah menghasilkan ekstrak yang mengandungi sebatian meruap yang signifikan tingginya iaitu tumerone, ar-turmerone dan curlone. Maka, kajian ini membuktikan bahawa kedua-dua kaedah berupaya menghasilkan produk ekstrak kunyit yang berkualiti tinggi.

Kata kunci: Pengekstrakan cecair supergenting, Pengekstrakan ultrasonik berbantu, peratusan hasil, kurkuminoids, sebatian meruap

References

- Singh, S., Joshi, R. K. and Nayak, S. (2013). Identification of elite genotypes of turmeric through agroclimatic zone based evaluation of important drug yielding traits. Industrial Crops and Products, 43: 165 – 171.

- Braga, M. E. M., Leal, P. F., Carvalho, J. E. and Meireles, M. A. A. (2003). Comparison of yield, composition and antioxidant activity of turmeric (Curcuma longa L.) extracts obtained using various techniques. Journal of Agricultural & Food Chemistry, 51: 6604 – 6611.

- Began, G., Goto, M., Kodama, A. and Hirose, T. (2000). Response surfaces of total oil yield of turmeric (Curcuma longa) in supercritical carbon dioxide. Food Research International, 33: 341 – 345.

- Haiyee, Z. A., Saim, N., Said, M., Illias, R. M., Mustapha, W. A. W. and Hassan, O. (2009). Characterization of cyclodextrin complexes with turmeric oleoresin. Food Chemistry, 114: 459 – 465.

- Chang, L-H., Jong, T-T., Huang, H-S., Nien, Y-F. and Chang, C-M. J. (2006). Supercritical carbon dioxide extraction of turmeric oil from Curcuma longa Linn and purification of turmerones. Separation and Purification Technology, 47: 119 – 125.

- Vinatoru, M. (2001). An overview of the ultrasonically assisted extraction of bioactive principles from herbs. Ultrasonics Sonochemistry, 8: 303 – 313.

- Daneshvand, B., Ara, K. M. and Raofie, F. (2012). Comparison of supercritical fluid extraction and ultrasound-assisted extraction of fatty acids from quince (Cydonia oblonga Miller) seed using response surface methodology and central composite design. Journal of Chromatography A, 1252: 1 – 7.

- Wang, L. and Weller, C. L. (2006). Recent advances in extraction of nutraceuticals from plants. Trends in Food Science & Technology, 17: 300 – 312.

- Zaibunnisa, A. H., Norashikin, S., Mamot, S. and Osman. H. (2009). An experimental design approach for the extraction of volatile compounds from turmeric leaves (Curcuma domestica) using pressurised liquid extraction (PLE). LWT - Food Science and Technology, 42(1): 233 – 238.

- Official Method of Analysis, AOAC 17th edn. (2000). Moisture in Spices / I.S Specification No I.S 1797- 1985. Methods of Test for Spices and Condiments.

- Zaibunnisa, A. H. (2009). Inclusion complexes of turmeric oleoresin with β and γ-cyclodextrin. Ph.D. Thesis, Universiti Kebangsaan Malaysia.

- Gilda, S., Kanitkar, M., Bhonde, R. and Paradkar, A. (2010). Activity of water-soluble turmeric extract using hydrophilic excipients. LWT - Food Science and Technology, 43(1): 59 – 66.

- Wakte, P. S., Sachin, B. S., Patil, A. A., Mohato, D. M., Band, T. H. and Shinde, D. B. (2011). Optimization of microwave, ultra-sonic and supercritical carbon dioxide assisted extraction techniques for curcumin from Curcuma longa. Separation and Purification Technology, 79: 50 – 55.

- Li, S., Yuan, W., Deng, G., Wang, P., Yang, P. and Aggarwal, B. B. (2011). Chemical Composition and Product Quality Control of Turmeric (Curcuma longa L.). Pharmaceutical Crops, 2: 28 – 54.

- Allawzi, M., Al-Otoom, A., Allaboun, H., Ajlouni, A. and Nseirat, F. A. (2011). CO2 supercritical fluid extraction of Jordanian oil shale utilizing different co-solvents. Fuel Processing Technology, 92(10): 2016 – 2023.

- Herrero, M., Castro-Puyana, M., Mendiola, J. A. and Ibañez, E. (2013). Compressed fluids for the extraction of bioactive compounds. TrAC Trends in Analytical Chemistry, 43: 67 – 83.

- Sovilj, M. N., Nikolovski, B. G. and Spasojević, M. Đ. (2011). Critical review of SFE of selected spice plant materials. Macedonian Journal of Chemistry and Chemical Engineering, 30(2): 197 – 220.

- Singh, G., Singh, O. P. and Maurya, S. (2002). Chemical and biocidal investigations on essential oils of some Indian Curcuma species. Progress in Crystal Growth and Characterization of Materials, 45: 75 – 81.

- Gounder, D. K. and Lingamallu, J. (2012). Comparison of chemical composition and antioxidant potential of volatile oil from fresh, dried and cured turmeric (Curcuma longa) rhizomes. Industrial Crops and Products, 38: 124 – 131.