Malaysian Journal of Analytical Sciences Vol 20 No 3 (2016): 525 - 530

XYLOOLIGOSACCHARIDES PRODUCTION FROM OIL PALM FROND BY Trichoderma longibrachiatum XYLANASE

(Penghasilan Xilooligosakarida daripada Pelepah Kelapa Sawit Menggunakan Xilanase Trichoderma Longibrachiatum)

Sabiha Hanim Saleh1*, Siti Normah Mohd Damanhuri Shah1, Khalilah Abdul Khalil2, Aishah Bujang3

1School of Chemistry and Environment

2School of Biology

3School of Food Science and Technology

Faculty of Applied Sciences,

Universiti Teknologi MARA, 40450 Shah Alam, Selangor Malaysia

*Corresponding author: sabihahanim@salam.uitm.edu.my

Received: 24 February 2015; Accepted: 27 October 2015

Abstract

Oil palm fronds containing rich hemicellulose are low cost resources that could be potentially converted into valuable products such as xylooligosaccharides. The main objective of this study was to investigate the production of xylooligosaccharides from OPF hemicelluloses using Trichoderma longibrachiatum xylanase. The OPF hemicellulose extracted by alkaline extraction was hydrolysed by xylanase at pH 4.6, temperature 40 °C, hemicellulose substrate concentration 2 % (w/v) and enzyme concentration 2 U/ml for different period of time from 0 to 48 hours to produce xylooligosaccharides. The hydrolysate obtained from enzymatic hydrolysis was further purified through ultrafiltration using 10 kDa molecular weight cut off membranes. The highest total of xylobiose and xylotriose was found to be 21.91 mg/mL and obtained at 8 hours of hydrolysis time. After ultrafiltration step, xylooligosaccharides mixture were obtained in the permeate and retentate. The highest xylobiose (56.64g/100g) and xylotriose (45.80g/100g) were found in retentate and permeate, respectively.

Keywords:  xylooligosaccharides, oil palm frond, ultrafiltration, enzymatic hydrolysis, xylanase

Abstrak

Pelepah kelapa sawit  yang kaya dengan hemisellulosa adalah sumber kos rendah yang berpotensi untuk ditukarkan kepada produk bernilai seperti xilooligosakarida. Objektif utama kajian ini adalah untuk menyiasat penghasilan xilooligosakarida daripada hemisellulosa pelepah kelapa sawit menggunakan enzim  xilanase Trichoderma longibrachiatum. Hemisellulosa yang diekstrak daripada pelepah kelapa sawit menggunakan alkali, dihidrolisis menggunakan enzim xilanase pada pH 4.6, suhu 40 °C, kepekatan hemiselulose 2 % (w/v) dan kepekatan enzim 2U/ml umtuk tempoh masa berbeza dari 0 hingga 48 jam untuk menghasilkan xilooligosakarida. Hidrolisat yang diperoleh daripada hidrolisis enzimatik kemudiannya ditulenkan melalui ultrafiltrasi menggunakan membran yang berat molekulnya 10 kDa. Jumlah tertinggi xilobios dan xilotrios adalah 21.91 mg/mL yang diperoleh pada masa hidrolisis 8 jam. Setelah ultrafiltrasi, campuran xilooligosakarida diperoleh dalam ‘permeate’ dan ‘retentate’. Kandungan tertinggi xilobios (56.64g/100g) dan xilotrios (45.80g/100g) ditemui di dalam ‘retentate’ dan ‘permeate’.

Kata kunci: xilooligosakarida, pelepah kelapa sawit, ultrafiltrasi, hidrolisis enzimatik, xilanase

References

- Ofori-Boateng, C. and Lee, K.T. (2013). Response surface optimization of ultrasonic-assisted extraction of carotenoids from oil palm (Elaeis guineensis Jacq.) fronds. Food Science and Nutrition, 1 (3): 209 – 221.

- Vázquez, M. J., Alonso, J. L., Domínguez, H. and Parajó, P. C. (2000). Xylooligosaccharides: manufacture and application. Trends in Food Science and Technology, 11: 387 – 393.

- Akpinar, O., Ak, O., Kavas, A., Bakir, U. and Yilmaz, L. (2007). Enzymatic production of xylooligosaccharides from cotton stalks. Journal of Agricultural and Food Chemistry, 55: 5544 – 5551.

- Carvalho, A. F. A., Neto, P. O., Silva, D. F. and Pastore, G. M. (2013). Xylooligosaccharides from lignocellulosic materials: Chemical structure, health benefits and production by chemical and enzymatic hydrolysis. Food Research International, 51: 75 - 85.

- Sabiha-Hanim, S., Nor, M. A. M. and Rosma, A. (2011). Effect of autohydrolysis and enzymatic treatment on oil palm (Elaeis guineensis Jacq.) frond fibres for xylose and xylooligosaccharides production. Bioresource Technology, 102: 1234 – 1239.

- Royer, J. C. and Nakas, J. P. (1991). Purification and characterization of two xylanases from Trichoderma longibrachiatum. European Journal of Biochemistry, 202: 521 – 529.

- Chen, C. S., Chen, J. L. and Lin, T. Y. (1997). Purification and characterization of a xylanase from Trichoderma longibrachiatum for xylooligosaccharide production. Enzyme and Microbial Technology, 21: 91 – 96.

- Akpinar, O. and Bostanci, S. (2009). Xylooligosaccharide production from lignocellulosic wastes with Trichoderma longibrachiatum xylanase. Journal of Food, Agriculture & Environment, 7 (1): 70 – 74.

- Anis. M. (2000). Extraction and functional properties of oil palm biomass. Doctoral dissertation, Universiti Sains Malaysia, Penang, Malaysia

- Akpinar, O., Erdogan, K. and Bostanci, S. (2009). Enzymatic production of xylooligosaccharides from selected agricultural wastes. Food and Bioproducts Processing, 87: 145 – 151

- Dubois, M., Gillies, K. A., Hamilton, J. K., Rebers, P. A. and Smith, F. (1956). Colorimetric method for determination of sugars and related substances. Analytical Chemistry, 28(3): 350 – 357.

- Ruiz, H. A., Cerquira, M. A., Silva, H. D., Rodríguez-Jasso, R. M., Vicente, A. A. and Teixeira, J. A. (2013). Biorefinery volarization of autohydrolysis wheat straw hemicellulose to be applied in a polymer-blend film. Carbohydrate Polymers, 92: 2154 – 2162.

- Luo, Q., Peng, H., Zhou, M., Lin, D., Ruan, R., & Wan, Y., Zhang, J. and Liu Y. (2012). Alkali extraction and physicochemical characterization of hemicelluloses from young bamboo (Phyllostachys pubescens mazel). Bioresources, 7(4): 5817 – 5828.

- Bian, J., Peng, F., Peng, P., Xu, F. and Sun, R-C. (2010). Isolation and fractionation of hemicelluloses by graded ethanol precipitation from Caragana korshinskii. Carbohydrate Research, 345: 802 – 809.

- Fazilah, A., Azemi, M. N. M., Karim, A. A., and  Norakma, M. N. (2009). Physicochemical properties of  hydrothermally treated hemicellulose from oil palm frond. Journal of Agricultural and Food Chemistry, 57: 1527 – 1531.

- Akpinar, O., Gunay, K., Yilmaz, Y., levent, O. and Bostanci, S. (2010). Enzymatic process and antioxidant activity of agricultural waste autohydrolysis liquors. Bioresources, 5(2): 699 – 711.

- Gullón, P., González-Muñoz, M. J., Domínguez, H. and Parajó, J. C. (2008). Membrane processing of liquors from Eucalyptus globulus autohydrolysis. Journal of Food Engineering, 87: 257 – 265.