# Malaysian Journal of Analytical Sciences Vol 20 No 6 (2016): 1318 - 1328

# DETERMINATION OF TOTAL ORGANIC CARBON CONCENTRATION

# IN SURFICIAL SEDIMENTS OF SUNGAI PINANG, PENANG, MALAYSIA

# (Penentuan Jumlah Karbon Organik dalam Sedimen Permukaan  di Sungai Pinang,

# Pulau Pinang, Malaysia)

# Ong Meng Chuan*, Fok Fei Mei, Yong Jaw Chuen

# School of Marine and Environmental Sciences

# Universiti Malaysia Terengganu, 21030 Kuala Terengganu, Terengganu, Malaysia

*Corresponding author: ong@umt.edu.my

Received: 17 August 2015; Accepted: 11 October 2016

Abstract

# The concentration of total organic carbon (TOC) and mean size sediment in a total of 24 samples of surficial sediment were determined. From the results of study, the concentration of TOC (5.33%) was found high nearby the housing area at Sungai Pinang. At the downstream of the river tend to have higher concentration of TOC (1.05% - 5.33 %) while at the upstream tend to have low degree of TOC in surficial sediments (0.36% - 0.78%). The major contribution in TOC contamination at the bottom part of the river maybe due to anthropogenic activities such as waste effluent from urban runoff, industrial activities and boat activities. Apart from that, the mean grain size was probably an important factor that influences the TOC content in surficial sediment at Sungai Pinang. This is because the distribution of the mean grain size that had similar trend to TOC. In addition, the high correlation relationship between TOC and mean grain size were observed and it had accounted for r = 0.67 owing to fine particles of sediment which had provided a large binding capacity for the sorption of organic carbon. Moreover, fine sediment probably indicates slow hydrodynamic energy at the downstream. It might affect the chemical sorption process and allow the particulate organic carbon to accumulate at the bottom of the water column. Thus, fine sediment maybe playing dominant role in the content of TOC in surficial sediment at Sungai Pinang.

# Keywords:  total organic carbon, mean size sediment, surficial sediments, Sungai Pinang

# Abstrak

# Kandungan jumlah karbon organik (TOC) dan purata saiz sedimen di dalam 24 sampel sedimen permukaan telah ditentukan. Dari hasil kajian ini, kepekatan TOC (5.33%) didapati tinggi berhampiran dengan kawasan perumahan di Sungai Pinang. Di kawasan hilir sungai tersebut mempunyai kepekatan TOC yang lebih tinggi (1.05% - 5.33%) manakala di kawasan hulu sungai mempunyai kepekatan yang rendah dalam permukaan sedimen (0.36% - 0.78%). Kemungkinan punca utama untuk pencemaran TOC di bahagian hilir sungai adalah dari aktiviti antropogenik seperti sisa buangan daripada bandar, aktiviti perindustrian dan aktiviti bot. Selain itu, purata saiz partikel kemungkinan adalah faktor penting menpergaruhi kandungan TOC dalam sedimen permukaan di Sungai Pinang. Ini kerana taburan purata saiz partikel mempunyai corak yang sama dengan TOC. Tambahan pula, hubungan korelasi antara TOC dan purata saiz partikel didapati tinggi dalam sedimen permukaan, iaitu r = 0.67 disebabkan saiz partikel yang halus memperolehi kapasiti pengikatan yang luas untuk penyerapan karbon organik. Selain itu, sedimen yang halus berkemungkinan menunjukkan tenaga hidrodinamik yang perlahan di bahagian hiliran. Hal ini berkemungkinan mempengaruhi proses penyerapan dan pengumpulan zarah karbon organik di bahagian bawah turus air. Oleh itu, sedimen yang halus berkemungkinan besar memainkan peranan yang paling dominan kepada kandungan TOC yang dalam sedimen permukaan di Sungai Pinang.

# Kata kunci:  jumlah karbon organik, purata saiz sedimen, sedimen permukaan, Sungai Pinang

References

- Lazăr, L., Gomoiu, M.T., Boicenco, L. and Vasili, D. (2012). Total organic carbon (TOC) of the surface layer sediments covering the seafloor of the Romanian black sea. Geo-Eco-Marina, 18: 121 – 132.

- Hossain, G. M. and Bhuiyan, M. A. H. (2016). Spatial and temporal variations of organic matter contents and potential sediment nutrient index in the sundarbans mangrove forest, Bangladesh. KSCE Journal of Civil Engineering, 20 (1): 163 – 174.

- Anawar, H. M., Yoshioka, T., Konohira, E., Akai, J., Freitas, M. C. and Tareq, S. M. (2010). Sources of organic carbon and depositional environment in the Bengal delta plain sediments during the holocene period. Limnology, 11 (2): 133 – 142.

- Manju, M. N., Resmi, P., Kumar, Ratheesh, C. S., Gireeshkumar, T. R., Chandramohanakumar, N. and Joseph, M. M., (2016). Biochemical and stable carbon isotope records of mangrove derived organic matter in the sediment cores. Environmental Earth Sciences, 75: 565 – 580.

- Naji, A., Ismail, A. and Ismail, A. R. (2010). Chemical speciation and contamination assessment of Zn and Cd by sequential extraction in surface sediment of Klang River, Malaysia. Microchemical Journal, 95 (2): 285 – 292.

- Kwaansa-Ansah, E. E., Voegborlo, R. B., Adimado, A. A., Ephraim, J. H. and Nriagu, J. O. (2012). Effect of pH, sulphate concentration and total organic carbon on mercury accumulation in sediments in the volta lake at yeji, ghana, Bulletin of Environmental Contamination and Toxicology, 88 (3): 418 – 421.

- Shoham-Frider, E., Azran, S. and Kress, N. (2012). Mercury speciation and total organic carbon in marine sediments along the Mediterranean coast of Israel. Bulletin of Environmental Contamination and Toxicology, 63 (4): 495 – 502.

- Hall Jr, L. W. and Anderson, R. D. (2014). Relationship of bifenthrin sediment concentrations to grain size and total organic carbon in California waterbodies: Implications for ecological risk. Bulletin of Environmental Contamination and Toxicology, 93 (6): 764 – 768.

- Zhao, C., Liu, S., Dong, S., Isange, S., Liu, Q., An, N. and Li, X. (2015). Spatial and seasonal dynamics of organic carbon in physically fractioned sediments associated with dam construction in the middle Lancang-Mekong River. Journal of Soils and Sediments, 15 (11): 2323 – 2333.

- Mei, J., Li, Z., Sun, L., Gui, H. and Wang, X. (2011). Assessment of heavy metals in the urban river sediments in Suzhou City, Northern Anhui Province, China. Procedia Environmental Sciences, 10: 2547 – 2553.

- Daud, H. (2008). The water quality monitoring system/ program & pollution control. Retrieved from Water Environment Partnership in Asia Web Site: http://www.wepa-db.net/pdf/0810malaysia/f.pdf

- Usali, N. and Ismail, M. H. (2010). Use of remote sensing and GIS in monitoring water quality. Journal of Sustainable Development, 3 (3): 228 – 238.

- Saad, F. N., Rahman, N. N., Kadir, M. O. and Omar, F. M. (2008). Identification of pollution sources within the Sungai Pinang River Basin. Proceeding of the Malaysian Research Group International Conference, Manchester, United Kingdom: pp. 478 – 485.

- Dinakaran, J. and Krishnayya, N. S. R. (2011). Variations in total organic carbon and grain size distribution in ephemeral river sediments in Western India. International Journal of Sediment Research, 26 (2): 239 – 246.

- Department of Environment (2012). Malaysia Environment Quality Report. Department of Environment Malaysia: pp. 112.

- Meng, J., Yao, P., Yu, Z., Bianchi, T. S., Zhao, B., Pan, H. and Li, D. (2014). Speciation, bioavailability and preservation of phosphorus in surface sediments of the Changjiang estuary and adjacent East China Sea inner shelf. Estuarine, Coastal and Shelf Science, 144: 27 – 38.

- Maznah, W. O. and Mansor, M. (2002). Aquatic pollution assessment based on attached diatom communities in the Pinang River basin, Malaysia. Hydrobiologia, 481(1): 229 – 241.

- Misnan, N. and Rindam, M. (2012). Morfometri lembangan sungai-sungai utama di Pulau Pinang. Malaysia Journal of Society and Space, 8(3): 71 – 81.

- Ong, M., Menier, D., Shazili, N. and Kamaruzzaman, B. (2013). Geochemical characteristics of heavy metals concentration in sediments of Quiberon Bay waters, South Brittany, France. Oriental Journal of Chemistry, 29 (1): 39 – 45.

- Sany, S. B., Salleh, A., Rezayi, M., Saadati, N., Narimany, L. and Tehrani, G. M. (2013). Distribution and contamination of heavy metal in the coastal sediments of Port Klang, Selangor, Malaysia. Water, Air, and Soil Pollution, 224 (4): 1 – 18.

- Tung J. W. T. and Tanner, P. A. (2003). Instrumental determination of organic carbon in marine sediment. Marine Chemistry, 80 (2-3): 161 – 170.

- Folk, R. L. (1980). Petrology of sedimentary rocks Austin: University of Texas Libraries

- Ong, M. C., Kamaruzzaman, B. Y., Shazili, N. A. and  Rosnan, Y. (2006). Distribution and normalization of lead, copper and zinc in Terengganu  , Malaysia. Journal Sustainability Science and Management, 1: 74 – 84.

- Plumb, R. H. (1981). Procedures for handling and chemical analysis of sediment and water samples. Retrieved from United State Environmental Protection Agency: http://yosemite.epa.gov/r10/CLEANUP.NSF/ph/ T4+Technical+Documents/$F

- Özkan, E. Y. (2012). A New assessment of heavy metal contaminations in an eutrophicated bay (Inner Izmir Bay, Turkey). Turkish Journal of Fisheries and Aquatic Sciences, 12: 135-147.

- Swarnalatha, K., Letha, J. and Ayoob, S. (2013). An investigation into the heavy metal burden of Akkulam–Veli Lake in South India. Environmental Earth Sciences, 68 (3): 795 – 806.

- Ali, S. A. M., Payus, C. and Ali, M. M. (2015). Surface sediment analysis on petroleum hydrocarbon and total organic carbon from coastal area of Papar to Tuaran, Sabah. Malaysian Journal of Analytical Sciences, 19 (2): 318 – 324.

- Okoro, H. K., Fatoki, O. S., Adekola, F. A., Ximba, B. J. and Snyman, R. G. (2016). Spatio-temporal variation of organotin compounds in seawater and sediments from Cape Town Harbour, South Africa using gas chromatography with flame photometric detector (GC-FPD). Arabian Journal of Chemistry, 9 (1): 95 – 104.

- Mulligan, C., Fukue, M. and Sato, Y. (2010). Sediment contamination and sustainable remediation. London: CRC Press.

- Li, G., Cao, Z., Lan, D., Xu, J., Wang, S. and Yin, W. (2007). Spatial variations in grain size distribution and selected metal contents in the Xiamen Bay, China. Environmetal Geology, 52 (8): 1559 – 1567.

- Naji, A. and Ismail, A. (2011). Assessment of metals contamination in Klang River surface sediments by using different indexes. EnvironmentAsia, 4(1): 30 – 38.

- Adnan, N. H.,  Zakaria, M. P., Juahir, H. and Ali, M. M. (2012). Faecal sterols as sewage markers in the Langat River, Malaysia: Integration of biomarker and multivariate statistical approaches. Journal of Environmental Sciences, 24(9): 1600 – 1608.

- Zhao, S., Feng, C., Wang, D., Liu, Y. and Shen, Z. (2013). Salinity increases the mobility of Cd, Cu, Mn, and Pb in the sediments of Yangtze Estuary: Relative role of sediments’ properties and metal speciation. Chemosphere, 91 (7): 977 – 984.

- Hu, B., Cui, R., Li, J., Wei, H., Zhao, J., Bai, F., Song, W. and Ding, X. (2013). Occurrence and distribution of heavy metals in surface sediments of the Changhua River estuary and adjacent shelf (Hainan Island). Marine Pollution Bulletin, 76 (1-2): 400 – 405.

- Wang, F., Wang, W. X. and Huang, X. P. (2012). Spatial distribution of gut juice extractable Cu, Pb and Zn in sediments from the Pearl River Estuary, Southern China. Marine Environmental Research, 77: 112 – 119.

- Relić, D., Đorđević, D. and Popović, A. (2011). Assessment of the pseudo total metal content in alluvial sediments from Danube River, Serbia. Environmental Earth Science, 63(6): 1303 – 1317.

- Chen, L. G., Huang, Y. M., Peng, X. C., Xu, Z., Zhang, S., Ren, M., Ye, Z. and Wang, X. (2009). PBDEs in sediments of the Beijiang River, China: Levels, distribution, and influence of total organic carbon. Chemosphere, 76 (2): 226 – 231.

- Wentworth C. K. (1922). A scale of grade and class terms for clastic sediments. Journal of Geology, 30: 377 – 392.

- Satpathy, K. K., Mohanty, A. K., Prasad, M. V., Natesan, U. and Sarkar, S. K. (2012). Studies on the variations of heavy metals in the marine sediments off Kalpakkam, East Coast of India. Environmental Earth Science, 65 (1): 89 –101.

- Kamaruzzaman, Y. and Ong., M. C., (2009). Geochemical proxy of some chemical elements in sediments of Kemaman River estuary, Terengganu, Malaysia. Sains Malaysiana, 38 (5): 631 – 636.

- Idriss, A. A. and Ahmad, A. K. (2013). Heavy metals nickel and chromiumin sediments in the Juru River, Penang, Malaysia. Journal of Environmental Protection, 7: 1245 – 1253.

- Huang, K. M. and Lin, S. (2003). Consequences and implication of heavy metal spatial variations in sediments of the Keelung River Drainage basin, Taiwan. Chemosphere, 53 (9): 1113 – 1121.