# Malaysian Journal of Analytical Sciences Vol 21 No 1 (2017): 182 - 187

# TREATMENT OF METHYLENE BLUE IN WASTEWATER USING

# Scirpus grossus

# (Rawatan Metilena Biru dalam Air Sisa Menggunakan Scirpus grossus)

# Enas Abdulqader Saeed Almaamary1, Siti Rozaimah Sheikh Abdullah1*, Hassimi Abu Hasan1,

# Reehan Adne Ab. Rahim1, Mushrifah Idris2

1Department of Chemical and Process Engineering, Faculty of Engineering and Built Environment

2Tasik Chini Research Center, Faculty of Science and Technology

Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

*Corresponding author: rozaimah@ukm.edu.my

Received: 21 October 2015; Accepted: 14 June 2016

Abstract

# Phytoremediation is an emerging technology that should be considered for the remediation of contaminated sites because of its aesthetic advantages and long-term applicability. The possibility of Scirpus grossus for degradation of a basic dye, methylene blue (MB) was investigated. The effect of the operational parameter of different dye concentrations (0, 200, 400, 600, 800 and 1000 mg/L) was determined, and the water quality parameters namely pH, dissolved oxygen (DO), biochemical oxygen demand (BOD), chemical oxygen demand (COD) and total organic carbon (TOC) were monitored. The UV-Visible absorption confirmed the degradation of MB within 72 days. The removal efficiency of methylene blue dye from synthetic wastewater was determined to be in the range of 86 – 38% for all treatments at different concentrations (200 – 1000 mg/L) respectively. Furthermore, the highest removals for BOD, COD in 400 mg/L and TOC in 200 mg/L MB were 69, 58 and 63% respectively.

# Keywords:  phytoremediation, Scirpus grossus, methylene blue, decolourisation, water quality

# Abstrak

# Fitopemulihan merupakan teknologi baru yang perlu dipertimbangkan untuk pemulihan tapak tercemar kerana kelebihan estetik dan kebolehgunaan bagi jangka panjang. Kemungkinan Scirpus grossus untuk degradasi pewarna asas, metilena biru (MB) telah dikaji. Kesan parameter operasiiaitu kepekatan pewarna yang berbeza (0, 200, 400, 600, 800 dan 1000 mg/L) ditentukan dan parameter kualiti air iaitu pH, oksigen terlarut (DO), permintaan oksigen biokimia (BOD), permintaan oksigen kimia (COD) dan jumlah karbon organic (TOC) dipantau. Penyerapan UV cahaya nampak mengesahkan degradasi MB dalam masa 72 hari. Kecekapan penyingkiran pewarna metilena biru daripada air sisa sintetik telah ditentukan dalam lingkungan 38 – 86% untuk semua rawatan dalam kepekatan yang berbeza (200 – 1000 mg/L) masing-masing. Tambahan pula, penyingkiran tertinggi bagi BOD, COD dalam 400 mg/L dan TOC dalam 200 mg/L MB masing-masing adalah 69, 58 dan 63%.

# Kata kunci:  pemulihan-fito, Scirpus grossus, metilena biru, penyahwarnaan warna, kualiti air

References

- Kadirvelu, K., Kavipriya, M., Karthika, C., Radhika, M., Vennilamani, N. and Pattabhi, S. (2003). Utilization of various agricultural wastes for activated carbon preparation and application for the removal of dyes and metal ions from aqueous solutions. Bioresource Technology, 87(1): 129 – 132.

- Kabra, A. N., Khandare, R. V, Waghmode, T. R. and Govindwar, S. P. (2012). Phytoremediation of textile effluent and mixture of structurally different dyes by Glandularia pulchella (Sweet) Tronc. Chemosphere, 87(3): 265 – 272.

- Kagalkar, A. N., Jagtap, U. B., Jadhav, J. P., Govindwar, S. P. and Bapat, V. (2010). Studies on phytoremediation potentiality of Typhonium flagelliforme for the degradation of Brilliant Blue R. Planta, 232(1): 271 – 285.

- Bulc, T. G. and Ojstrsek, A. (2008). The use of constructed wetland for dye-rich textile wastewater treatment. Journal of Hazardous Materials, 155(1–2): 76 – 82.

- Khataee, A. R., Movafeghi, A., Torbati, S., Salehi Lisar, S. Y. and Zarei, M. (2012). Phytoremediation potential of duckweed (Lemna minor L.) in degradation of C.I. Acid Blue 92: artificial neural network modeling. Ecotoxicology and Environmental Safety, 80: 291 – 298.

- Aubert, S. and Schwitzguébel, J. P. (2004). Screening of plant species for the phytotreatment of wastewater containing sulphonated anthraquinones. Water Research, 38(16): 3569 – 3575.

- Kalme, S. D., Parshetti, G. K., Jadhav, S. U. and Govindwar, S. P. (2007). Biodegradation of benzidine based dye Direct Blue-6 by Pseudomonas desmolyticum NCIM 2112. Bioresource Technology, 98(7): 1405 – 1410.

- Khandare, R. V, Kabra, A. N., Tamboli, D. P. and Govindwar, S. P. (2011). The role of Aster amellus Linn. in the degradation of a sulfonated azo dye Remazol Red: a phytoremediation strategy. Chemosphere, 82(8): 1147 – 1154.

- Kalyani, D. C., Telke, A. A., Dhanve, R. S. and Jadhav, J. P. (2009). Ecofriendly biodegradation and detoxification of Reactive Red 2 textile dye by newly isolated Pseudomonas sp. SUK1. Journal of Hazardous Materials, 163(2–3): 735 – 742.

- Shedbalkar, U., Dhanve, R. and Jadhav, J. (2008). Biodegradation of triphenylmethane dye cotton blue by Penicillium ochrochloron MTCC 517. Journal of Hazardous Materials, 157(2–3): 472 – 479.

- Patil, A. V. and Jadhav, J. P. (2013). Evaluation of phytoremediation potential of Tagetes patula L. for the degradation of textile dye Reactive Blue 160 and assessment of the toxicity of degraded metabolites by cytogenotoxicity. Chemosphere, 92(2): 225 – 232.

- Carliell, C. M., Barclay, S. J. Naidoo, N. Buckley, C.A., Mulholland, D. A. and Senior, E. (1994). Anaerobic decolourisation of reactive dyes in conventional sewage treatment processes. Water SA, 20: 341 – 344.

- Reema R. M., Saravanan P., Dharmendira K. M. and Renganathan, S. (2011). Accumulation of methylene blue dye by growing Lemna minor. Separation Science and Technology, 46: 1052 – 1058.

- Suteu, D. Zaharia, C. Muresan, A. Muresan, R. and Popescu, A. (2009). Using of industrial waste materials for textile wastewater treatment. Environmental Engineering and Management Journal, 8(5): 1097 – 1102.

- Mohan, S., V. Rao, N. C. and Karthikeyan, J. (2002). Adsorptive removal of direct azo dye from aqueous phase onto coal based sorbents: A kinetic and mechanistic study. Journal of Hazardous Materials, 90(2): 189 – 204.