Malaysian Journal of Analytical Sciences Vol 21 No 1 (2017): 213 - 220

THE EFFECT OF GLUCOSE ADDITION IN ACETONE-BUTANOL-ETHANOL FERMENTATION FROM PALM OIL MILL EFFLUENT

BY Clostridium Acetobutylicum NCIMB 619

(Kesan Penambahan Glukosa dalam Fermentasi Aseton-Butanol-Etanol daripada Efluen Kilang Kelapa Sawit oleh Clostridium Acetobutylicum NCIMB 619)

Azima Syafaini Japar1*, Mohd. Sobri Takriff2, Jamaliah Md. Jahim1, Abdul Amir Hassan Kadhum1

1Department of Chemical and Process Engineering

2 Research Center for Sustainable Process Technology

Faculty of Engineering and Built Environment,

Universiti Kebangsaan Malaysia, 43600, UKM Bangi, Selangor, Malaysia

*Corresponding author: azima.syafaini@siswa.ukm.edu.my

Received: 21 October 2015; Accepted: 14 June 2016

Abstract

This study investigates the production of acetone, butanol and ethanol (ABE) in sterilized sludge of palm oil mill effluent (POME) with the addition of glucose by Clostridium acetobutylicum NCIMB 619. Glucose concentrations of 5 g/L, 10 g/L and 15 g/L were used with volume percentage of 3% (v/v) and 5% (v/v) for each concentration. The highest ABE production of 9.89 g/L was observed with the addition of 10 g/L glucose with volume percentage of 5% (v/v) compared to 0.49 g/L in the control batch, with highest ABE productivity and yield, YP/S of 0.14 g/L/h and 1.36 g ABE/g substrate, respectively. Providing culture with 15 g/L glucose with volume percentage of 5% (v/v) was found to reduce the ABE production with lower productivity and ABE yield. The lower production at higher sugar level might be due to sugar inhibition that occurred during the process.

Keywords:  acetone-butanol-ethanol fermentation, palm oil mill effluent, Clostridium acetobutylicum, glucose addition

Abstrak

Kajian ini dilakukan bagi menentukan kesan penamabahan glukosa terhadap penghasilan aseton, butanol dan etanol (ABE) di dalam medium mendakan efluen kilang kelapa sawit (POME) steril oleh Clostridium acetobutylicum NCIMB 619. Kepekatan glukosa yang digunakan adalah 5 g/L, 10 g/L dan 15 g/L dengan peratus isipadu sebanyak 3% (v/v) dan 5% (v/v) bagi setiap kepekatan. Penghasilan ABE yang paling tinggi iaitu sebanyak 9.89 g/L diperoleh pada penambahan 10 g/L glukosa dengan peratus isipadu 5% (v/v) berbanding 0.49 g/L ABE yang diperoleh di dalam medium kawalan. Produktiviti dan hasil ABE yang diperoleh juga adalah yang tertinggi iaitu masing-masing sebanyak 0.14 g/L/j dan 1.36 g ABE/g substrat. Penambahan 15 g/L glukosa dengan peratus isipadu sebanyak 5% (v/v) didapati mengurangkan penghasilan ABE dengan produktiviti dan hasil ABE yang paling rendah disebabkan oleh perencatan gula yang berlaku semasa proses fermentasi.

Kata kunci:  	fermentasi aseton-butanol-etanol, efluen kilang kelapa sawit, clostridium acetobutylicum, penambahan glukosa

References

- Jones, D. T. and Woods, D. R. (1986). Acetone-butanol fermentation revisited. Microbiology and Molecular Biology Reviews, 50 (4): 484 – 524.

- Wyman, C. E. (2007). What is (and is not) vital to advancing cellulosic ethanol. Trends Biotechnology, 25: 153 – 157.

- Lynd, L. R., Laser, M. S., Bransby, D., Dale, B. E., Davison, B. and Hamilton, R. (2008). How biotech can transform biofuels. Nature Biotechnology, 26: 169 – 172.

- Rajeev, K. S., Lalitha, D. G., Kuniparambil, R., Deepthy, A. and Ashok, P. (2011). Butanol fuel from biomass: revisiting ABE fermentation. Biofuels: Alternative feedstocks and conversion processes:  pp. 571 – 586.

- Ugoji, E. (1997). Anaerobic digestion of palm oil mill effluent and its utilization as fertilizer for environmental protection. Renewable Energy, 10: 291 – 294.

- Somrutai, W., Takagi, M. and Yoshida, T. (1996). Acetone-butanol fermentation by Clostridium aurantibutyricum ATCC 17777 from a model medium for palm oil mill effluent. Journal of Fermentation and Bioengineering, 81: 543 – 547.

- Kalil, M. S., Pang, W. K., Wan Yusoff, W. M., Sadazo, Y. and Abdul Rahman, R. (2003). Direct fermentation of palm oil mill effluent to acetone-butanol-ethanol by solvent producing Clostridia. Pakistan Journal of Biological Sciences, 6: 1273 – 1275.

- Parveen, F. R., Rajeev, P. S., Hakimi Ibrahim, M. and Esa, N. (2010). Review of current palm oil mill effluent (POME) treatment methods: Vermicomposting as a sustainable practice. World Applied Sciences Journal, 10 (10): 1190 – 1201.

- Ma, A. N. (1999). Treatment of palm oil mill effluent. In. Oil palm and the environment: A Malaysian perspective. Kuala Lumpur: Malaysia Oil Palm Grower’s Council: 113 – 126.

- Ahmad, A. L., Ismail, S. and Bhatia, S. (2003). Water recycling from palm oil mill effluent (POME) using membrane technology. Desalination, 157: 87 – 95.

- Sethupathi, S. (2004). Removal of residues oil from palm oil mill effluent (POME) using chitosan, Thesis Universiti Sains Malaysia.

- Azima, S. J., Mohd Sobri, T., Jamaliah, M. J. and Abd Amir, H. K. (2014). The potential of supernatant and sludge of POME in ABE fermentation by Clostridium acetobutylicum. Australian Journal of Basic and Applied Sciences 8(19): 44 – 46.

- Japar, A.S., Takriff, M. S., Jahim, J. M. and Kadhum, A. A. H. (2013). Acetone-butanol-ethanol fermentation from palm oil mill effluent using Clostridium acetobutylicum. Developments in Sustainable Chemical and Bioprocess. New York: Springer Science & Business Media: pp. 35 – 41.

- Yusof, S. J. H. M. (2010). Fermentasi aseton-butanol-etanol (ABE) dan perolehan ABE daripada kaldu fermentasi. Thesis Master. Universiti Kebangsaan Malaysia.

- Masngut, N. (2007). Aplikasi reaktor aliran berayun bagi penghasilan aseton-butanol-etanol oleh Clostridium acetobutylicum NCIMB 13357. Thesis Master. Universiti Kebangsaan Malaysia.

- Long, S., Jones, D. T. and Woods, D. R. (1984). The relationship between sporulation and solvent production in Clostridium acetobutylicum P262. Biotechnology Letters, 6: 529 – 534.

- Monot, F., Engasser, J. M. and Petitdemange, H. (1983). Regulation of acetone butanol production in batch and continuous cultures of Clostridium acetobutylicum. Biotechnology Symposium, 13: 207 – 216.

- Bahl, H., Andersch, W., Braun, K. and Gottschalk, G. (1982). Effect of pH and butyrate concentration on the production of acetone and butanol by Clostridium acetobutylicum grown in continuous culture. European Journal of Applied Microbiology and Biotechnology, 14: 17 – 20.

- Haggstrom, L. and Enfors, S. O. (1982). Continuous production of butanol with immobilized cells of Clostridium acetobutylicum. Applied Biochemistry and Biotechnology, 7: 35 – 37.

- Rogers, P. (1984). Genetics and biochemistry of Clostridium relevant to development of fermentation processes. Applied Microbiology, 31: 1 – 60.

- Malaysian Palm Oil Board, MPOB. (2012). Oil palm and the environment. www.mpob.com.my/en/palm-info/environment/520-achievements [20 Mac 2013].

- Miller, G. L. (1959). Use of dinitrosalicylic acid reagent for determination of reducing sugar. Analytical Chemistry 31: 426 – 429.

- Boudreau, T. M. and Gordon, A. H. (2006). Improved ethanol-water separation using fatty acids. Process Biochemistry, 41: 980 – 983.

- Yaacob, N. S. (2003). Performance of solvent (acetone-butanol-ethanol) fermentation by Clostridium saccharobutylicum strain P262 and NCIMB 8052 using free and immobilized cells system. Thesis Master. Universiti Putra Malaysia.

- Lee, S. Y., Park, J. H., Jang, S. H., Nielsen, L. K., Kim, J. and Jung, K. S. (2008). Fermentative butanol production by clostridia. Biotechnology and Bioengineering, 101 (2): 209 – 228.

- Qureshi, N., Cotta, M. A. and Saha, B. C. (2013). Bioconversion of barley straw and corn stover to butanol (a biofuel) in integrated fermentation and simultaneous product recovery bioreactors. Food and Bioproducts Processing, 92(3): 298 – 308.

- Madihah, M. S., Sahaid, K. M., Suraini, A. A. and Karim, M. I. A. (2001). Direct fermentation of gelatinized sago starch to acetone-butanol-ethanol by Clostridium acetobutylicum. World Journal of Microbiology & Biotechnology, 17: 567 – 576.

- Gao, K and Rehmann, L. (2014). ABE fermentation from enzymatic hydrolysate of NAOH-pretreated corncobs. Biomass and Bioenergy, 66: 110 – 115.

- Qureshi, N., Saha, B. C., Hector, R. E. and Cotta, M. A. (2008). Removal of fermentation inhibitors from alkaline peroxide pretreated and enzymatically hydrolyzed wheat straw: Production of butanol from hydrolysate using Clostridium beijerinckii in batch reactors. Biomass and Bioenergy, 32: 1353 –1358.

- Abd-Alla, M. H and El-Enany, A. E. (2012). Production of acetone-butanol-ethanol from spoilage date palm (Phoenix dactylifera L.) fruits by mixed culture of Clostridium acetobutylicum and Bacillus subtilis. Biomass and Bioenergy, 42: 172 – 178.

- Liu, Z., Ying, Y., Li., F., Ma, C. and Xu, P. (2010). Butanol production by Clostridium beijerinckii ATCC 55025 from wheat bran. Journal of Industrial Microbiology & Biotechnology, 37 (5): 495 – 501.