# Malaysian Journal of Analytical Sciences Vol 21 No 2 (2017): 381 - 390

# FABRICATION OF PES/MnO MIXED MATRIX MEMBRANES FOR CADMIUM REMOVAL

# (Pembuatan PES/MnO Membran Campuran Matrik dalam Penyingkiran Kadmium)

# Soon Onn Lai1, Kok Chung Chong1, Zhi Wei Kerk1, Boon Seng Ooi2*,Woei Jye Lau3

# 1Lee Kong Chian Faculty of Engineering and Science,

# Universiti Tunku Abdul Rahman, Jalan Sungai Long, Bandar Sungai Long, 43300 Kajang, Malaysia

# 2School of Chemical Engineering, Engineering Campus,

# University Sains Malaysia, Seri Ampangan, 14300 Nibong Tebal, Pulau Pinang, Malaysia.

# 3Advanced Membrane Technology Research Centre (AMTEC),

# Universiti Teknologi Malaysia, 81310 Skudai, Johor, Malaysia

*Corresponding author: chobs@usm.my

Received: 26 August 2016; Accepted: 8 January 2017

Abstract

# With increasing industrial activities around the globe, water source had been severely polluted by various heavy metal substances that lead to adverse effect not only to the environment but also to human being. The content of heavy metal in wastewater especially cadmium was increasing in the recent years and can be attributed to the strong demand of cadmium in the battery and electroplating industry. In this study, the potential of novel nanocomposite mixed matrix membrane incorporated with manganese oxide (MnO2) nanoparticle for cadmium removal was investigated. The membranes were fabricated using polyethersulfone (PES) by phase inversion technique with different manganese oxide nanoparticle loading. Prior to performance evaluation, the membranes were characterized by their structural morphology, surface chemistry and cadmium adsorption capability. The results suggested that PES membrane incorporated with 2.0 ratio of MnO2/PES exhibited the best result in both cadmium adsorption (51.7 mg/g) and water permeability (1462 L/m2.h.bar). This indicated the prospect of this membrane to be applied in wastewater treatment industry.

# Keywords:  mixed matrix membrane, polyethersulfone, cadmium, wastewater

# Abstrak

# Sumber air di seluruh dunia telah teruk tercemar dengan pelbagai bahan logam berat yang akan memberi kesan buruk kepada alam sekitar dan manusia. Pencemaran air ini adalah akibat daripada aktiviti-aktiviti perindustrian yang semakin pesat di dunia kita. Kandungan logam berat dalam air sisa terutamanya kadmium semakin meningkat dalam beberapa tahun ini disebabkan oleh permintaan kadmium yang tinggi terutamanya dalam industri bateri dan elektropenyaduran. Dalam kajian ini, potensi membran campuran matrik yang baharu dengan mangan oksida (MnO2) dikajikan dalam sistem turas aliran lintang untuk penyingkiran kadmium. Membran ini dibuat daripada polietersulfona (PES) dengan teknik fasa berbalik dengan pelbagai berat zarah nano mangan oksida. Sebelum kajian dijalankan, membran dicirikan dengan kajian struktur dan permukaan morfologi, keliangan, sudut kenalan permukaan, kajian pembenaman MnO2 dan kajian penjerapan kadmium. Hasil kerja kajian ini mencadangkan bahawa membran PES dengan nisbah 2.0 MnO2 menunjukkan keputusan yang memberangsangkan dalam penjerapan kadmium (51.7 mg/g) dan fluks air tulen yang paling tinggi (1,462 L/m2.h.bar). Keputusan kajian ini telah menunjukkan membran mempunyai potensi yang besar digunakan dalam industri rawatan air sisa.

# Kata kunci:  membran matriks campuran, polietersulfon, kadmium,  air sisa

References

- Mehdi, G. and Hamid, R. M. (2015). Enhancing performance of hybrid liquid membrane process supported by porous anionic exchange membranes for removal of cadmium from wastewater. Chemical Engineering Journal, 264(1): 241 – 250.

- Tingting, D., Liangrong, Y., Menghao, Z., Zhini, L., Xitong, S., Jiemiao, Y. and Huizhou L. (2015). Removal of cadmium (II) from wastewater with gas-assisted magnetic separation. Chemical Engineering Journal, 280(1): 426 – 432.

- Yilmaz, Y. (2016). Removal of heavy metals in wastewater by using zeolite nano-particles impregnated polysulfonemembranes. Journal of Hazardous Materials, 309(1): 53 – 64.

- Chong, K. C., Lai, S. O., Lee, K. M., Lau, W. J., Ismail, A. F. and Ooi, B. S. (2014). Characteristic and performance of polyvinylidene fluoride membranes blended with different polymeric additives in direct contact membrane distillation. Desalination and Water Treatment, 2014 (1): 1 – 9.

- Mocha, N. M., Lau, W. J., Ng, B. C., Ismail, A. F. and Veerasamy, D. (2015). Preparation and characterization of PVDF membranes incorporated with different additives for dyeing solution treatment using membrane distillation. Desalination and Water Treatment, 56(1): 1999 – 2012.

- Bourawi, M., Ding, Z. and Khayet, M. (2006). A framework for better understanding membrane distillation process. Journal of Membrane Science, 285(1): 4 – 29.

- Subramanian, V., Zhu, H. and Wei, B. (2008). Alcohol-assisted room temperature synthesis of different nanostructured manganese oxides and their pseudocapacitance properties in neutral electrolyte. Chemical Physics Letters, 453(4-6): 242 – 249.

- Ahmad, A. L., Ideris, N., Ooi, B. S., Low S. C. and Ismail, A. (2012). Synthesis of polyvinlidene fluoride (PVDF) membranes for protein binding: Effect of casting thickness. Journal of Applied Polymer Science, 128(5): 3438 – 3445.

- Chong, K .C., Lai, S. O., Lau, W. J. and Ooi, B.S. (2014). Preparation, characterization and performance of polyvinylidene fluoride membrane for sodium chloride rejection in direct contact membrane distillation. Materials Research Innovation, 18(S6): 359 – 363.

- Jamshidi Gohari, R., Halakoo, E., Nazri, N., Lau, W. J, Matsuura, T. and Ismail, A. (2014). Improving performance and antifouling capability of PES UF membranes via blending with highly hydrophilic hydrous manganese dioxide nanoparticles. Desalination, 335(1): 87 – 95.

- Al-Homaidan A., Jamila A. A., Al-Hazzani A. A., Al-Ghanayem A., Alabbad A. F. (2015). Adsorptive removal of cadmium ions by Spirulina platensis dry biomass. Saudi Journal of Biological Sciences, 22: 795 – 800.

- Adhiya J., Xiaohua C., Richard T. S., Samuel J. T. (2002). Binding of aqueous cadmium by the lyophilized biomass of Chlamydomonas reinhardtii. Colloids and Surfaces, 210: 1 – 11.