Malaysian Journal of Analytical Sciences Vol 21 No 2 (2017): 426 - 434

THE MORPHOLOGICAL PROPERTIES STUDY OF PHOTOCATALYTIC TIO2/PVDF DUAL LAYER HOLLOW FIBER MEMBRANE FOR ENDOCRINE DISRUPTING COMPOUNDS DEGRADATION

(Kajian Sifat Morfologi Membran Fotopemangkin Gentian Berongga Dwi Lapisan TiO2/PVDF untuk Penguraian Sebatian Pengganggu Endokrin)

Roziana Kamaludin1, Mohd Hafiz Dzarfan Othman1*,Siti Hamimah Sheikh Abdul Kadir2,

Mukhlis A Rahman1, Juhana Jaafar1

1Advanced Membrane Technology Research Centre (AMTEC),

Universiti Teknologi Malaysia, 81310 Skudai, Johor, Malaysia

2Institute of Medical Molecular Biotechnology, Faculty of Medicine, Sungai Buloh Campus,

Universiti Teknologi MARA, Jalan Hospital, 47000, Sungai Buloh, Selangor, Malaysia

*Corresponding author: hafiz@petroleum.utm.my

Received: 26 August 2016; Accepted: 8 January 2017

Abstract

Various Endocrine Disrupting Compounds (EDCs) either natural or synthetics have been identified including bisphenol A (BPA), dioxin, polychlorinated biphenyls, and dichlorodiphenyltrichloroethane (DDT). These compounds which present widely in the product of solvent industry, agricultural, pharmaceuticals and household convenience may directly or indirectly interrupt the normal function of endocrine system. Recently, the EDCs threats to human health have led to the increasing demand of clean water sources which excites a challenge for the contaminants removal processes. Conventional treatment methods may not completely remove the contaminants, meanwhile advanced oxidation process (AOPs) especially photocatalysis have been proven efficient in removing the contaminants. Even though photocatalysis is efficient in suspension of either nano- or microscale, the immobilizing TiO2 in the membrane matrix for catalyst separation purposes have decreased the adsorption of organic substances on the TiO2 surface with potential loss of TiO2 during long-time usage. This lead to the emerging in the exploration and fabrication of dual layer hollow fiber (DLHF) membrane for better performance of immobilized TiO2. Therefore, we exploit this fact to investigate the incorporation of photocatalysis into dual-layer hollow fiber membrane which are believed may improve the photocatalytic degradation and separation efficiency.  The FESEM results showed that both layers are compatible with each other and there are no delamination found between layers. The presence of outer layer does not influence the surface roughness; however improve the hydrophilicity and membrane strength. Altogether, photocatalytic dual layer hollow fiber (DLHF) membrane was successfully fabricated using co-extrusion technique.

Keywords:  dual layer, hollow fiber, TiO2, photocatalytic

Abstrak

Pelbagai sebatian pengganggu endokrin sama ada semula jadi atau buatan telah dikenalpasti termasuk bisphenol A (BPA), dioxin, polychlorinated biphenyls, dan dichlorodiphenyltrichloroethane (DDT). Sebatian ini yang wujud secara meluas di dalam produk industri pelarut, pertanian, farmaseutikal, dan keperluan rumah mampu menggangu secara lansung atau tidak langsung fungsi normal sistem endokrin. Baru-baru ini, ancaman sebatian pengganggu endokrin kepada kesihatan manusia telah membawa kepada peningkatan permintaan sumber air bersih yang merangsang satu cabaran bagi proses penyingkiran bahan pencemaran.. Kaedah rawatan konvensional mungkin tidak membuang bahan pencemaran dengan sepenuhnya, tetapi proses pengoksidaan maju (AOP) terutama fotopemangkinan telah terbukti berkesan dalam menyingkirkan bahan pencemaran. Walaupun fotopemangkinan cekap dalam skala nano atau skala hablur pemangkin, immobilisasi TiO2 ke dalam matriks membran untuk tujuan pemisahan pemangkin telah mengurangkan penyerapan bahan pencemar di permukaan TiO2 dengan potensi kerugian TiO2 untuk penggunaan yang lama. Ini membawa kepada penerokaan dan fabrikasi dwi lapisan gentian berongga (DLHF) membran untuk prestasi immobilisasi TiO2 yang lebih baik. Oleh itu, kami mengeksploitasi fakta ini untuk menyiasat penubuhan fotopemangkinan ke dalam dwi-lapisan membran gentian berongga yang dipercayai boleh meningkatkan kecekapan fotopemangkin dalam proses degradasi dan pemisahan bahan pencemar. Keputusan FESEM menunjukkan bahawa kedua-dua lapisan serasi dengan satu sama lain dan tidak ada delaminasi di antara dua lapisan. Kehadiran lapisan luar tidak mempengaruhi kekasaran permukaan; bagaimanapun meningkatkan sifat hidrofilik dan membran kekuatan. Secara keseluruhan, dwi lapisan membran gentian berongga telah berjaya direka menggunakan teknik penyemperitan bersama.

Kata kunci:  dwi lapisan, gentian berongga, TiO2, fotopemangkin

References

- Schmidler, C. (2015). Endocrine System Glands and Hormones. Access online http://www.healthpages.org/ anatomyfunction/endocrine system-gland/

- Chang, H-S., Choo, K-H., Lee B-W. and Choi, S-J. (2009). The methods of identification, analysis, and removal of endocrine disrupting compounds (EDCs) in water. Journal of Hazardous Materials 172: 1 –12.

- Filali-Meknassi, Y., Tyagi, R. D., Surampalli, R. Y., Barata, C. and Riva, M. C. (2004). Endocrine disrupting compounds in wastewater, sludge treatment processes and receiving waters: Overview. Practical Periodical Hazard, Toxic, and Radioactive Waste Management, 8: 1 – 18.

- Kralchevska, R. P., Milanova, M. M., Hristova, I. L. and Todorovsky, D. S. (2013). Some endocrine disrupting compounds in the environment and possibilities for their removal / degradation. Bulgarian Chemical Communications, 45, 2: 131 – 143.

- Pan, B., Lin, D., Mashayekhi, H. and Xing, B. (2008). Adsorption and hysteresis of bisphenol A and 17α-ethinyl estradiol on carbon nanomaterials. Environmental Science & Technology, 42(15): 5480 –5485.

- Grasselli, F., Baratta, L., Baioni, L., Bussolati, S., Ramonib, R., Grolli, S. and Basini, G. (2010). Bisphenol A disrupts granulosa cell function. Domestic Animal Endocrinology, 39: 34 – 39.

- Rubin, B. S. (2011). Bisphenol A: An endocrine disruptor with widespread exposure and multiple effects. Journal of Steroid Biochemistry and Molecular Biology, 127(1-2): 27 – 34.

- Stahlhut, R. W., Welshons, W. V. and Swan, S. H. (2009). Bisphenol A data in NHANES suggest longer than expected half-life, substantial non-food exposure, or both. Environmental Health Perspective, 117: 784 – 789.

- Kondarides, D. I. (2010). Photocatalysis. Catalysis. Encyclopedia of life support systems (EOLSS), developed under the auspices of the UNESCO. EOLSS Publishers, Paris.

- Dalrymple, O. K., Yeh, D. H. and Trotz, M. A. (2007). Removing pharmaceuticals and endocrine-disrupting compounds from wastewater by photocatalysis. Journal of Chemical Technology and Biotechnology, 82: 121 – 134.

- Xin, Y., Gao, M. Wang, Y. and Ma, D. (2014). Photoelectrocatalytic degradation of 4-nonylphenol in water with WO3/TiO2 nanotube array photoelectrodes. Chemical Engineering Journal, 242: 162 – 169.

- Umar, M. and Aziz, H. A. (2013). Photocatalytic degradation of organic pollutants in water. INTECH Open Access Publisher: pp. 195 – 208.

- Hashimoto, K., Irie, H. and Fujishima, A. (2005). TiO2 photocatalysis: A historical overview and future prospects. Japanese Journal of Applied Physics, 44(12): 8269 – 8285.

- Rao, K. V. S., Subrahmanyam, M. and Boule, P. (2004). Immobilized TiO2 photocatalyst during long-term use: decrease of its activity. Applied Catalysis B: Environmental, 49(4): 239 – 249.

- Peng, N., Chung, T.-S., Chng, M. L. and Aw, W. (2010). Evolution of ultra-thin dense-selective layer from single-layer to dual-layer hollow fibers using novel Extem® polyetherimide for gas separation. Journal of Membrane Science, 360: 48 – 57.

- Yang, Q., Wang, K. Y. and Chung, T.-S. (2009). Dual-layer hollow fibers with enhanced flux as novel forward osmosis membranes for water production. Environmental Science and Technology,, 43: 2800 – 2805.

- Ong, Y. K. and Chung, T. (2012). High performance dual-layer hollow fiber fabricated via novel immiscibility induced phase separation (I2PS) process for dehydration of ethanol. Journal of Membrane Science, 421–422: 271 – 282.

- Sun, S. P. Wang, K.Y. Peng, N. Hatton, T. A. and Chung, T.-S. (2010). Novel polyamideimide/ cellulose acetate dual-layer hollow fiber membranes for nanofiltration. Journal of Membrane Science, 363: 232 –242.

- Lee, J., Park, B., Kim, J. and Park, B. S. (2015). Effect of PVP, lithium chloride, and glycerol additives on PVDF dual-layer hollow fiber membranes fabricated using simultaneous spinning of TIPS and NIPS. Macromolecular Research, 23(3): 291 – 299.

- Khayet, M., Mengual, J., I. and Matsuura, T. (2005). Porous hydrophobic/hydrophilic composite membranes application in desalination using direct contact membrane distillation. Journal of Membrane Science, 252(1): 101 – 113.

- Setiawan, L., Wang, R., Shi, L., Li, K. and Fane, A. G. (2012). Novel dual-layer hollow fiber membranes applied for forward osmosis process. Journal of Membrane Science, 421 – 422: 238 – 246.

- Dzinun, H., Othman, M. H. D., Ismail, A. F., Puteh, M. H., Rahman, M. A. and Jaafar, J. (2015). Fabrication of dual layer hollow fibre membranes for photocatalytic degradation of organic pollutants. International Journal of Chemical Engineering and Applications, 6(4): 289 – 292.