Malaysian Journal of Analytical Sciences Vol 22 No 5 (2018): 885 - 892

LIQUID NATURAL RUBBER TOUGHENED POLY(LACTIC ACID) BLEND: EFFECTS OF COMPATIBILIZER TYPES AND LOADINGS ON

THERMO-MECHANICAL PROPERTIES

(Adunan Asid Polilaktik Diperkuat Getah Asli Cecair: Kesan Jenis dan Kandungan Penserasi Terhadap Sifat Terma-Mekanik)

Mohd Farid Hakim Mohd Ruf, Sahrim Ahmad, Ruey Shan Chen*, Dalila Shahdan, Farrah Diyana Zailan

School of Applied Physics,

Faculty of Science and Technology,

Universiti Kebangsaan Malaysia, 43600 UKM Bangi, Selangor, Malaysia

*Corresponding author:  chen@ukm.edu.my

Received: 20 July 2017; Accepted: 28 April 2018

Abstract

This research was carried out to investigate the addition of grafted copolymers of maleic anhydride grafted-polylactic acid (PLA-g-MA) and maleic anhydride grafted-natural rubber (NR-g-MA) on the mechanical and thermal properties of polylactic acid/ liquid natural rubber (PLA/LNR) blends. Prior to blend preparation, the PLA-g-MA and NR-g-MA were the first self-synthesized by mixing maleic anhydride (MA) and dicumyl peroxide (DCP) which acted as initiator with the PLA and natural rubber (NR), respectively. The PLA/LNR, PLA/LNR/PLA-g-MA and PLA/LNR/NR-g-MA blends were prepared via melt-blending method. The loading of PLA-g-MA and NR-g-MA was varied between 5, 10 and 15%, respectively. By comparing to the control sample (neat PLA/LNR blend), the addition of PLA-g-MA with 5 and 10% led to increment in tensile strength up to 22 MPa, whereas NR-g-MA promoted a negative trend of tensile properties to the blends. Differential scanning calorimetry (DSC) and thermogravimetry analysis (TGA) results showed the enhanced thermal stability for PLA/LNR/PLA-g-MA blend. Fourier transform infrared (FTIR) analysis confirmed the chemical interaction between the PLA and NR components where the grafted copolymers were successfully grafted onto PLA backbone and bonded with NR.

Keywords:  polymer blend, polylactic acid, toughening agent, compatibilizer, thermal

Abstrak

Kajian ini dijalankan bagi mengkaji kesan penambahan kopolimer maleik anhidrida cantuman-asid polilaktik (PLA-g-MA) dan maleik anhidrida cantuman-getah asli (NR-g-MA) terhadap sifat mekanik and terma polimer adunan asid polilaktik/cecair getah asli (PLA/CGA). Sebelum penyediaan adunan, PLA-g-MA dan NR-g-MA terlebih dahulu disintesiskan dengan mengadun maleik anhidrida (MA) dan dikumil peroksida (DCP) yang bertindak sebagai pemula bersama dengan PLA dan getah asli (GA). Adunan PLA/LNR, PLA/LNR/PLA-g-MA dan PLA/LNR/NR-g-MA disediakan dengan menggunakan kaedah pengadunan dalaman. Kandungan PLA-g-MA dan NR-g-MA masing-masing divariasikan kepada 5, 10 dan 15%. Berbanding dengan sampel kawalan (adunan PLA/LNR), penambahan PLA-g-MA sebanyak 5 dan 10% memberikan peningkatan terhadap kekuatan tegangan sehingga 22 MPa manakala NR-g-MA memberikan corak penurunan terhadap adunan. Keputusan kalorimeter pemgimbasan pembezaan (DSC) dan analisa termogravimetri (TGA) menunjukkan peningkatan kestabilan terma adunan PLA/LNR/PLA-g-MA. Analisa spektroskopi inframerah transformasi Fourier (FTIR) membuktikan interaksi kimia berlaku antara komponen PLA dan NR di mana kopolimer cantuman berjaya dicantum pada tulang belakang PLA dan diikat dengan NR.

Kata kunci:  polimer adunan, asid polilaktik, agen penguat, penserasi, terma

References

- Chen, R. S., Ab Ghani, M. H., Salleh, M. N., Ahmad, S. and Gan, S. (2014). Influence of blend composition and compatibilizer on mecahnical and morphological properties of recycled HDPE/PET blends. Material Sciences and Applications, 5: 943-952.

- Ahmad, S., Ibrahim, A., Che Som, S., Kohjiya, S. and Yoon, J. R. (1994). Natural rubber-HDPE blends with liquid natural rubber as a compatibilizer: thermal and mechanical properties. Journal of Applied Polymer Science, 51: 1357- 1363.

- Lee, Y. S., Ahmad, S. Rasid, R., Se Yong, E. N., Yew, C. H. and Tarawneh, M. A. (2011). Effects of liquid natural rubber (LNR) on the mechanical properties of LNR toughened epoxy composite. Sains Malaysiana, 40(7): 679-683.

- Tsou, C. H., Hung, W. S., Wu, C. S., Chen, J. C., Huang, C. Y., Chiu, S. H., Tsou, C. Y., Yao, W. H., Lin, S. M., Chu, C. K., Hu, C. C., Lee K. R. and Suen, M. C. (2014). New composition of maleic-anhydride-grafted poly(Lactic Acid)/rice husk with methylenediphenyl diisocyanate. Material Science, 20: 446-451.

- Orozco, V. H., Brostow, W., Chonkaew, W. and Lopez, B. L. (2009). Preparation and characterization of poly(lactic acid)-g-maleic anhydride + starch blends. Maromol Symposium, 277: 69-80.

- Mohammad, N. N. B., Arsad, A., Abdullah Sani, N. S. and Basri, M. H. (2017). Effect of compatibilisers on thermal and morphological properties of polylactic acid/natural rubber blends. Chemical Engineering Transactions, 56: 1027- 1032.

- Ma, P., Jiang, L., Ye, T., Dong, W. and Chen, M. (2014). Melt free-radical grafting of maleic anhydride onto biodegradable poly(lactic acid) by using styrene as a comonomer. Polymers, 6: 1528-1543.

- Afifi, H. and El-Wakil, A. A. (2008). Study of the effect of natural rubber-graft-maleic anhydride (NR-g-MA) on the compatibility of NR-NBR blends using the ultrasonic technique. Polymer-Plastic Technology and Engineering, 47: 1032-1039.

- Dong, Z. X., Liu, M. X., Jia, D. M. and Zhou, Y. H. (2013). Synthesis of natural rubber-g-maleic anhydride and its use as a compatibilizer in natural rubber/short nylon fiber composites. Chinese Journal of Polymer Science, 31: 1127-1138.

- Rui, Z., Hongzhi, L. and Jinwen, Z. (2012). Compatibilizing effects of maleated poly(lactic acid) on properties of PLA/soy protein composites. American Chemical Society, 51(22): 7786-7792.

- Bijarimi, M., Ahmad, S. and Rasid, R. (2012). Mechanical, thermal and morphological properties of PLA/PP melt blends. International Conference on Agriculture, Chemical And Environmental Sciences: pp. 115-117.

- Shahdan, D., Ahmad, S. and Flaifel, M. H. (2014). Effect of ultrasonic treatment on tensile properties of PLA/LNR/NiZn ferrite nanocomposite. AIP Conference Proceedings, 1571: 75-82.

- Abdullah, I. (1992). Liquid natural rubber as a compatibilizer in the blending of natural rubber with polypropylene. Materials Forum, 16: 353-357.

- Abdullah Sani, N. S., Arsad, A. and Rahmat, A. R. (2014). Synthesis of a compatibilizer and the effects of monomer concentration. Applied Mechanics and Materials, 554: 96-100.

- Abdullah Sani, N. S., Arsad, A., Rahmat, A. R. and Mohammad N. N. B. (2015). Effects of compatibilizer on thermal and mechanical properties of PLA/NR belnds.  Material Science Forum, 819: 241-245.

- Yi, X., Xu, L., Wang, Y. L., Zhong, G. J., Ji, X. and Li, Z. M. (2010). Morphology and properties of isotactic polypropylene/poly(ethyleneterepthalate) in situ microfibrillar reinforced blends: influence of viscosity ratio. European Polymer Journal, 46: 719-730.