Malaysian Journal of Analytical Sciences Vol 23 No 1 (2019): 138 - 146

PRODUCTION OF COMPOST FROM NON-SHREDDED EMPTY FRUIT BUNCHES MIXED WITH ACTIVATED LIQUID ORGANIC FERTILIZER IN TOWER COMPOSTER

(Penghasilan Kompos dari Tandan Kosong Kelapa Sawit Tanpa Cincang Dicampur dengan Baja Cair Aktif dalam Menara Komposter)

Irvan1,2*, Muhammad Rahman1, Dedy Anwar1, Bambang Trisakti1,2, Hiroyuki Daimon3

1Chemical Engineering Department

2Sustainable Energy and Biomaterial Center of Excellence

Universitas Sumatera Utara, Medan 20155, Indonesia

3Department of Environmental and Life Sciences,

Toyohashi University of Technology, Toyohashi, 441-8580, Japan

*Corresponding author:  irvan@usu.ac.id

Received: 13 April 2017; Accepted: 17 April 2018

Abstract

Empty fruit bunches (EFBs) with activated liquid organic fertiliser (ALOF) were composted in a tower composter. This study was conducted to evaluate the degradation of non-shredded EFB mixed with ALOF as an activator. The concrete tower composter with dimensions of 0.40 m x 0.40 m x 3.00 m was used to accelerate composting time by achieving thermophilic conditions. The moisture content (MC) of the compost was maintained at 55% - 65%. Compost quality, temperature, pH, MC, C/N ratio, bacterial count (BC) and electrical conductivity (EC) were recorded and evaluated. Composting was completed in 40 days, even though EFB turned to dark brown on day 10, indicating that the compost was mature. The quality of the final compost from day 10 to day 40 was slightly similar: pH of 7.6–8.4, MC of 41.16%–58.44%, C content of 20.27%–21.34%, N content of 1.02%–1.25%, C/N ratio of 19.94–20.24, BC of 17–41 x 106 CFU g−1 and EC of 3.74–4.90 dS m−1. These results suggested that the quality of the obtained compost satisfied the Indonesian National Standard.

Keywords:  activated liquid organic fertiliser, composting, empty fruit bunches, tower composter

Abstrak

Buah tandan kosong (EFB) dengan baja organik cecair teraktif (ALOF) telah dikompos di dalam menara komposter. Kajian ini dijalankan untuk menilai data degradasi EFB yang tidak diparut bercampur dengan ALOF sebagai pengaktif. Penggunaan menara komposter, yang diperbuat daripada konkrit dengan dimensi 0.40 m W x 0.40 m L x 3.00 m H, adalah untuk pecutan masa pengomposan bagi mencapai keadaan termofilik. Kandungan lembapan (MC) kompos dikekalkan dalam lingkungan 55 - 65%. Kualiti kompos, suhu, pH, kandungan lembapan, nisbah C/N, bilangan bakteria (BC), dan kekonduksian elektrik (EC) telah direkod dan dinilai. Proses pengomposan telah lengkap dalam masa yang singkat iaitu dalam 40 hari, dan pada hari yang ke-10, EFB telah bertukar warna menjadi coklat tua yang menunjukkan kematangan kompos. Kualiti kompos akhir dari hari 10 hingga ke hari 40 menunjukkan kualiti yang hampir sama iaitu pH antara 7.6 hingga 8.4; kandungan lembapan 41.16 - 58.44%; C 20.27 - 21.34%; N 1.02 -  1.25%; nisbah C/N 19.94 - 20.24; bilangan bakteria 17 - 41 x 106 CFU g-1, dan kekonduksian elektrik 3.7 - 4.9 dS.m-1. Kualiti kompos keluaran yang diperolehi telah mencapai Piawai Kebangsaan Indonesia (SNI).

Kata kunci:  baja organik cecair teraktif, pengomposan, buah tandan kosong, menara komposter

References

- Irvan, Trisakti, B., Sosanty, F. and Tomiuchi, Y. (2016). Effect of discontinuing sodium bicarbonate on fermentation process of palm oil mill effluent. Asian Journal of Chemistry, 28(2): 377 - 380.

- Trisakti, B., Lubis, J., Husaini, T. and Irvan (2017). Effect of turning frequency on composting of empty fruit bunches mixed with activated liquid organic fertilizer. IOP Conference Series: Materials Science and Engineering, 180: 012150.

- Kavitha, B., Jothimani, P. and Rajannan, G. (2013). Empty fruit bunch – a potential organic manure for agriculture. International Journal of Science, Environment and Technology, 2(5): 930 - 937.

- Suhaimi, M. and Ong, H. K. (2001). Composting empty fruit bunches of oil palm. food and fertilizer technology centre. Retrieved from http://www.agnet.org/library/eb/505a/pdf. [Access online 11 December 2013].

- Thambirajah, J. J., Zulkali, M. D. and Hashim, M. A. (1995). Microbiological and biochemical changes during the composting of oil palm empty fruit bunches – effect of nitrogen supplementation on the substrate. Bioresource Technology, 52(2): 133 - 144.

- Baharuddin, A. S., Wakisaka, M., Shirai, Y., Abd-Aziz, S., Abdul Rahman, N. A. and Hassan, M. A. (2009). Co–composting of empty fruit bunches and partially treated palm oil mill effluents in pilot scale. International Journal of Agricultural Research, 4(2): 69 - 78.

- Saiidi, S., Hasani, M., Hashemi, J. and Amini, M. (2012). Investigation of optimum conditions of co-composting process by using of sewage sludge and municipally waste. The 1st International and the 4th national congress on recycling of organic waste in agricultural, Isfahan, Iran. 26 - 27 April 2012.

- Shen, Y., Rein, L., Li, G., Chen, T. and Guo, R. (2010). Influence of aeration on CH4, N2O and NH3 emissions during aerobic composting of a chicken manure and high C/N waste mixture.  Waste Management, 31(1): 33 - 38.

- Hock, L. S., Baharuddin, A. S., Ahmad, M. N., Md Shah, U. K., Rahman, N. A. A., Abd-Aziz, S., Hassan, M. A. and Shirai, Y. (2009). Physicochemical changes in windrow co-composting process of oil palm mesocarp fiber and palm oil mill effluent anaerobic sludge. Australian Journal of Basic and Applied Sciences, 3(3): 2809 - 2816.

- Baharuddin, A. S., Hook, L. S., Yusof, M. Z. M., Rahman, N. A. A., Shah, U. K. M., Hasan, M. A., Wakisaka, M., Sakai, K. and Shirai, Y. (2010). Effects of palm oil mill effluent (POME) anaerobic sludge from 50 cm3 of closed anaerobic methane digested tank on pressed-shredded empty fruit bunch (EFB) composting process. African Journal of Biotechnology, 9(16): 2427 - 2436.

- Tiquia, S. M., Tam, N. F. Y. and Hodgkiss, I. J. (1997). Effects of turning frequency on composting of spent pig-manure sawdust litter. Bioresource Technology, 62: 37 - 42.

- Saidi, N., Chérif, M., Jedidi, N., Mahrouk, M., Fumio, M., Boudabous, A. and Hassen, A. (2008). Evolution of biochemical parameters during composting of various wastes compost. American Journal of Environmental Sciences, 4(4): 332 - 341.

- Kananam, W., Suksaroj, T. T. and Suksaroj, C. (2011). Biochemical changes during oil palm (Elaeis guineensis) empty fruit bunches composting with decanter sludge and chicken manure. Science Asia, 37: 17 - 23.

- Sundberg, C., Smars, S. and Jonsson, H. (2004). Low pH as an inhibiting factor in the transition from mesophilic phase in composting. Bioresource Technology, 95: 145 - 150.

- Nutongkaew, T., Duangsuwan, W., Prasertsan, S. and Prasertsan, P. (2011). Production of compost from palm oil mill biogas sludge mixed with palm oil mill wastes and biogas effluent. TIChE International Conference, Hatyai, Songkhla, Thailand. 10 – 11 November 2011.

- Zahrim, A. Y. and Asis, T. (2010). Production of non-shredded empty fruit bunch semi-compost. Journal-The Institution of Engineers, 71(4): 11 - 17.

- Razali, W. A. W., Baharuddin, A. S., Talib, A. T., Sulaiman, A., Naim, M. N., Hassan, M. A. and Shirai, Y. (2012). Degradation of oil palm empty fruit bunches (OPEFB) fibre during composting process using in vessel composter. BioResources, 7(4): 4786 - 4805.

- Gao, M., Liang, F., Yu, A., Li, B. and Yang, L. (2009). Evaluation of stability and maturity during forced-aeration composting of chicken manure and sawdust at different C/N ratios. Chemosphere, 78 (5): 614 - 619.

- Pathak, A. K., Singh, M. M., Kumara, V., Arya, S. and Trivedi, A. K. (2012).  Assessment of physico-chemical properties and microbial community during composting of municipal solid waste. Recent Research in Science and Technology, 4(4): 10 - 14.