Malaysian Journal of Analytical Sciences Vol 23 No 3 (2019): 488 - 494

# GREEN MICROWAVE-ASSISTED AQUEOUS ENZYMATIC EXTRACTION OF Elateriospermum tapos SEED OILS

# (Pengekstrakan Hijau Minyak Bijian Elateriospermum tapos Menggunakan Kaedah Enzim Berakues Berbantu Gelombang Mikro)

# Nuraimi Azlan Hadi Tan1 and Ida Idayu Muhamad1,2*

# 1Department of Bioprocess and Polymer Engineering, Faculty of Chemical and Energy Engineering

# 2 IJN-UTM Cardiovascular Engineering Centre

# Universiti Teknologi Malaysia, 81310 Johor Bahru, Johor, Malaysia

# *Corresponding author:  idaidayu@utm.my

Received: 25 October 2017; Accepted: 22 January 2019

Abstract

# Microwave-assisted aqueous enzymatic extraction (MAAEE) is a promising green technology that is fast, efficient and energy saving. The physicochemical parameters and fatty acids composition of the MAAEE-extracted oil were reportedly quite comparable with those of solvent-extracted oil and control oil, showing no significant differences (p> 0.05) among the extraction methods. However, the oxidative stability of the MAAEE obtained oil was found to be considerably improved in the comparison as evident by the determinations of chemical profile values. Moreover, the microwave-enzymatic adjuvant improved the oil extraction yield as well as the oxidation properties of the oil produced without altering its other quality attributes. The present work compared the quality characteristics of microwave-assisted aqueous enzymatic extracted Elateriospermum tapos seed oil with those of hexane-extracted oil and control oil. Interestingly, MAAEE process has achieved 12% and 16% higher extraction efficiency and concentrated omega-3 fatty acid, respectively compared to non-enzymatic microwave-assisted extraction (MAE). Although resulted in lower yield than hexane-extraction (Soxhlet), MAAEE extracted 24% higher concentrated omega-3 fatty acid with almost 99% removal of anti-nutrient amygdalin within 75 seconds. Hence the study on the interaction of extraction parameters as well as the MAAEE optimization process is sustainably feasible.

# Keyword:  microwave assisted aqueous enzymatic extraction, seed oils, Elateriospermum tapos, amygdalin

# Abstrak

# Pengekstrakan enzim berakues yang dibantu dengan gelombang mikro (MAAEE) adalah teknologi hijau yang menjanjikan proses yang pantas, cekap dan penjimatan tenaga. Parameter fizikokimia dan komposisi asid lemak daripada minyak yang diekstrak MAAEE dilaporkan setara berbanding dengan minyak ekstrak-pelarut dan minyak kawalan serta tidak menunjukkan perbezaan ketara (p> 0.05) di antara kaedah pengekstrakan. Walau bagaimanapun, kestabilan oksidatif minyak MAAEE didapati lebih baik dalam perbandingan seperti yang ditunjukkan oleh penentuan nilai-nilai profil kimia. Tambahan pula, adjuvan gelombang mikro-berenzim meningkatkan hasil pengeluaran minyak serta ciri pengoksidaan minyak yang dihasilkan tanpa mengubah sifat-sifat kualiti yang lain. Kajian ini membandingkan ciri-ciri kualiti minyak biji Elateriospermum tapos secara kaedah pengekstrakan enzim berakues-berbantu gelombang mikro dengan minyak ekstrak-pelarut heksana dan minyak kawalan. Menariknya, proses MAAEE telah masing-masing mencapai kecekapan pengekstrakan 12% dan kepekatan asid lemak omega-3 16% lebih tinggi berbanding pengekstrakan berbantu gelombang mikro tanpa berenzim (MAE). Walaupun hasilan lebih rendah daripada pengekstrakan heksana (Soxhlet), MAAEE memperoleh asid lemak omega-3 berkepekatan 24% lebih tinggi dengan hampir 99% penyingkiran amigdalin anti-nutrien dalam 75 saat. Oleh itu, kajian mengenai interaksi parameter pengekstrakan serta proses pengoptimuman MAAEE menunjukkan kewajaran secara lestari.

Kata kunci:  pengekstrakan enzim berakues dibantu gelombang mikro, minyak bijian, Elateriospermum tapos, amigdalin

References

- Chan, C., Yusoff, R., and Ngoh, G. (2013). Modeling and kinetics study of conventional and assisted batch solvent extraction. Chemical Engineering Research and Design. 92(6): 1169–1186

- Gai, Q.-Y., Jiao, J., Wei, F.-Y., Luo, M., Wang, W., Zu, Y.-G. and Fu, Y.-J. (2013). Enzyme-assisted aqueous extraction of oil from forsythia suspense seed and its physicochemical property and antioxidant activity. Industrial Crops and Products, 51: 274–278.

- Jiao, J., Li, Z., Gai, Q., Li, X., Wei, F., Fu, Y. and Ma, W. (2014). Microwave-assisted aqueous enzymatic extraction of oil from pumpkin seeds and evaluation of its physicochemical properties, fatty acid compositions and antioxidant activities. Food Chemistry, 147: 17–24.

- Kapoor, R. and Patil, U. K. (2011). Importance and production of omega-3 fatty acids from natural sources. International Food Research Journal, 18: 493–499.

- Yong, O. Y., and Salimon, J. (2006). Characteristics of Elateriospermum tapos seed oil as a new source of oilseed. Industrial Crops and Products, 24(2): 146 – 151

- Temelli, F., Saldaña, M. D. A., Moquin, P. H. L. and Sun, M. (2008). Supercritical fluid extraction of specialty oils. Martínez, J. L. (Ed).  Supercritical fluid extraction of nutraceuticals and bioactive compounds. London: CRC Press/Taylor & Francis: pp. 52 – 99.

- Sánchez-Camargo, A. P., Meireles, M. Â. A., Ferreira, A. L. K., Saito, E. and Cabral, F. A. (2012). Extraction of ω-3 fatty acids and astaxanthin from brazilian rrdspotted shrimp waste using supercritical CO2 + ethanol mixtures. Journal of Supercritical Fluids, 61: 71 – 77.

- Li, J., Zu, Y.-G., Luo, M., Gu, C.-B., Zhao, C.-J., Efferth, T. and Fu, Y.-J. (2013). Aqueous enzymatic process assisted by microwave extraction of oil from yellow horn (Xanthoceras sorbifolia Bunge.) seed kernels and its quality evaluation. Food Chemistry, 138(4): 2152 – 2158.

- Latif, S. and Anwar, F. (2011). Aqueous enzymatic sesame oil and protein extraction. Food Chemistry, 125(2): 679 – 684.

- Vlachos, N., Skopelitis, Y., Psaroudaki, M., Konstantinidou, V., Chatzilazarou, A, and Tegou, E. (2006). Applications of Fourier transform-infrared spectroscopy to edible oils. Analytica Chimica Acta, 573-574: 459–465.