Malaysian Journal of Analytical Sciences Vol 23 No 5 (2019): 870 - 879

AN ASSESSMENT OF FT-IR AND FT-NIR CAPABILITY IN SCREENING CRUDE PALM OIL AUTHENTICITY AND QUALITY COMBINED

WITH CHEMOMETRICS

(Satu Penilaian Terhadap Kebolehan FT-IR Dan FT-NIR Untuk Menyaring Kesahihan dan Kualiti Minyak Sawit Mentah Digabung Dengan Kemometri)

Ng Jing Sheng1, Widad Fadhullah1,2, Mohd Omar Ab Kadir1, Ainolsyakira Mohd Rodhi3,

Noor Hana Hanif Abu Bakar4, Syahidah Akmal Muhammad1,3*

1Environmental Technology Division, School of Industrial Technology,

Universiti Sains Malaysia, 11800 USM, Penang, Malaysia

2Environmental and Occupational Health Program, School of Health Sciences, Health Campus,

Universiti Sains Malaysia, 16150 USM, Kubang Kerian, Kelantan, Malaysia.

3Analytical Biochemistry Research Centre

4School of Chemical Sciences

Universiti Sains Malaysia, 11800 USM, Penang, Malaysia

*Corresponding author:  syahidah.muhammad@usm.my

Received: 16 January 2019; Accepted: 26 August 2019

Abstract

This study was conducted to assess the authenticity and evaluate the quality of palm oil with the aid of mid-infrared and near-infrared spectroscopy. Experiments were carried out to imitate the effect of frying in food service industry, therefore crude palm oil (CPO) samples were fried with different meat products (beef, mutton, fish, pork) at varying frequency of 3, 10, 20 and 30 times. Then the used frying oils were mixed with fresh and authentic CPO at different proportions, 5, 10, 20 and 30 (m/m %). Discriminant analysis on the infrared spectrum was carried out and the analysis was based on two class classification, which is to separate between 2 classes of samples. The results showed that the analysis with mid-IR spectrum successfully classified the adulterated oil samples from the authentic CPO with 2 misclassifications of adulterated oil into the authentic group out of 66 samples. The performance index of this model was 95.1 based on the Mahanalobis distance. The success rate of authentication was 96.7% for the mid-IR spectrum. However, the NIR spectrum analysis showed different results from the mid-IR as 11 adulterated samples out of 66 samples showed false positive result. The performance index of this model was 85.7 and the success rate of authentication was 83.3%. On the other hand, for the work on the classification of oil quality using mid-IR, the success rate was only 42.4% with the performance index of 73.4. Additionally, the NIR model for oil quality discrimination had 36.4% of success rate with a performance index of 56.4. The evaluation of oil quality based on the frequency of frying was not very successful and further work will be carried out. Overall, the authentication of palm oil was quite successful, but the evaluation of the quality of palm oil did not achieve our current objective. These rapid screening techniques, which are mid-infrared and near infrared spectroscopy have been proven to be useful for the authentication of edible palm oil but the evaluation of quality has to be further improved.

Keywords:  crude palm oil, authenticity, mid-infrared, near-infrared, discriminant analysis

Abstrak

Kajian ini telah dijalankan untuk menilai kesahihan dan kualiti minyak sawit dengan bantuan spektroskopi inframerah-tengah dan hampir. Eksperimen dijalankan dengan menggoreng minyak sawit mentah (CPO) dengan beberapa jenis produk daging haiwan (daging lembu, kambing, ikan dan khinzir) bertujuan untuk meniru kesan menggoreng makanan dalam industri perkhidmatan makanan, dengan kekerapan yang berbeza iaitu 3, 10, 20 dan 30 kali. Minyak goreng tersebut kemudian telah dicampur dengan minyak sawit mentah dengan peratusan berbeza iaitu 5, 10, 20 dan 30 (m/m %). Selepas itu, spektrum inframerah telah digunakan untuk menjalankan analisis diskriminan dengan menggunakan kemometrik. Analisis diskriminan ini merupakan analisis berasaskan 2 kelas, dan analisis ini berkeupayaan untuk membezakan 2 kelas yang berbeza. Keputusan menunjukkan bahawa analisis specktrum inframerah-tengah telah berjaya mengasingkan minyak goreng terpakai daripada minyak tulen dengan hanya 2 pengelasan salah daripada 66 sampel. Indeks prestasi untuk model ini ialah 95.1 berdasarkan jarak Mahalonobis. Kadar berjaya bagi model ini ialah 96.7%. Seterusnya, model spektrum inframerah-hampir menunjukkan keputusan yang berbeza daripada inframerah-tengah kerana terdapat 11 sampel daripada 66 sampel telah diklasifikasi sebagai positif palsu. Indeks prestasi untuk model ini merupakan 85.7 dan kadar berjaya untuk klasifikasi kesahihan minyak ialah 83.3%. Manakala, dalam bahagian penilaian kualiti minyak menggunakan inframerah-tengah, kadar berjaya hanya 42.4% dengan indeks prestasi sebanyak 73.4. Seterusnya, penilaian kualiti minyak goreng dengan menggunakan model inframerah-hampir menunjukkan kadar berjaya sebanyak 36.4% dan indeks prestasi serendah 56.4. Penilaian kualiti minyak goreng berdasarkan kekerapan menggoreng adalah kurang berjaya dan kajian lebih lanjut akan dijalankan pada masa yang akan datang. Secara keseluruhannya, pengesahan kesahihan minyak sawit adalah berjaya tetapi penilaian kualiti minyak masih tidak dapat mengecapi objektif kajian ini. Teknik pemeriksaaan pantas ini, iaitu spektroskopi inframerah-tengah dan hampir telah dibuktikan berkesan untuk pengesahan kesahihan minyak sawit tetapi penilaian kualiti masih memerlukan penambahbaikan.

Kata kunci:  minyak sawit mentah, kesahihan, inframerah-tengah, inframerah-hampir, analisis diskriminan

References

- Malaysia Palm Oil Board. (2015). Overview of the Malaysia Oil Palm Industry 2015. http://palmoilis.mpob.gov.my/index.php/overview-of-industry/408-overview-of-industry-2015. Access online 30/10/2017.

- Nunes, C. A. (2014). Vibrational spectroscopy and chemometrics to assess authenticity, adulteration and intrinsic quality parameters of edible oils and fats. Food Research International, 60: 255-261.

- Park, Y. W., Chang, P.-S. and Lee, J. (2010). Application of triacylglycerol and fatty acid analyses to discriminate blended sesame oil with soybean oil. Food Chemistry, 123(2): 377-383.

- Lerma-García, M. J., Herrero-Martínez, J. M., Ramis-Ramos, G. and Simó-Alfonso, E. F. (2008). Evaluation of the quality of olive oil using fatty acid profiles by direct infusion electrospray ionization mass spectrometry. Food Chemistry, 107(3): 1307-1313.

- Bertran, E., Blanco, M., Coello, J., Iturriaga, H., Maspoch, S. and Montoliu, I. (1999). Determination of olive oil free fatty acid by Fourier transform infrared spectroscopy. Journal of the American Oil Chemists' Society, 76(5): 611-616.

- Yang, H., Irudayaraj, J. and Paradkar, M. M. (2005). Discriminant analysis of edible oils and fats by FTIR, FT-NIR and FT-Raman spectroscopy. Food Chemistry, 93(1): 25-32.

- Moros, J., Garrigues, S. and Guardia, M. D. l. (2010). Vibrational spectroscopy provides a green tool for multi-component analysis. TrAC Trends in Analytical Chemistry, 29(7): 578-591.

- Reich, G. (2005). Near-infrared spectroscopy and imaging: Basic principles and pharmaceutical applications. Advanced Drug Delivery Reviews, 57(8): 1109-1143.

- van de Voort, F. R., Ghetler, A., García-González, D. L. and Li, Y. D. (2008). Perspectives on quantitative Mid-FTIR spectroscopy in relation to edible oil and lubricant analysis: Evolution and Integration of analytical methodologies. Food Analytical Methods, 1(3): 153-163.

- Maggio, R. M., Kaufman, T. S., Carlo, M. D., Cerretani, L., Bendini, A., Cichelli, A. and Compagnone, D. (2009). Monitoring of fatty acid composition in virgin olive oil by Fourier transformed infrared spectroscopy coupled with partial least squares. Food Chemistry, 114(4): 1549-1554.

- Rohman, A., Che Man, Y. B., Hashim, P. and Ismail, A. (2011). FTIR spectroscopy combined with chemometrics for analysis of lard adulteration in some vegetable oils Espectroscopia FTIR combinada con quimiometría para el análisis de adulteración con grasa de cerdo de aceites vegetales. CyTA - Journal of Food, 9(2): 96-101.

- Rohman, A., Che Man, Y. B., Ismail, A. and Hashim, P. (2010). Application of FTIR Spectroscopy for the determination of virgin coconut oil in binary mixtures with olive oil and palm oil. Journal of the American Oil Chemists' Society, 87(6): 601-606.

- Wang, L., Lee, F. S. C., Wang, X. and He, Y. (2006). Feasibility study of quantifying and discriminating soybean oil adulteration in camellia oils by attenuated total reflectance MIR and fiber optic diffuse reflectance NIR. Food Chemistry, 95(3): 529-536.

- Basri, K. N., Hussain, M. N., Bakar, J., Sharif, Z., Khir, M. F. A. and Zoolfakar, A. S. (2017). Classification and quantification of palm oil adulteration via portable NIR spectroscopy. Spectrochimica Acta Part A: Molecular and Biomolecular Spectroscopy, 173: 335-342.

- Blanco, M. and Pagès, J. (2002). Classification and quantitation of finishing oils by near infrared spectroscopy. Analytica Chimica Acta, 463(2): 295-303.

- Che Man, Y. B., Syahariza, Z. A., Mirghani, M. E. S., Jinap, S. and Bakar, J. (2005). Analysis of potential lard adulteration in chocolate and chocolate products using Fourier transform infrared spectroscopy. Food Chemistry, 90(4): 815-819.

- Goburdhun, D., Jhaumeer-Laulloo, S. B., Musruck, R. (2001). Evaluation of soybean oil quality during conventional frying by FTIR and some chemical indexes. International Journal of Food Sciences and Nutrition, 52(1): 31-42.

- Yang, H. and Irudayaraj, J. (2001). Comparison of near-infrared, fourier transform-infrared, and fourier transform-raman methods for determining olive pomace oil adulteration in extra virgin olive oil. Journal of the American Oil Chemists' Society, 78(9): 889.

- Innawong, B., Mallikarjunan, P., Irudayaraj, J. and Marcy, J. E. (2004). The determination of frying oil quality using Fourier transform infrared attenuated total reflectance. LWT - Food Science and Technology, 37(1): 23-28.