Malaysian Journal of Analytical Sciences Vol 24 No 1 (2020): 53 - 61

# APPLICATION OF MICROWAVE-ASSISTED EXTRACTION COUPLED

# WITH DISPERSIVE LIQUID-LIQUID MICROEXTRACTION FOR THE

# DETERMINATION OF POLYCYCLIC AROMATIC HYDROCARBONS

# IN VEGETABLES

# (Applikasi Pengekstrakan Berbantukan Mikrogelombang Gandingan dengan Pengekstrakan Mikro Cecair-Cecair Serakan bagi Penentukan Hidrokarbon Aromatik Polisiklik dalam

# Sayur-Sayuran)

# Chai Mee Kin1*, Tan Yeong Hwang1, Wong Ling Shing2

# 1College of Engineering,

# Universiti Tenaga Nasional, Jalan Ikram-Uniten, 43000 Kajang, Selangor, Malaysia

# 2Faculty of Health and Life Science,

# INTI International University, Persiaran Perdana BBN, Putra Nilai, 71800 Nilai, Negeri Sembilan, Malaysia

# *Corresponding author:  mkchai@uniten.edu.my

Received: 20 November 2019; Accepted: 21 January 2020

Abstract

# Microwave-assisted extraction (MAE) coupled with dispersive liquid-liquid microextraction (DLLME) followed by gas- chromatography flame ionization detector (GC-FID) for the determination of 13 PAHs in vegetable samples was developed in this study. The analytical performances of the optimized DLLME and MAE-DLLME including limit of detection (LOD), limit of quantification (LOQ), precision, accuracy, relative extraction recovery and relative extraction factor were validated and compared. The LOD of DLLME and MAE-DLLME were in the range of 0.040-0.400 and 0.0200-0.080 µg/L, respectively. For DLLME, precision and accuracy were 1.22-7.50% (RSD) and 71.77% to 90.93%, respectively; while for MAE-DLLME, were 0.77%-3.07% (RSD) and 83.65-98.42%, respectively. The relative extraction recovery was improved from 70.81% – 85.41% in DLLME to 85.79% - 99.61% in MAE-DLLME. The relative enrichment factors were ranged from 126-156 and 165 to 202 for DLLME and MAE-DLLME, respectively. The volume of extraction solvent was reduced from 50 µL to 30 µL in MAE- DLLME. The overall analytical performances of MAE-DLLME is better than DLLME. The application of proposed MAE- DLLME in real samples was also investigated and discussed.

# Keywords: dispersive liquid-liquid microextraction, microwave-assisted extraction, polycyclic aromatic hydrocarbons, extraction solvent

# Abstrak

Pengekstrakan berbantukan mikrogelombang (MAE) gandingan dengan pengekstrakan mikro cecair-cecair serakan (DLLME) yang diikuti dengan gas kromatografi-pengesan nyalaan ion (GC-FID) bagi penentukan 13 jenis hidrokarbon aromatik polisiklik (PAHs) dalam sampel sayur-sayuran telah dibangunkan dalam kajian ini. Analisis prestasi optimum DLLME dan MAE-DLLME seperti had pengesanan (LODs), kepersisan, kejituan, perolehan semula pengekstrakan secara relatif dan faktor-faktor pengayaan secara relatif telah disahkan dan dibandingkan. Had pengesanan (LODs) bagi DLLME dan MAE-DLLME adalah dalam lingkungan 0.040-0.400 dan 0.0200-0.080 μg/L, masing-masing. Bagi DLLME, kepersisan dan kejituan adalah dalam lingkungan 1.22-7.50% dan 71.77-90.93%, masing-masing; manakala bagi MAE-DLLME, adalah dalam lingkungan 0.77-3.07% dan 83.65-98.42%, masing-masing. Perolehan semula pengekstrakan secara relatif telah ditingkatkan daripada 70.81-85.41% pada DLLME hingga 85.79 - 99.61% pada MAE-DLLME. Faktor-faktor pengayaan secara relatif adalah 126-156 dan 165-202 bagi DLLME dan MAE-DLLME, masing-masing.  Jumlah  pengekstrakan pelarut yang digunakan telah dikurangkan daripada 50 μL di DLLME kepada 30 μL di MAE-DLLME. Prestasi keseluruhan analisis MAE-DLLME adalah lebih baik daripada DLLME. Penggunaan MAE-DLLME dalam analisis sampel sebenar juga disiasat dan dibincangkan.

# Kata kunci: 	pengekstrakan mikro cecair-cecair serakan, pengekstrakan berbantukan mikrogelombang, hidrokarbon aromatik polisiklik, pelarut pengekstrakan

References

- Li, H., Zhu, D., Lu, X., Du, H., Guan, S. and Chen, Z. (2018). Determination and risk assessment of sixteen polycyclic aromatic hydrocarbons in vegetables. Journal of Environmental Science and Health, Part A, 53(2): 116-123.

- Singh, L. and Agarwal, T. (2018). PAHs in Indian diet: Assessing the cancer risk. Chemosphere, 202: 366-376.

- Soceanu, A., Dobrinas, S., Stanciu, G. and Popescu, V. (2014). Polycyclic aromatic hydrocarbons in vegetables grown in urban and rural areas. Environmental Engineering and Management Journal, 13(9): 2311-2315.

- International Agency for research on cancer. (2017). IARC Monographs on the evaluation of carcinogenic risks to humans. http://monographs.iarc.fr/ENG/Classification/latest_classif.php. [Access online 18 April 2019].

- European Commission (2002). Opinion of the scientific committee on food on the risks to human health of polycyclic aromatic hydrocarbons in food. https://ec.europa.eu/food/sites/food/files/safety/docs/sci- com_scf_out153_en.pdf. [Access online 19 April 2019].

- Saraji, M., and Boroujeni, M. K. (2013). Recent developments in dispersive liquid-liquid microextraction microextraction techniques. Analytical and Bioanalytical Chemistry, 406: 2027-2066.

- Rezaee, M., Assadi, Y., Milani Hosseini, M.R., Aghaee, E., Ahmadi, F. and Berijani, S. (2006). Determination of organic compounds in water using dispersive liquid-liquid microextraction. Journal of Chromatography. A, 1116 (1-2): 1-9.

- Peng, G., Lu, Y., He, Q., Mmereki, D., Zhou, G., Chen, J. and Tang, X. (2016). Dispersive liquid–liquid microextraction using low-toxic solvent for the determination of heavy metals in water samples by inductively coupled plasma–mass spectrometry. Journal of AOAC International, 99 (1): 260-266.

- Chai, M. K., Chander, P. D. and Wong, L. S. (2016). Modified dispersive liquid-liquid microextraction using green solvent for extraction of polycyclic aromatic hydrocarbosn (PAHs) in vegetable samples. Malaysian Journal of Analytical Sciences, 20(1): 14 – 20.

- Wang, H., Ding, J. and Ren, N. (2016). Recent advances in microwave-assisted extraction of trace organic pollutants from food and environmental samples. Trends in Analytical Chemistry, 75: 197-208.

- Campillo, N., Viñas, P., Martínez-Castillo, N. and Hernández-Córdoba, M. (2011). Determination of volatile nitrosamines in meat products by microwave-assisted extraction and dispersive liquid-liquid microextraction coupled to gas chromatography-mass spectrometry. Journal of Chromatography A, 1218: 1815-1221.

- Mahmoudpour, M., Mohtadinia, J., Mousavi, M. M., Ansarin, M. and Nemati, M. (2016). Application of the microwave-assisted extraction and dispersive liquid–liquid microextraction for the analysis of PAHs in smoked rice. Food Analytical Methods, 10(1): 1-10.

- Mari, A., Ahmadi-Jouibari, T. and Nazir, F. (2016). Application of microwave-assisted dispersive liquid– liquid microextraction and graphite furnace atomic absorption spectrometry for ultra-trace determination of lead and cadmium in cereals and agricultural. International Journal of Environmental Analytical Chemistry. 96: 271-283.

- Tan, Y. H., Chai, M. K. and Wong, L. S. (2018). The effects of parameters on the efficiency of DLLME in extracting of PAHs from vegetable samples. International Journal of Engineering and Technology, 7 (4.35): 15-21.

- ICH Expert Working Group. (2005). Validation of analytical procedures: text and methodology Q2 (R1).

- Xu, F., Liu, L., Wei, W. and Xu, R. (2017). Determination of five endosulfan pesticides in the fish pond water by dispersive liquid-liquid microextraction combined with GC-MS. Forensic Sciences Research, 2(1): 1-6.

- Wagner, I. and Stichlmair, J. (2001). The effect of viscosity on mass transfer in pulsed sieve-tray extraction columns. Chemical Engineering and Technology, 24 (6): 616-619.

- Camargo, M. C. R., and Toledo, M. C. F. (2003). Polycyclic aromatic hydrocarbons in Brazilian vegetables and fruits. Food Control, 14(1): 49-53.

- Ashraf, M. W. and Salam, A. (2012). Polycyclic aromatic hydrocarbons (PAHs) in vegetables and fruits produced in Saudi Arabia. Bulletin of Environmental Contamination and Toxicology, 88 (4): 543-547.

- Zhao, X., Liu, X., Zhao, Z., Huang, C., Zhang, M., Wang, H. and Wang, X. (2009). Homogeneous liquid-liquid extraction combined with high performance liquid chromatography-fluorescence detection for determination of polycyclic aromatic hydrocarbons in vegetables. Journal of Separation Science, 32(12): 2051-2057.

- Lima, G. P. P., and Vianello, F. (2011). Review on the main differences between organic and conventional plant-based foods. International Journal of Food Science and Technology, 46: 1-13.

- Dugaya, A., Herrenknechtb , C., Czokc, M., Guyona, F and Pages, N. (2002). New procedure for selective extraction of polycyclic aromatic hydrocarbons in plants for gas chromatographic–mass spectrometric analysis. Journal of Chromatography A, 938: 1-7.

- Paris, A., Ledauphin, J., Poinot, P. and Gaillard, J. (2018). Polycyclic aromatic hydrocarbons in fruits and vegetables: Origin, analysis, and occurrence. Environmental Pollution, 234: 96-106.