VALIDATION AND DETERMINATION OF ASCORBIC ACID IN MULTIVITAMIN TABLETS BY DIFFERENTIAL PULSE ANODIC STRIPPING VOLTAMMETRIC TECHNIQUE AT A BARE GLASSY CARBON ELECTRODE

(Validasi dan Penentuan Asid Askorbik dalam Tablet Multivitamin Menggunakan Teknik Voltammetrik Perlucutan Anodik Denyutan Pembezaan pada Elektrod Karbon Berkaca)

Nur Syamimi Zainudin*, Siti Norbaitina Shaari, Megat Ahmad Kamal Megat Hanafiah

Faculty of Applied Sciences,

Universiti Teknologi MARA Pahang Jengka Campus, 26400 Bandar Tun Abdul Razak Jengka, Pahang, Malaysia

*Corresponding author:  nursyamimizainudin@uitm.edu.my

Received: 8 August 2020; Accepted: 29 September 2020; Published:  xx December 2020

Abstract

Growth of multivitamin tablet production increases curiosity among the researchers whether ascorbic acid (AA) content is in accordance with the standards or not. The differential pulse anodic stripping voltammetry (DPASV) technique using bare glassy carbon electrode and phosphate buffer at pH 4.2 has been proposed to analyse the ascorbic acid content. The experimental voltammetric parameters were optimized to obtain a maximum response with analytical validation of the technique. The optimum instrumental conditions for electroanalytical determination of AA by the proposed DPASV technique were initial potential = 0 V, final potential = 0.7 V, accumulation time = 60 s, scan rate = 0.125 V/s, accumulation potential = 0 V and pulse amplitude = 0.150 V. The curve was linear from 5 mg L-1 to 300 mg L-1 (R2 = 0.9999) with detection limit of 0.25 mg L-1. The precisions in terms of relative standard deviation (RSD) were 1.3%, 0.5% and 0.06%. The recoveries for AA content in the two multivitamin tablets were 98% and 102%, respectively. It can be concluded that the proposed technique is precise, accurate, rugged, low cost, fast and has the potential to be an alternative method for routine analysis of AA in other pharmaceutical products in future.

Keywords:  ascorbic acid, glassy carbon electrode, multivitamin, voltammetry

Abstrak

Pertambahan penghasilan multivitamin dalam bentuk tablet telah meningkatkan rasa ingin tahu para penyelidik samada kandungan asid askorbik (AA) memenuhi piawaian atau tidak. Teknik perlucutan anodik denyutan pembezaan yang menggunakan elektrod karbon berkaca tanpa pengubahsuaian dan larutan penimbal fosfat (pH 4.2) telah dicadangkan untuk menganalisa kandungan asid askorbik. Parameter-parameter eksperimen voltammetrik dioptimakan untuk mendapatkan tindakbalas maksima diikuti dengan validasi atau pengesahan teknik voltammetrik. Keadaan instrumentasi optima untuk penentuan AA dengan menggunakan teknik DPASV yang dicadangkan adalah keupayaan awal = 0 V, keupayaan akhir = 0.70 V, masa penjerapan = 60 s, kadar imbasan = 0.125 V/s, keupayaan penjerapan = 0 V dan amplitud denyut = 0.15 V. Keluk penentukuran adalah lurus daripada 5 mg L-1 to 300 mg L-1 (R2 = 0.9999) dengan had pengesanan 0.25 mg L-1. Kejituan dalam bentuk sisihan piawai relatif adalah 1.3%, 0.5% and 0.06%. Perolehan semula peratusan kandungan AA di dalam dua tablet multivitamin adalah 98% dan 102%. Kesimpulannya, teknik yang dicadangkan adalah jitu, tepat, tahan lasak, melibatkan kos yang rendah, cepat dan berpotensi untuk menjadi satu kaedah alternatif untuk menganalisa AA dalam produk pharmaseutikal yang lain pada masa akan datang.

Kata kunci:  asid askorbik, elektrod karbon berkaca, multivitamin, voltammetri

Introduction

Vitamins are one of the major nutrients that play very important roles in the body. The vitamins involved in the processes of normal metabolism, cell regulation and they are necessary for growth and development. There are thirteen vitamins that are recognized contributing in human nutrition [1]. These vitamins can be divided into fat-soluble vitamins and water-soluble vitamins. Vitamin A, D, E and K are fat-soluble vitamins. Meanwhile, ascorbic acid (AA) and B-complex are water-soluble vitamins [2]. The water-soluble vitamins dissolve easily in water and are readily excreted from the body via urine [3].

The AA in food was discovered in the twentieth century in 1907 by Holst and Frolich as a solution for scurvy. During that year, the disease had been reported to be ‘ship beriberi’. In order to avoid this disease, an experiment was done on guinea pigs by feeding them with fresh apples, fresh potatoes, fresh cabbage or fresh lemon juices which is high in the AA. The guinea pigs were also fed with a simple diet like oat, barley and wheat. The results indicated that deficiency of AA caused the scurvy [4]. The AA is also known as vitamin C, L-ascorbic and 2,3-didehydro-L-threohexono-1,4-lactone as well as 3-keto-L-gulofuranolactone. It has a chemical formula of C6H8O6 and molecular weight of 176 gmol-1 while its chemical structural is shown in Figure 1. The AA is important for collagen production, a protein which gives structure to bones, cartilages, muscles and blood vessels [5] as well as an antioxidant and free radical scavenger [6]. The AA also helps in iron absorption and maintains capillaries, bones and teeth [7]. The main natural sources of AA are various fruits and vegetables especially kiwi, mangoes, papayas, lettuce, tomatoes, peppers, strawberries, cantaloupe and broccolis [8].

It is known that humans cannot synthesize AA. Therefore, AA must be obtained by body through diet [9]. Pharmaceutical products such as multivitamins have become one of the sources of AA to meet requirement of daily dietary intake instead of fresh fruits and vegetables [10]. Unfortunately, the AA content in pharmaceutical products tends to be lost during processing, storing and packaging since it is highly sensitive to light, heat, temperature and oxygen [11]. Hence, it is very important to have a simple, very sensitive, precise, accurate, rugged, low cost and fast method for analysing AA even at a very low-level content.

Figure 1.  Chemical structure of ascorbic acid

Various analytical techniques have been applied for the AA determination. There are titrimetric method with an oxidant solution such as dichlorophenol (DCPIP) [12], potassium iodide or bromate [13], chromatographic method, particularly high performance liquid chromatography (HPLC) [11, 14, 15], spectrometric method [3, 16, 17], capillary electrophoresis method [18, 19] and fluorimetry [20]. However, HPLC methods use expensive equipment, require a long time of sample pre–treatment, use toxic organic solvent as mobile phase and various harmful reagents [21]. In titrimetric method, the difficulties are encountered with titrants and interferences often occur with coloured samples which lead to lack of specificity [6]. Meanwhile, in direct spectrophotometry, there is a matrix effect in ultraviolet (UV) region since many organic compounds in samples may exhibit UV absorbance during measurement.

Voltammetry techniques seem to have high potential for an alternative analytical method as they are very powerful and versatile tool due to their simplicity, fastness, low waste emission, low cost of chemicals and instrumentation. They have comparable sensitivity to more expensive techniques and abundance direct individual or simultaneously analysis of analyte without any or minimal sample pre-treatment or extraction [22]. These techniques are less sensitive toward the matrix effects than the other analytical techniques. Electroanalytical techniques have been applied for qualitative and quantitative analysis of a wide range of drug compounds [23].

Square-wave voltammetric technique has been used to determine AA based on its oxidation at a zeolite modified carbon paste electrode as a working electrode [24]. Cyclic and differential pulse voltammetric techniques were also used for electrochemical behaviour investigation and determination of AA at a modified carbon paste electrode with 2,7-bis (ferrocenylethynyl) fluoren-9-one [25]. Carbon nanotube paste electrode modified with 2,2-[1,2-ethanediylbis (nitriloethylidine)]-bis-hydroquinone has been used for electrochemical behaviour investigation and determination of AA using cyclic and differential pulse voltammetric techniques [26]. A multi-walled carbon nanotubes-tetradecyltrimethylammonium bromide film coated graphite electrode was used to investigate the electro-oxidation of AA by applying cyclic, differential pulse voltammetric and square-wave voltammetric techniques [27]. Simultaneous determination of AA and dopamine was carried out by differential pulse voltammetric technique at a boron doped diamond electrode [28] and gold nanoparticles film electrode [29]. The simultaneous determination of AA and acetaminophen has been conducted by differential pulse voltammetric technique at a boron doped diamond electrode using sodium sulphate as the supporting electrolyte [30].

The purpose of this study is to develop and validate a differential pulse anodic stripping voltammetric (DPASV) technique using bare glassy carbon electrode as a working electrode and phosphate buffer solution (pH 4.2) as a supporting electrolyte for quantitative analysis of AA in multivitamin tablets without prior sample preparation.

Materials and Methods

Materials

All chemicals used in this study were analytical grade reagent and all solutions were prepared in deionized water. The AA standard (MW = 176 g mol-1) was obtained from Sigma Aldrich, UK. For the preparation of 1000 mg L-1 AA stock solution, 100 mg of the AA was dissolved in 100 mL volumetric flask. The standard working solution series were prepared by the appropriate dilution of the stock solution. All solutions were protected from light and used within 24 hours to avoid decomposition. Phosphate buffer solution (PBS) was prepared by adding and dissolving 2.70 mL of ortho phosphoric acid, 27.218 g of potassium dihydrogen phosphate, 71.630 g disodium hydrogen phosphate in the 1000 mL of volumetric flask. Sodium hydroxide with the concentration of 0.1 M was used to adjust the pH of the PBS solution to the pH 4.2.

Instrumentation

The voltammetric experiments were carried out using Autolab (Metrohm, Switzerland) that consisted of a glassy carbon electrode (GCE) as the working electrode (WE), a platinum wire as the auxiliary electrode (AE) and an Ag/AgCl (3M KCl) as the reference electrode (RE). The GCE was polished with polishing alumina on an alumina pad and then rinsed with deionized water. The Autolab was connected to a computer, installed with NOVA 1.1 for data processing. The pH meter (Hanna Instruments, UK) was employed for all pH measurements.

Voltammetric measurement: Differential pulse anodic stripping voltammetry (DPASV)

AA standard  working  solution  was  spiked  into  the  PBS solution at pH 4.2 to give the final concentration of 10 mg L-1. The initial operational instrumental parameters applied were initial potential (Ei) = 0 V, final potential (Ef) = 0.7 V, accumulation time (tacc) = 10 s, scan rate (v) = 0.05 V/s, accumulation potential (Eacc) = 0 V and pulse amplitude = 0.075 V. These instrumental parameters for voltammetric measurements were optimized to increase the response of the proposed DPASV technique.

An appropriate linear range with acceptable correlation coefficient (R2), limit of detection (LOD), limit of quantification (LOQ), precision, accuracy, ruggedness and recovery of AA standard in the pharmaceutical samples were analysed in order to verify the suitability of the proposed DPASV technique for AA analysis as proposed by Miranda et al. [31]. The LOD was determined by additional lower concentration of the AA standard solution until obtaining a response that was significantly different from PBS solution at pH 4.2. The LOQ were calculated by the equations; LOD = 3 SD/m and the value of the LOQ was 3.333 times the value of the obtained LOD. The AA standard solution at the concentration of 5, 50 and 150 mg L-1 was applied for intra-day and inter-day precision, ruggedness and robustness with five replicates (n=5). The ruggedness of the proposed DPASV technique was investigated using the same instrument (Autolab) but had been conducted by two different analysts under the same optimum parameters. Statistical F-test was carried out for the ruggedness and robustness.

Collection and preservation of multivitamin tablets

Two different brands of multivitamin tablets with different labelled content of AA were obtained from a pharmacy in Jengka, Pahang. The multivitamin tablets of each brand were ground to a fine powder and dissolved in 200 mL of deionized water. Then, the samples were transferred to a covered scotch bottle to avoid degradation.

Recovery studies of AA standard in multivitamin tablets

The recovery of AA standard solution in the multivitamin tablet was determined by spiking 0.5 mL of the sample into the measuring cell containing PBS solution at pH 4.2. This study was carried out with three replicates (n = 3). Recovered concentrations of the spiked AA standard solution in the samples were calculated using the regression equation from a calibration curve from the following formula in equation 1 [32];

Cmeans solution   x  Vtotal

Csample =  	                                                       (1)

Vsample

where, Cmeans solution is concentration in the measuring solution (mg L-1), Vtotal is total volume of the measuring solution (L), Vsample is volume of sample in the measuring cell (L) and Csample is concentration in the sample (mg L-1).

Results and Discussion

DPSAV of AA

DPSAV offers a well-defined and sharp peak even at lower concentration of AA [5]. It is very important to optimize all the parameters that may influence the measurements in order to obtain the optimum response [33]. The effects of scan rate (v), accumulation time (tacc), accumulation potential (Eacc) and pulse amplitude on the Ip of 10 mg L-1 AA standard were studied in the PBS solution at pH 4.2.

The effect of accumulation time (tacc)

An accumulation time (tacc) is a period of time given for AA to accumulate or adsorb onto a glassy carbon electrode before being analysed further. The tacc is strongly affected the peak response and contributed to the high peak in electroanalytical measurements [34].  The influence of tacc on the peak response of AA in PBS solution at pH 4.2 has been studied over the range between 10 to 80 s. The Ip has increased starting from 10 s to 60 s. The tacc of 60 s was considered as the optimum tacc as the highest Ip has been obtained, which was 426 nA, as shown in Figure 2.

The increase of the Ip of AA with tacc was contributed by the accumulation of AA at the electrode surface in large amount since longer time has been given for the accumulation to occur [35]. Besides, the higher response of measurement can be achieved with longer of tacc. After the tacc of 60 s, the Ip has decreased probably due to saturation of the accumulated AA at the working electrode surface [36].

The effect of scan rate (v)

The relationships between Ip of AA and various v such as  25,  50,  75, 100, 125 and 150 mV/s  were  observed

using the initial parameters. The maximum current obtained was 469 nA at Ep of 0.39978 V with v of 0.125 V/s in PBS solution at pH 4.2

The effect of accumulation potential (Eacc)

Accumulation potential (Eacc) was the potential for AA to be accumulated at the surface of a mercury electrode, whereas an optimum Eacc was the most appropriate potential for the accumulation process to take place [34]. Figure 3 shows the maximum response was observed at Eacc of 0 mV with Ip 469 nA and Ep of 0.39978 V. It indicated that the accumulation of AA at the electrode surface effectively occurred at 0 mV.

The effect of pulse amplitude

According to De Lima et al. [37], pulse amplitude is one of the factors that affects the sensitivity of voltammetric measurements. The Ip and Ep have sharply increased from 0.075 V to 0.15 V and slowly decreased after 0.15 V, as shown in Figure 4. The pulse amplitude of 0.15 V was noted as optimum condition for analysis due the highest Ip observed which was 857 nA and Ep of 0.3598 V in PBS solution at pH 4.2.

Validation of the developed method: Linearity, limit of detection (LOD) and limit of quantification (LOQ)

The capability of the proposed DPASV technique as an analytical method for determination of AA was studied by evaluating the obtained Ip as the function of concentration under the optimized operational and instrumental parameters. The constructed calibration graph from 5 to 300 mg L-1 was represented by the linear equation of Ip (nA)  = 109.79x – 224.01 with 0.9999 of correlation coefficient (R2), as shown in Figure 5.The observed  LOD  for  the  standard  solution  was  0.25 mg L-1 and the LOQ was found to be 0.83 mg L-1.

Figure 2.   Plot of Ip against accumulation time (tacc) of AA in phosphate buffer solution pH 4.2

Figure 3.  Plot of Ip against accumulation potential (Eacc) of AA in phosphate buffer solution pH 4.2

Figure 4.  Plot of Ip against pulse amplitude in phosphate buffer solution pH 4.2

Figure 5.  Linear curve of Ip against concentration of ascorbic acid in phosphate buffer solution pH 4.2. Instrument parameters were Ei = +0 V, Ef = 0.7 V, tacc = 60 s, v = 0.125 V/s, Eacc = 0 V and pulse amplitude = 0.15 V

Precision

The precision of the proposed DPASV technique for AA analysis was determined in three replicates (n=3) at the concentrations of 5 mg L-1, 50 mg L-1 and 150 mg L-1. The RSD values obtained were 1.3%, 0.5% and 0.06% for respective concentrations. The RSD values found to be less 5% indicating that the measurements were precise and confidence [38].

Accuracy

The accuracy of the developed technique was tested by spiking three known different concentrations of AA which were 5 mg L-1, 50 mg L-1 and 150 mg L-1 into a voltammetric cell containing PBS solution at pH 4.2. These spiked AA standard solutions were measured in three replicates (n=3) and the discovered amounts of the spiked solutions in the cell were calculated using the equation of Ip (nA) = 109.79x – 224.01, where x is the found concentrations of AA in the voltammetric cell. The satisfactory recoveries of 93.08 %, 98.66 % and 99.00 % were respectively achieved for the spiked concentrations of 5 mg L-1, 50 mg L-1 and 150 mg L-1.

Ruggedness

The RSD values ranged from 0.07 % to 2.93 % for the first and second analyst, respectively for the measurements of 5 mg L-1, 50 mg L-1 and 150 mg L-1 AA using the same instrument [39]. From the two-tailed F test, there were no significant differences between the obtained variances for AA when the measurements were performed by two different analysts with the same instrument at the 5 % significance level. Hence, the results indicated that the developed technique was considered rugged.

Determination of AA in multivitamin tablets

To evaluate the application to a real sample analysis, the developed DPASV technique was used to analyse AA in multivitamin tablets. The multivitamin tablets were analysed by the developed technique under optimum parameters without any sample pre-treatment. Table 1 shows the content of AA in the multivitamin samples.

Assay results for the determination of the AA in the multivitamin tablets are shown in Table 2 by comparing the actual AA content with the labelled AA content in terms of mg/tablet [32]. The samples were analysed under the optimum instrumental parameters. The recoveries achieved for AA content in Sample A and B were 98 % and 102 %, respectively. These satisfactory recoveries indicated that the matrix did not affect the measurement of the AA in the multivitamin tablets by the proposed DPASV technique.

Table 1.  Ascorbic acid content in the multivitamin tablet were analysed by the proposed DPASV technique in phosphate buffer solution at pH 4.2 (n = 3)

| Sample | Ip (x 10-7 A) ± SD (RSD) | Concentration of Ascorbic Acid (mg/L) |
| --- | --- | --- |
| A | 4.40 ± 0.15 (3.40%) | 490 |
| B | 8.19 ± 0.12 (1.46%) | 769 |

Table 2.  Assay results for the determination of the AA in multivitamin tablets

| Sample | Labelled Content (mg/tablet) | Actual Content (mg/tablet) | Recovery (%) |
| --- | --- | --- | --- |
| A |  | 100 | 98 | 98 |
| B |  | 150 | 153 | 102 |

Conclusion

The proposed DPASV technique had been successfully applied to determine the AA in multivitamin tablets. The present method had an advantage that a very simple sample pre-treatment was required. It was also found to be practically rapid, convenient, sensitive, accurate, precise, rugged and low in cost. Therefore, it could be an excellent alternative method for the routine determination of AA in the multivitamin tablets.

References

- Eggersdorfer, M., Laudert, D., Letinos, U., McClymont, T., Medlock, J., Netscher, T. and Bonranth, W. (2012). One hundred years of vitamins-A success story of the natural science. Angewandte Chemie, 51: 12960-12990.

- Zhang, Y., Zhou, W., Yan, J., Liu, M., Zhou, Y., Shen, X., Ma, Y., Feng, X., Yang, J. and Li, G. (2018). A review of the extraction and determination methods of thirteen essential vitamins to the human body: An update from 2010. Molecules, 23: 1-25.

- Masoud, R.S., and Zahra, A. (2014). A highly sensitive kinetic spectrophotometric method for the determination of ascorbic acid in pharmaceutical samples. Iranian Journal of Pharmaceutical Research, 13(2): 372-382.

- Packer, L., and Fuchs, J. (1997). Vitamin C in health and disease. CRC Press: pp. 1-525.

- Yilmaz, S., Sadikoglu, M., Saglikoglu, G., Yagmur, S. and Askin, G. (2008). Determination of ascorbic acid in tablet dosage forms and some fruit juices by DPV. International Journal of Electrochemical Science. 3: 1534-1542.

- Sona, S., Jiri, M., Jiri, S., Mojmir, B., Jindrich, K. and Tunde, J. (2015). Determination of ascorbic acid by electrochemical techniques and other methods. International Journal of Electrochemical Science, 10: 2421-2431.

- Pisoschi, A. M., Pop, A., Negulescu, G. P., and Pisoschi, A. (2011). Determination of ascorbic acid content of some fruit juices and wine by voltammetry performed at Pt and carbon paste electrodes. Molecules, 16: 1349-1365.

- Sadia, G., Azizuddin., Rafi, A., Kousar, Y., Fareed, A. and Iftekhar, S. (2014). Determination of ascorbic acid content of some capsicum cultivars by cyclic voltammetry performed at GCE by external standard series calibration method. International Journal of Electrochemical Science, 9: 5751-5762.

- Snezana, S. M., Danijela, A. K., Danijela, C. N. D., and Milan, N. M. (2011). Rapid and reliable HPLC method for the determination of vitamin C in pharmaceutical samples. Tropical Journal of Pharmaceutical Research, 10(1): 105-111.

- Rahman, M. M., Khan, M. M. R., and Hosain, M. M. (2007). Analysis of vitamin c (ascorbic acid) contents in various fruits and vegetables by UV-spectrophotometry. Bangladesh Journal of Scientific and Industrial Research, 42: 417-424.

- Gazdik, Z., Zitka, O., Petrlova, J., Adam, V., Zehnalek, J., Horna, A., Reznicek, V., Beklova, M. and Kizek, R. (2008). Determination of vitamin C (ascorbic acid) using high performance liquid chromatography coupled with electrochemical detection. Sensors, 8: 7097-7112.

- Papuc, C., Pop, A., Serban, M. (2001). Metode analitice in biochimia veterinara. editura printech (Bucharest) 2001: pp. 167-169.

- Matei, N., Magearu, V., Bieghila, S., Dobrinas, S. (2004). The determination of vitamin C from sweet cherries and cherries. Revista de Chimie (Bucharest), 55: 294-296.

- Santos, J., Mendiola, J. A., Oliveira, M. B., Ibáñez, E. and Herrero, M. (2012). Sequential determination of fat-and water-soluble vitamins in green leafy vegetables during storage. Journal of Chromatography A, 1261: 179-188.

- Klimczak, I. and Gliszczy´nska-Swigło, A. (2015). Comparison of UPLC and HPLC methods for determination of vitamin C. Food Chemistry. 175: 100-105.

- Mohamed, A. M. I., Mohamed, H. A., Abdel-Latif, N. M. and Mohamed, M. R. A. (2011). Spectrofluorimetric determination of some water-soluble vitamins. Journal of AOAC International, 94: 1758-1769.

- Mohamed, A. M., Mohamed, H. A., Mohamed, N. A. and El-Zahery, M. R. (2011). Chemometric methods for the simultaneous determination of some water-soluble vitamins. Journal of AOAC International. 94: 467-481.

- Da Silva, D. C, Visentainer, J. V., de Souza, N. E., and Oliveira, C. C. (2013). Micellar electrokinetic chromatography method for determination of the ten water-soluble vitamins in food supplements. Food Analytical Methods, 6: 1592-1606.

- Aurora-Prado, M. S., Silva, C. A., Tavares, M. F. M., and Altria, K. D. (2010). Rapid determination of water-soluble and fat-soluble vitamins in commercial formulations by MEEKC. Chromatographia. 72: 687–694.

- Borowski, J., Szajdek, A., Borowska, E. J., Ciska, E., and Zielinski, H. (2008). Content of selected bioactive components and antioxidant properties of broccoli (Brassica oleracea L.). European Food Research and Technology. 226: 459-465.

- Radulescu, M. C., Bucur, B., Bucur, M. P. and Radu, G. L. (2014). Bienzymatic Biosensor for rapid detection of aspartame by flow injection analysis. Sensors, 14: 1028-1038.

- Amorello, D. and Orecchio, S. (2013). Micro-determination of dithiocarbamates in pesticide formulations using voltammetry. Microchemical Journal, 110: 334-339.

- Zhao, X., Tao, X., Wei, D. and Song, Q. (2006). Pharmacological activity and hydrolysis behaviour of novel ibuprofen glucopyranoside conjugates. European Journal of Medicinal Chemistry, 41: 1352-1358.

- Nezamzadeh, A., Amini, M. K. and Faghihian, H. (2007). Square-wave voltammetric determination of ascorbic acid base on its electrocatalytic oxidation at zeolite-modified carbon-paste electrodes. International Journal of Electrochemical Science. 2: 583-594.

- Raoof, J. B., Ojani R. and Beitollahi, H. (2007). Electrocatalytic determination of ascorbic acid at chemically modified carbon paste electrode with 2, 7-bis (ferrocenyl ethynyl) fluoren-9-one. International Journal of Electrochemical Science, 2: 534-548.

- Beitollahi, H., Ardakani, M. M., Naeimi H. and Ganjipour, B. (2009). Electrochemical characterization of 2,2’–[1,2-ethanediylbis (nitrilo ethylidine)]-bis-hydroquinone-carbon nanotube paste electrode and its application to simultaneous voltammetric determination of ascorbic and uric acid. Journal of Solid State Electrochemistry, 13: 353-363.

- Motahary, M., Ghoreishi, S. M., Behpour M. and Golestaneh, M. (2010). Electrochemical determination of ascorbic acid at the surface of a graphite electrode modified with a multi-walled carbon nanotubes/tetradecyltrimethylammonium bromide. Journal of Applied Electrochemistry, 40: 841-847.

- Zhao, G. H., Li, M. F., Li, M. L. (2007). Differetial pulse voltammetric determination of dopamine with the coexistence of ascorbic acid on boron-doped diamond surface. Central European Journal of Chemistry, 5: 1114-1123.

- Raoof, J. B., Kiani, A., Ojani, R., Valiollahi, R. and Rashid-Nadimi, S. (2010). Simultaneous voltammetric determination of ascorbic acidand dopamine at the surface of electrodes modified with self-assembled gold nanoparticle films. Journal of Solid State Electrochemistry, 14: 1171-1176.

- Cofan, C., Radovan, C. and Cinghita, D. (2009). Sumultaneous anodic assessment of ascorbic acid and acetaminophen in unbuffered solutions. Revista de Chimie. 60: 368-372.

- Miranda, M. P., del Rio, R., del Valle, M. A., Faundez, M. and Armijo, F. (2012). Use of fluorine-doped tin oxide electrodes for lipoic acid determination in dietary supplements. Journal of Electroanalytical Chemistry. 668: 1-6.

- Barbara, Z. (2011). Voltammetric determination of vitamin C on a carbon electrode. Monograph: Electrochemistry, A Laboratory Textbook, Switzerland: pp 35-38.

- Švorc, L., Sochr, J., Svĭtková, J., Riejav, M. and Bustin, D. (2013). Rapid and sensitive electrochemical determination of codeine in pharmaceutical formulations and human urine using a boron-doped diamond film electrode. Electrochimica Acta, 87: 503-510.

- Nur Syamimi, Z., Mohamad Hadzri, Y. and Noor Zuhartini, M. M. (2016). Voltammetric determination of Reactive Black 5 in wastewater samples from the batik industry. Malaysian Journal of Analytical Science, 20: 1254-1268.

- Jain, R. and Sharma, S. (2012). Glassy carbon electrode modified with multi-walled carbon nanotubes sensor for the quantification of antihistamine drug pheniramine in solubilized systems. Journal of Pharmaceutical Analysis, 2(1): 56-61.

- Geremedhin, W., Amare, M. and Admassie, S. (2013). Electrochemically pretreated glassy carbon electrode for electrochemical detection of fenitrothion in tap water and human urine. Electrochimica Acta, 87: 749-755.

- De Lima, F., Gozzi, F., Fiorucci, A. R., Cardoso, C. A. L., Arruda, G. J. and Ferreira, V. S. (2011). Determination of linuron in water and vegetable samples using stripping voltammetry with carbon paste electrode. Talanta, 83: 1763-1768.

- Ngai, K. S, Tan, W. T, Zulkarnain, Z., Ruzniza, M. Z. and Mohammed, Z. (2013). Voltammetry detection of ascorbic acid at glassy carbon electrode modified by single-walled carbon nanotube/zinc oxide. International Journal of Electrochemical Science, 8: 10557-10567.

- Jain, R. and Rather, J. A. (2011). Stripping voltammetry of tinidazole in solubilized system and biological fluids. Colloids and Surfaces A: Physicochemical and Engineering Aspects, 378: 27-33.